YOUR NAME, ADDRESS & TELEPHONE NUMBERS GOES HERE

(also referred to as The Specifier’ within this document)

OUR REF:				YOUR REF:	-		DATE:    2010

| Descriptions of each specification | Used for |
| --- | --- |
| Part 1 Specification Manual - Preliminaries & tender return form | Client, Tender, Main Contractor & final contract documents. |
| Part 2 Specification Manual - Main Specification & Notes | Client, Tender, Main Contractor, Building Regulations application, Structural Engineer & final contract documents. |
| Part 3 Specification Manual - General Specification | Client, Tender, Main Contractor & final contract documents. Used selectively & only on larger schemes |

SPECIFICATION MANUAL - PART 3

GENERAL SPECIFICATION

FOR:-

A

AT:-

FOR:-

MR. & MRS.

This Specification Manual must be read in conjunction with Drg. No's **** together with the separate the Structural Engineers details & any other relevant external suppliers or consultants documents where applicable.

HOW TO USE THIS SPECIFICATION MANUAL

This specification manual is split into 3 main parts being:-

PART 1.	The ‘Preliminaries (1)’ section - This section describes the general requirements of the contract & 	the obligations of both parties.    This is a very important section that has a monetary cost to the 	tendering contractor that is deemed to be included within the fixed price quotation for the works.

The ‘tender return form’ at the rear - This section summaries the documents & is to be 	completed by a tendering contractor.    It is intended to be removed from the specification manual (or 	copied) & retuned to the client.

PART 2.	The ‘Specification (2)’ section - This section describes each element of the works which must be 	read in conjunction with all of the drawings.    This section is usually very site specific to the project.    This 	is the main document that describes the work elements & is to support the Building Regulations 	application as well.

The ’Notes’ section  at the rear has general notes that may be applicable to several different 	elemental descriptions throughout the specification manual.    Some Notes are just contractor conditions 	as well.

PART 3.	The ‘Specification (3)’ section - This section ‘generally’ describes all inclusive descriptions

(this part) 	where only a small section may be relevant for this particular project which become apparent 	after reading the section.    This is to be used as a ‘general reference’ for good practice elements of 	construction for use by The Main Contractor for back up & for areas of work that may need completing 	that are not fully scheduled in detail in the part 2 specification section.

- All reference to ‘ The Specifier’ within this Specification Manual relates to the person or company stated in the top title block of the front page.

- All reference to the employer, client, owner, relates to Mr. & Mrs. ********

- All reference to the contractor, main contractor, builder relates to the person or company quoting to complete the works.

CONTENTS

GS5	EXCAVATIONS & FILL - GENERALLY

GS10	CONCRETE WORK - GENERALLY

GS15	BRICKWORK & BLOCKWORK - GENERALLY

GS20	CARPENTRY - GENERALLY

GS25	JOINERY - GENERALLY

GS30	STRUCTURAL METALWORK - GENERALLY

GS35	PAINTING - GENERALLY

SPECIFICATION (PART 3)

GS5	EXCAVATIONS & FILL - GENERALLY

General requirements

- Codes of practice: Comply with BS 6031 Earthworks and BS 8004 Foundations subject to any qualifications given below.

- Site exploration: No trial holes or site investigations have been undertaken. The Contractor shall make local enquiries and carry out his own investigations in respect of the ground conditions, water table and other factors affecting the proposed excavations.

- Existing services: Make all necessary enquiries from Local Authorities and Statutory Undertakers concerning the possible existence of live services on the site: report findings to the specifier and agree procedures.

- Existing waterways:    Subject to approval, temporarily divert and reinstate any ditches or other existing waterways.

Materials

- Earth filling: Sub-soil from the excavations selected for suitability for backfilling and compaction.

- Earth filling arising from the excavations: Selected by the Contractor for backfilling, the best of the excavated material; free from:

a) Top soil,roots or other vegetable matter.

b) Soil with high moisture content and frozen soil.

c) Combustible or perishable material.

d) Any material not capable of being satisfactorily compacted.

- Imported earth filling: Cohesive soil or well graded granular material capable of being compacted to the bearing capacity required, free from:

a) Top soil, roots or other vegetable matter.

b) Soil with a high moisture content or frozen soil.

c) Combustible or other perishable material.

- Imported top soil: Clean friable soil to BS 3882 with adequate humus content, obtained from approved sources: do not spread until inspected and approved.

- Turf: Weed-free turf to BS 3969, minimum 20 mm thick; there should be no delay between cutting the turf and delivery to site and laying.

- Hardcore: Hard impermeable, incombustible, imperishable and chemically inert material broken to pass a 100 mm    ring in all directions: with limited fines content to ensure free-draining after compaction: free from water-soluble sulphates. Inspect hardcore obtained from demolition works to ensure freedom from timber particles liable to spread dry rot.    Second hand hardcore is not to be used for building ground bearing floors.

- Blinding for hardcore: Sharp sand/cement dry mix.

- Hoggin: Naturally occurring gravel with a sufficient proportion of cohesive fines to produce self- binding characteristics.

- Gravel, etc on the site: Subject to approval and certain provisions, gravel, sand or other building material in the ground and arising from the Works' excavations may be used in the Works. No excavations purely to raise this material will be permitted.

- Site materials: Close site supervision would be necessary to ensure this facility was not abused.

- Clayboard: Installed as required By Structural Engineer or Local Authority Building Control Officer.

Workmanship

Preliminary site work

- Levels: Set up and maintain an accurately ascertained site datum level.

- Material to be removed from site: The term    remove' used below in this section means the removal from the site of all unwanted material such as debris, tree roots, surplus soil, old drains and foundations, etc. Remove all such material to a tip authorised by the local authority and pay all charges.

- Site clearance: Before commencing excavations, clear site of rubbish, shrubs and general vegetation and remove.

- Prevent nuisance: Prevent nuisance as stated elsewhere.

- Preservation of turf: Carefully cut and lift turf to provide evenly sized turves of consistent thickness not less than 20 mm and stack on site in approved locations. Protect until required for reuse.

- Preservation of plants: Mark and protect all trees and other planting to be retained. Do not cut or otherwise damage branches or roots. Erect barriers where necessary.

- Trees to be felled: Fell trees as indicated. on plans. Excavate stumps and all major roots: cut up and remove all timber: backfill stump holes with top soil.

- Reducing levels: Reduce levels down to the specified formation level and remove soil.

Excavations

- Safety: Take adequate precautions to ensure the safety of excavations and everything affected by them. Comply with the procedures given in Section 1.4 of BS 6031 and Section 11 of BS 8004.

- Excavating: Excavate to the required depths and profiles.

- Excavating for drains and services: See Drainage section.

- Unexpected situations: Record any circumstances not anticipated by the drawings or otherwise; seek instructions if the Works are affected in any way.

- Obstructions in the ground: Break up and remove surface beds and pavings, old foundations, disused drains and manholes and all other obstructions in the ground. The use of explosives is not permitted.

- Voids in the ground: Report any voids, tanks or other potential hazards encountered and seek instructions. Backfill any voids caused by removing obstructions with hardcore/& dry mix conc. in 300mm layers thoroughly compacted.

- Toxic substances: Inform & seek help immediately if found.

- Disused drains: Plug any pipes cut off at the perimeter of excavations with foundation grade concrete rammed in to refusal (but not less than 300 mm).

- Existing live services: Report as soon as encountered and protect pending instructions: notify all authorities concerned.    Give adequate notice of when the work is to be undertaken in order that inspections can be made.

- Surface and ground water: Keep the site generally, and the excavations in particular, free from water arising from any cause: adopt the following procedures and precautions:

a) Use methods that cause no harm or nuisance.

b) Do not discharge water into any watercourses or drains without approval from the relevant authorities.

c) Do not pump or use any other method that draws fine material from adjacent ground.

d) If sumps are formed, backfill them and reinstate ground to original condition.

e) Take special precautions against accidental or sudden flooding.

- Bottom of excavations: Adopt the following procedures:

a) Report if a firm bearing is encountered before the specified foundation depth is reached.

b) Report if a firm bearing cannot be established at the specified foundation depth.

c) Probe the bearing surfaces for soft spots: report the extent and location of those encountered pending instructions in respect of additional excavation and backfilling with suitable material.

d) Obtain approval for the excavations from the site visiting Building Inspector immediately before placing foundations.

e) Remove the final 75 mm of soil and level and compact immediately before placing the concrete.

f) Record and agree any levels or depths that differ from those specified or declared provisional.

- Excess excavation: Should there be any excavation in excess of that required for the Works, make good the ground as directed and according to circumstances: note that reinstatement with lean-mix concrete may be ordered.

- Safety of the excavations: Adopt the following procedures:

a) Take whatever precautions necessary to eliminate any risk of collapse of the sides of the excavations. This is a Contractor's risk and if any damage occurs he shall undertake any repair work as necessary or as directed.

b) Ensure that excavations cause no risks to adjacent roads, paths or buildings, etc.

c) Keep raised soil well back from edges of excavations.

d) Prevent (by barriers or other means) vehicles, plant, spoil or anything approaching the area adjacent to the edges of excavations.

- Disposal of excavated material: Dispose of soil, etc as follows:

a) Keep all top soil separate and remove or stack on site as directed unless surplus to requirements.

b) Select the best of the excavated soil for filling and deposit in spoil heaps for that purpose.

c) Remove all surplus spoil from site to an approved council tip or recycling yard.

Filling

- Inspection: Do not backfill around foundations, etc until inspected and approved.

- Frozen material or ground: Do not undertake any filling when this situation occurs.

- Backfilling excavations: Backfill around foundations with selected material arising from the excavations: carefully compact in approx 250mm layers.

- Compaction: Compact all fill materials in layers as specified and in accordance with the following general requirements:

a) Undertake all compaction with suitable mechanical plant.

b) Compact around structures with an unsupported side (eg retaining walls) in thin layers and with care: otherwise fill and compact both sides together.

c) Compact hard fill (eg hardcore) in layers of thickness approx one third more than the max nominal particle size of the fill: continue compaction until there is no surface movement ahead of the roller, all surface voids are filled and the surface is smooth and level or graded as required.

d) Undertake initial compaction over drains and ducts, etc with care to avoid fractures.

GS10	CONCRETE WORK - GENERALLY

- General requirements: Concrete shall conform to the relevant parts of BS 5328: Parts 2, 3 and 4.    The guidance in BS8110 on concrete workmanship shall be followed unless agreement to alternative procedures is given.

Method of specifying concrete

- Contractor's option: The Contractor may purchase ready-mixed concrete and/or mix concrete on the site.

- Refer to BS 5328: This specification invokes BS 5328 :Parts 2, 3 and 4 Concrete as a tender document and shall be used in conjunction with this specification (which takes precedence). The Contractor should keep a copy on the site while concreting is in progress. The qualities of concrete described below are selected from BS 5328: Part 1 which embraces both ready-mixed and site mixed concrete. Where designated mixes have been specified, standard mixes agreed as equivalent are permitted.    Where standard mixes have been specified, designated mixes agreed as equivalent are permitted.

Materials

- Cements

- Sulphate-resisting Portland cement: BS 4027.

- Aggregates

- Coarse aggregate: Single sized natural gravel, crushed gravel or crushed stone to BS 882.

- Fine aggregate: Well graded natural sand or mixed sand to BS 882.

- All-in aggregate: Aggregate to BS 882 containing a proportion of material in all sizes graded from    sand to the nominal maximum size specified . Do not use for mixes exceeding 15 N/mm2 .

- Aggregates for exposed concrete: Aggregates as stated above free from all impurities likely to cause discolouration (eg iron pyrites) and of consistent colour.

- All aggregates: Obtain each type from one source capable of maintaining adequate supplies of consistently graded material for the duration of the concrete work.

- Water

- Water: Mixing water shall be of drinkable quality or conform to the limits set in Appendix A of BS3148: 1980.

- REINFORCEMENT (refer to Structural Engineers details)

- Mild steel: Hot rolled mild steel bars to BS 4449.

- High yield: Grade 460 hot rolled high yield steel deformed bars to BS 4449.

- Cold worked: Cold worked steel deformed bars to BS 4461.

- Fabric: Steel fabric to BS 4483.

- Admixtures

- Generally: Admixtures shall conform to BS 5075.

- Calcium chloride: Do not use under any circumstances.

- WATERPROOF MEMBRANES

- Self-adhesive vertical bituminous damp proof membranes: Bituthene applied to primered surface.

- Loose laid horizontal DPM Polythene sheeting: 1200g Visqueen.    Use large continuous sheets off a roll where possible to avoid joints & laps.    Lap & seal all unavoidable joints by 200mm.

- STORING AND HANDLING MATERIALS

- Delivery of cement: Reject any bagged cement if the bags have been subjected to damp or if they are torn.

- Protect from contamination and heavy or continuous rain.

- Store fabric reinforcement flat. Do not accept delivery of any sheets unless delivered flat.

- Do not subject to rough handling or shock loading.

- Ready-mixed concrete

- Single source of supply: Obtain concrete from one depot of the British Ready Mixed Concrete Association

- Supply of ready-mix: Add    with quality control procedures' if required (ie not required if only prescribed mixes are being used).

- Quality control: The Contractor (ie the purchaser referred to in BS 5328) shall accept responsibility for ensuring that the concrete complies with this specification and BS 5328. Retain all delivery tickets for inspection.

Making concrete on site

- Batching and mixing

- Generally: Batch and mix all concrete stronger than 15 N/mm2 by weight; measure water by volume or weight; all in accordance with BS 5328 : Part 3.

- Water in aggregates: Measure water content at the beginning of each day whilst concreting. Display simple adjustment tables near the batching plant. Adjust water content in order to attain the desired workability.

- Admixtures: Carefully measure and add coincidentally with the mixing water. Mix powder pigments with the cement and aggregate before adding water.

- Mixing: Clean the mixer frequently and on completion. Commence loading the mixer with coarse aggregate. Reduce the amount of coarse aggregate slightly in the first mix in a clean mixer. Continue mixing until the concrete is consistent and uniform in colour. Discharge each batch completely from the mixer.

- Batching aggregates by volume: Permitted only for small quantities of concrete 15 N/mm2 and under. Measure proportions into gauge boxes. Completely fill and empty boxes at each batching.

- Handmixing: Hand mixing should be avoided but if unavoidable increase the cement content by 10% and mix on a smooth clean impervious surface; thoroughly mix dry before adding water. Preferably, use a small portable modern powered mixer.

Workmanship to concrete

- Transporting

- Transport quickly:Transport concrete from the mixer to as near as possible to its final position as quickly as practicable in clean containers by methods which prevent segregation or loss of ingredients, maintain adequate workability and afford protection from contamination and adverse weather.

PLACING

- Workability: Do not place any concrete if the workability is unsuitable. Do not add more water.

- Placing: Place concrete as follows:

a) Continuously between construction joints and predetermined abutments.

b) Without disturbance to forms or reinforcement.

c) Without disturbance to pipes, ducts, fixings and the like to be cast in; ensure that such items are securely fixed. Ensure that concrete cannot enter open ends of pipes and conduits, etc.

d)Without dropping in a manner that could cause segregation or shock.

e) In deep pours only when the concrete and formwork designed for this purpose and by using suitable chutes or pipes.

f) Do not place if the workability is such that full compaction cannot be achieved.

g) Without disturbing the unsupported sides of excavations; prevent contamination of concrete with earth. Provide sheeting if necessary. In supported excavations, withdraw the linings progressively as concrete is placed.

h) If placed directly onto hardcore or any other porous material, dampen the surface to reduce loss of water from the concrete.

j) Ensure that there is no damage of displacement to sheet membranes.

k) Record the time and location of placing structural concrete.

COMPACTION

- Generally: Compact concrete until it forms a solid mass free from voids thoroughly worked around reinforcement and embedded fixtures and into all corners of the formwork.

- Mechanical vibration: Compact all structural concrete with internal vibrators.    Vibrators shall not be permitted to touch reinforcement nor shall they be used as a means for moving concrete. Vibrate until all air is expelled but not in a manner that will cause segregation or bring grout to the top surface. Do not permit vibrators to touch formwork required to impart a special finish.

CURING

- Generally: Select a type and method of curing that will ensure the concrete attains a satisfactory durability and strength with a minimum of distortion, shrinkage cracking and efflorescence. If necessary insulate the concrete to maintain it at a suitable temperature.

- Approved methods: Subject to the need to undertake early work to the surface of the concrete, approved methods are:

a) Maintaining forms in place.

b) Waterproof sheeting kept in close contact.

c) A curing compound, unless likely to adversely affect a specified surface finish or colour or the bending of the materials to the concrete surface .

- Guide times: Two days for normal conditions which are not naturally damp. Four days in hot weather and/or drying winds and at least 7 days for surfaces subject to abrasion.

- Generally: In addition to references to the colour of finished concrete given in preceding clauses, adopt the following procedures when a consistent colour is required:

a) Keep concrete mixes constant and ensure that constituents are obtained from the same source.

b) Use the same formwork release agent and ensure it is compatible with the surface finish required.

c) Keep formwork casting faces in good condition; do not mix new and re-used panels for the same elevation or concrete member.

d) Employ carefully controlled and consistent curing methods.

e) Obtain approval for the type and location of construction joints.

CONSTRUCTION JOINTS

- Generally: The clauses below refer to joints required by the Contractor for convenience of concreting; they shall be as few as possible consistent with precautions against shrinkage. Movement joints and other designed joints are dealt with below under appropriate headings.

- Location: Locate straight joints at approved points of minimum shear, at right angles to the general direction of a member and to coincide with any features in the finished work.

- Preparation: Provide clean flat stop boards. Immediately before recommencing casting, clean and roughen the face of the joint to the extent of removing laitence and exposing but not disturbing the large aggregate. Thoroughly compact the concrete up to the joint. Other methods of forming construction joints shall be subject to approval.

- Kickers: Provide locating frames or form kickers where required at least 70 mm high cast monolithic with the slab. In kickerless construction, a method to ensure the correct alignment of the forms shall be used.

- Reinforcement: Provide split and notched stop boards where reinforcement crosses a construction joint. Do not bend or otherwise adapt reinforcement in order to facilitate construction joints. Prevent grout leaks.

- Finish: No trace of construction joints will be permitted in concrete with a specified surface finish.

MOVEMENT JOINTS

- Generally: Provide movement joints where specified as follows:

a) Form a fair flat face to the concrete.

b) Allow 48 hours before casting adjacent bays up to contraction joints.

c) Compact the concrete on both sides of the joint with additional care to form dense uniform concrete.

d) Ensure that the entire movement area does not contain any concrete or other material other than the joint filler or sealant.

e) Ensure that no concrete impregnates or penetrates the joint filler.

f) Where a sealant is required position the joint filler accurately to form a correctly sized space. See 660 for fixing sealants.

DOWELLED JOINTS

- Dowels: Ensure the specified bars are clean, perfectly straight and with the free end slightly chamfered. Cast in one half of the dowel length at correct level and at right angles to the joint face. Coat or wrap the debonded projection to ensure free movement.

WEATHER PRECAUTIONS

- Generally: Take precautions to prevent damage due to unfavourable weather of any description as stated in 400 of General Requirements. Take particular precautions in respect of cold and freezing conditions as stated hereunder.

- Hot weather: In hot weather and during drying winds adopt the following precautions as appropriate:

a) Extend curing procedures as long as possible ,but at least for the periods given in table 6.5 of BS 8110

b) Prevent evaporation or other water loss from the concrete.

c) Use water spray to cool formwork or other hot surfaces.

d) Do not use water to cool hot immature concrete surfaces (this induces cracking).

e) Keep the concrete temperature below 30o C when placing.

f) Shade concrete from long exposure to hot sunshine.

- Cold weather: Prevent the temperature of concrete falling to freezing point during the early stages of hardening. Do not place concrete against frozen ground or forms.

- Heavy rain: Do not start or continue concreting in the open in heavy rain unless the risks of damage can be avoided.

- Concreting at low temperatures: If concreting is to proceed when the air temperature is below 2o C, the minimum temperature of the concrete during placing and hardening shall be 5o C.

- Methods: Recommended methods of concreting at low temperatures, used singly or in conjunction as appropriate, are as follows:

a) Heating the water (not above 80o C).

b) Keeping aggregates dry.

c) Increasing the cement content.

d) Using rapid hardening cement.

e) Use chloride free accelerating admixture

f) Heating the containers used for transporting concrete.

g) Insulating the placed concrete, particularly the tops of slabs.

h) Providing heated enclosures.

i) Heating the formwork and reinforcement.

Precautions: Take the following precautions (as appropriate) when concreting at low temperatures.

a) Do not adopt water curing.

b) Prevent water and aggregates from freezing; protect from ice and snow.

c) Provide wind breaks.

d) Insulate any steel formwork.

e) Do not permit cement to come into contact with water over 60o C unless the water is mixed with the aggregate first.

SURFACE FINISHES TO UNIFORMED CONCRETE

- Generally: Undertake surface finishing (normally confined to upper surfaces) at the correct time in relation to the setting and hardening of the concrete. Sprinkling cement onto the surface of concrete is not permitted. See under Formwork for cast finishes.

- Trowelled finish to receive thin floor finishings: Repeatedly trowel by appropriate means (eg power float) to produce a dense very smooth surface that is visually flat and suited to the direct application of thin floor coverings. Any defect in the finished concrete that shows through the floor finish will not be accepted. Maximum permissible deviation from flat: 3 mm from a 3.00 m straight edge.

- Textured non-slip surface: Trowel to a dense smooth surface and immediately roll with a ribbed, studded or otherwise suitably patterned roller to produce an even non-slip finish.

- Trowelled non-slip finish: Close the surface by float and sprinkle silicon carbide or aluminium oxide in grain form mesh size No 20 to 26 at the rate of 1 kg per m2 and repeatedly trowel to produce a dense smooth non-slip wearing surface free from ridges and depressions.

- Dust-proof (or hardened) finish: Approximately 10 days after casting scrub any dirty areas clean with soapy water, wash off the entire surfaces with clean water and allow to dry. Inspect the concrete and chemically remove any oil or other contamination. Apply with a soft broom three coats of sodium silicate to BS 3984 in water (1 litre to 4, 3 and 2 litres of water respectively) allowing at least 24 hours between each coat. When the final coat is dry, wash with clean water.

- Falls: Form falls, currents, cambers and slopes where required. Ensure that water drains naturally towards outlets and other run-off points. Ensure there are no hollows or other catchment areas.

- Monolithic toppings:    ake precautions to ensure that any concrete left to receive monolithic toppings is undisturbed and protected from traffic or contact with other materials.

Precast concrete

Manufacture

- Limitations: Excepting only small units normally associated with masonry, such as lintels and padstones, all precast concrete shall be made off site under factory conditions in accordance with the appropriate BS.

- Site casting (large units): Obtain approval to cast large structural units on site. Submit proposals for the casting shed and procedures to be adopted.

- Site casting (lintels, etc): Make units permitted to be cast on site as follows:

a) Use materials and workmanship as previously specified for insitu work.

b) Vibrate the moulds with external vibrators or thoroughly rod by hand to ensure thorough compaction.

c) If units are reinforced, maintain 25 mm cover for internal faces and 40 mm cover for external faces.

d) Hook the ends of steel bars in lintels.

e) Cast in any fixings or mortices.

f) Key as previously described (See 675) all surfaces receiving insitu finishings.

g) Roughen all bedding surfaces.

h) Mark each unit with the date of casting.

j) Mark the tops of lintels.

k) Do not demould until 17 N/mm2 strength attained (this strength to be calculated).

l) Do not build in any unit until 28 days old unless approval is given.

Handling and fixing

- Storing: Comply with the basic requirements of General Requirements. Store reinforced units supported on their designed bearing surfaces in the position they will adopt when built in. Separately stack units with timber battens (non-staining and resin-free timber for fair faced units).

GS25	BRICKWORK & BLOCKWORK - GENERALLY

- Code of practice: Comply with BS 5628: Use of masonry subject to any qualifications given below.

Materials

Bricks

- Common bricks: Clay bricks to BS 3921:

- Calcium silicate bricks: Calcium silicate bricks to BS 187, Class 3 and above.

- Engineering bricks: Clay bricks to BS 3921, Class B.

- Concrete bricks: BS 6073: compressive strength 20N/mm2.

- Facing bricks: To match existing & approved by client. Must be frost protected / resistant.

- Brick standard-special shapes: BS 4729.

BLOCKS

- Dense concrete blocks with natural aggregate: BS 6073: compressive strength 14N/mm2.

- Lightweight concrete blocks: BS 6073: compressive strength 7N/mm2 .

MATERIALS FOR MORTAR

- Portland cement: BS 12 or BS 146.

- Sulphate-resisting cement: BS 4027.

- Masonry cement: BS 5224.

- Hydrated lime powder: BS 890.

- Sand: BS 1200, Table 1.

- Plasticiser: BS 4887.

- Pigment: BS 1014.

- Water: As specified in the Concrete Work section.

- Calcium chloride: Not permitted.

ANCILLARY MATERIALS

- Joint sealant: Two part silicone based mastic to BS 5889: to approved colour.

- Backing to sealants: An expanded polyethylene cord or strip as recommended by the sealant manufacturer.

- Airbricks: BS 493

- Copings: BS 5642

- Sills: BS 5642

- Tiles for sills and creasing: Plain clay tiles to BS 402.

- Clay flue linings: BS 1181:

- Clay flue terminals: BS 1181:

- Wall ties: BS 1243

- Damp proof courses: BS 743

- Cavity insulation: Dritherm 32.

- Precast concrete lintels: Factory made with steel bar hooked reinforcement to BS 4449 as detailed: crushing strength 25 N/mm2 at 28 days: marked    Top'. Sides keyed or fair as detailed:concrete as in the Concrete Work section.

- Prestressed precast concrete lintels: Designed in accordance with BS 5977: Part 1 and supplied by an approved specialist manufacturer to suit the spans and loadings which apply.

- Steel lintels: Pressed steel galvanized lintels designed in accordance with BS 5977: Part 1.

SAMPLES

- Sample bricks: Deliver samples of each facing brick to the client or the appointed specifier before any bricks are delivered to site.

STORING AND HANDLING MATERIALS AND COMPONENTS

- Damp proof courses: Store rolls on end on a level surface. Store preformed dpcs and cavity trays in a manner that will protect them from distortion.

Workmanship

- Preliminary work

- Setting out: Set out the building (or check the setting out). Ensure that pegs or other marks are securely fixed and protected from disturbance. Check for squareness with diagonal measurements. Provide clearly marked storey or gauge rods and any necessary templates to ensure the accuracy of the walls and the openings in them. Set out the starting course to allow for the position of openings etc to ensure that broken or irregular bond and cutting is kept to a minimum. Double check the first course of walling.

MORTAR TYPES AND MIXING

- Generally: Use only ready mixed mortar, except for small quantities by specific agreement.

- Mix proportions: Use mortars that comply with the following table. Use only one type for any one type of work. Adjust proportions within the permitted limits to suit the sand and weather conditions. Mixes are designated in nominal volume proportions of cement and dry hydrated lime to dry sand. For sulphate-resisting mixes, substitute sulphate-resisting cement for ordinary cement.

Proportions of mortar mixes

Type                                                        General        Blockwork    Below ground:

brickwork                                    exposed conditions

Cement:lime:sand                      1:1:5-6            1:2:8-9            1:1/4:3

Masonry :cement:sand        1:4-5                  1:6                    ...........

Cement:sand :plasiticiser 1:5-6                1:7-8                ...........

For more detailed information refer to BS 5628

- Generally: Mix site-proportioned mortars by machine, subject to clause 440. Stated mixing times are for guidance; mix materials sufficiently to obtain a uniform colour and consistency. Clean the mixer at least once a day and also when a different mix is used.

- Batching by volume: Measure sand, cement and lime (or cement and coarse stuff if applicable) each into a separate gauge box made to suit the proportions specified. Completely fill and empty boxes at each batching and keep the cement gauging boxes dry at all times. Crude gauging by the shovel is strictly forbidden.

- Assessing volume proportions: Calculate proportions of materials on the following basis:

Cement:                                    1440 kg/m3

Lime (dry hydrated):    575 kg/m3

Sand (dry):                            1520 kg/m3

- Note: when gauging cement with ready mixed coarse stuff, calculate cement addition based on the suppliers' quoted weight of coarse stuff per delivery.

- Batching by weight: Measure sand, cement and lime (or cement and coarse stuff if applicable) each separately in weight-batching plant in proportions specified. Adjust for damp sand.

- Control of mixer: Use an approved type of machine suited to mortar mixing. Carefully control the mixing in order that the materials are properly mixed to produce consistent batches of mortar. When all materials added, mix for 3 to 10 mins according to the type of mortar. Do not exceed recommended mixing times for plasticised/air entrained/masonry mortars.

- Measuring and adding water: Measure into a suitable container and do not add more to the mix than necessary to produce mortar of desired workability.

- Site preparation of coarse stuff: Mix dry hydrated lime with sand, first dry and then with water. Keep resultant coarse stuff at least 16 hours, protected as stated in clause 355.

- Cement: sand mixes: Add approx 3/4 of water and sand to mixer: gradually add cement and thoroughly mix: add remainder of sand. Add more of measured water if necessary to attain required workability. Mix for at least 5 mins.

- Cement: dry hydrated lime:sand mixes: Add approximately 3/4 of water and sand to mixer: gradually add cement and lime and mix thoroughly. Add remainder of sand. Add more of the measured water if necessary to attain desired workability. Mix for at least five mins.

- Plasticised cement: sand mixes: Mix correct dosage of plasticiser with the water. Add approx 3/4    of sand and adequate proportion of solution to mixer (taking care not to add too much solution at the start). Gradually add the cement and mix until incorporated, then add remainder of sand. Add more of the measured solution if necessary to attain the required workability. Cease mixing immediately the correct workability is attained.

- Masonry cement: sand mixes: Add approx 3/4 of sand and an adequate proportion of the water to the mixer (taking care not to add too much water at the start). Gradually add cement and mix until incorporated: then add remainder of sand. Add more of the measured water if necessary to attain the required workability. Cease mixing immediately the correct workability is attained.

- Cement:coarse stuff mixes: Add approx 3/4 of the water and coarse stuff to the mixer: gradually add cement and mix until incorporated and forming a thin lump-free paste; then add remainder of coarse stuff. Add more of the measured water if necessary to attain the required workability. Mix for at least five mins.

- Small quantities of mortar: Use hand mixing only for small quantities or when machine mixing impracticable. Measure materials into gauge boxes as stated in clause 385 and thoroughly mix on a clean smooth non-absorbent surface. Add 25 per cent more cement to the specified mix proportions, and up to 50 per cent more dry hydrated lime (if used). Do not add water until materials are thoroughly mixed dry to an even colour.

- Generally: No lime-gauged mortars shall be mixed on site except in respect of small quantities when it is impracticable to obtain ready-mixed mortar.

- Source of supply: Obtain ready-mixed lime: sand from a reputable supplier who will undertake to supply in accordance with BS 4721 Ready-mixed building mortars. Obtain dated certificates from the supplier verifying that the lime:sand mix complies with BS 4721.

- Site delivery: Provide a clean impervious surface for the tipping vehicle: cover the coarse stuff with waterproof sheeting.

- Cement addition: The approx weight of cement to be added per tonne of coarse stuff is as the table below

Nominal proportion            Cement in kg

cement/sand/lime

1:1/4:3                                                250

1:1:5-6                                                125-135

1:2:8-9                                                90-95

Method of adding cement: Add water and cement to the mixer.

READY-MIXED RETARDED MORTAR

- Generally: No mortar shall be mixed on site except in respect of small quantities when it is impracticable to obtain ready-mixed mortar.

- Source of supply: Obtain ready-mixed retarded mortar from a reputable supplier who will undertake to supply in accordance with BS 4721 Ready-mixed building mortars. Agree the retardation time with the specifier, selecting a time that is as short as possible in relation to site needs. Obtain dated certificates from the supplier stating the retardation time and verifying that the mortar complies with BS 4721.

- Site delivery: Provide a clean impervious surface for the tipping vehicle and cover the mortar with waterproof sheeting. Alternatively arrange for delivery in suitable containers that will protect the mortar.

- Do not re-mix: Remixing of retarded mortars or the addition of any further material is forbidden.

PRE-PACKED MORTARS

- Limitations of use: Pre-packed mortars may be used for small isolated portions of the work, subject to compliance with specification and BS 5838: Part 2 Pre-packed mortar mixes.

ALL MORTARS

- Cleanliness: Keep mixing and handling equipment clean. Remove traces of one type of mortar before another is mixed and used; this is particularly important with coloured mortars.

- Setting time: Use normal setting mortars within two hours of the addition of the cement to the mix. Use retarded mortars within the agreed setting time. Discard any mortar kept beyond these time limits.

- Prevent moisture loss: Dampen mortar boards and other absorbent surfaces; cover mortar not in use during drying conditions.

- Retempering: Excluding retarded and/or coloured mortars, a little water may be added if necessary to retemper mortar but only within two hours of mixing.

LAYING

- Scaffolding:    The building of putlogs or other restraints into the faced walls is not permitted.

- Wetting bricks: Except in cold weather, wet dry clay bricks sufficiently to prevent undue suction. Do not wet sandlime,    concrete or dense engineering bricks except sparingly in very hot or dry conditions. Avoid excessive wetting and do not wet brick stacks.

- Frogged bricks: Lay frog up (if two frogs, the deepest uppermost).

- Cellular bricks: Lay cavity down, unfilled.

- Bedding and jointing: Lay bricks in true and regular courses on a full, fairly smooth and levelled bed of mortar and fill all joints, frogs and other recesses solid (except cellular bricks). Press joints back with the trowel and strike off flush. Walls built against other structures to be filled solid at the back with mortar or grout. Keep mortar off the face of the work. In very dry conditions lay the mortar beds in short lengths to limit water loss before the bricks are laid.

- Joint thickness: Keep joints to a . consistent average thickness of 10 mm. Subject to approval, the specified joint thickness may be evenly modified to suit adjacent work. (eg a thicker joint will be necessary for dpc's). Check overall panel sizes in order to establish the correct joint thickness before commencing the work.

- Bonding: Where any bonding is unspecified, use a minimum bond of quarter unit and select a bond that will reduce cutting to a minimum and avoid irregular or broken bond. No bricks are to project into, or be set back from, the cavity.

- Tolerances: Not withstanding any other requirement , brickwork is to be arranged to achieve good fit with adjacent components/ construction. Where this cannot be achieved obtain instructions.

- Stability of walls: Do not lay bricks at such a pace that lower courses are disturbed. Temporarily brace large unsupported areas of wall if disturbance by winds or other influences could occur.

SLEEPER WALLS

- Honeycomb bond: Build sleeper walls under timber floors with 65-75 mm vertical joints unfilled to permit through ventilation.

CAVITY WALLS

- Keep cavities clean as follows:

a) Exclude mortar or debris by efficient means (eg laths, drawholes, thick rope).

b) Ensure there are no ledges, etc which could permit mortar accumulation.

c) Prevent any accidental mortar droppings from adhering to cavity trays or wall ties.

d) Leave temporary openings at the bottom and in other areas where mortar could accumulate and carefully clean out as necessary, avoiding damage to dpcs.

- Partial cavity fill insulation: Fix specified insulation boards as follows:

a) Do not use any boards that are warped (eg due to incorrect storage).

b) Build the insulated skin to a sufficient height above the board being secured to it, ie in advance of the other skin.

c) Unless otherwise specified commence the insulation below dpc level to overlap any ground slab insulation by at least 150 mm.

d) Do not push insulation into cavities under any circumstances.

e) Completely clean the wall face of mortar or projections and fix the insulation tightly against it with ties or other fixings as specified

f) Tightly butt the boards with staggered vertical joints, ensure there are no gaps in the insulation

g) Keep ties and cavity clean.

h) Protect the top of any insulation not extending to the top of the wall with a cavity tray if the gap between the insulation and the outer skin is less than 50 mm.

j) Ensure no insulation off-cuts or mortar fall into the cavity.

- Complete cavity fill insulation: Fix specified batts (slabs) as follows:

a) Ensure the batts are the correct thickness for the cavity width.

b) Build the outer leaf first to at least the height of the first row of batts, with ties projecting.

c) Unless otherwise specified commence the insulation below dpc level to overlap any ground slab insulation by at least 150 mm.

d) Do not push batts into cavities under any circumstances.

e) If batts fixed below dpc (as c) cut them at top to line of first row of ties

f) Space wall ties so that each batt is supported by at least two ties.

g) Fix the batts a row at a time, the bottom edge resting on the ties, the top edges pushed below the ties above.

h) Carefully and accurately slit batts and fit them over any additional ties around openings etc.

j) Carefully cut and tightly fit batts around openings, air bricks, services and other interruptions to the cavity

k) Carefully cut batts at wall angles (or bend if permitted).

l) Protect batts from mortar droppings, particularly at top edges.

m) Protect the top of any insulation not extending to the top of the wall with a cavity tray.

n) Fix all batts with their sides vertical, ie do not close gaps with any cut pieces with their edges vertical.

p) Stagger all vertical joints.

- Bottom of cavity: Form the bottom of the cavity at least 150 mm below the lowest damp proof course at any point.

- Top of cavity:    If bridged with insitu concrete , provide and leave in a strip of rigid durable formwork (e.g. mineral fibre board.)

- Coursing: Raise the skins of cavity walls together and approximately equally so that the difference in height does not exceed:

a) the vertical spacing of consecutive rows of vertical twist pattern ties

b) 1.2 m for all other ties. If one skin is constructed from units of a different height to the other skin, arrange the coursing of the unfaced skin so that appropriate horizontal courses regularly coincide during the height of the walls.

- Weep holes: eave perpends open at approximately 800 mm centres at the base of the cavity and where bridged.

- Build in specified ties as follows:

a) Ensure tie lengths are suited to the cavity width; ends to be embedded 50 mm.

b) Embed one end in the mortar of the higher skin with a slight fall to the outer face.

c) Space the ties as indicated in the table below, staggered and evenly distributed

d) Provide additional ties within 225 mm of the sides of openings at 300 mm vertical centres.

Spacing of wall ties

Min thickness Type of          Cavity            Ties    Spacing of ties:

either side              tie                      width              per        Horiz : Vert

(mm)                                                            (mm)              m2        (mm)      (mm)

65 to 90                  All                          50 to 75          5          450          450

90 or more          All                          50 to 75          2½        900          450

90 or more          Vertical          75 to 100        3          750          450

twist

90 or more          Vertical          100 to 150    5          450          450

twist

- Note: See BS 5628 for full details

- Openings: Seal openings through cavity walls with masonry and damp proof materials as specified. Line small ducts with slates or tiles rendered smooth.

- Filling cavities: Fill cavities below ground level with mortar or fine concrete as specified, the top sloping outwards.

- Sealing cavities: Do not seal without approval.

FACED WORK

- Generally: In faced work execute the highest quality of bricklaying.

- Bond: Stretcher or as otherwise stated on plans..

- Pointing as the work proceeds: Point the work with a concave (bucket handle) joint as soon as the mortar has sufficiently stiffened and press back to a firm profile.

- Finish level: Finish faced work level at the end of each day's work, ie do not leave faced work racked back overnight.

- Cutting: Minimise cutting by the use of standard special or purpose-made bricks. Undertake essential cutting showing on the face with a mechanical cutting disc or masonry saw.

- Overhand work: Do not undertake overhand work without approval.

- Keep clean: Protect the faced work as stated in clause 865.

- Efflorescence: Reduce the risk of efflorescence by keeping brick stacks and recently completed brickwork as dry as possible. Dry brush away any efflorescence that occurs; do not wash away.

FAIR FACE

- Meaning of fair face: This term together with building fair' means facing work not undertaken in facing bricks: it can also refer to the reverse side of faced-one-side bricks.

- Keep clean: Protect the fair faced work.

BLOCKWORK

- Generally: Subject to substituting the word block' for    brick' where appropriate, the preceding clauses shall apply here.

- Laying: Unless shell bedding is specified (see below), lay the blocks on a full (unfurrowed) bed of mortar with all joints and keys filled solid. Unless otherwise specified maintain a consistent 12 mm joint thickness. Adjust mortar stiffness to prevent extrusion by heavy blocks. Strike away any mortar squeezed from joints.

- Shell bedding: If specified, shell bed the outer and inner faces of hollow blocks in external walls with mortar strips in the horizontal and vertical joints on both sides of the wall.

- Bonding to other walls: Stretcher.

- Fairfaced work: Ensure that mortar is not smeared into the exposed face of the blocks. Brush away any accidental smears after the mortar has taken its first set.

ANCILLARY WORK

- General labours: Subject to any specified requirements, form all chases and fixings, etc as the work proceeds. Fully bed and solidly flush up lintels and other components built in. Ensure that the ends of lintels bear on whole units. Temporarily support or brace built-in items liable to distortion (eg frames). Perform all ancillary work in the mortar used for the surrounding brickwork except where strong mortars specified for weatherings and pointing, etc.

- Cutting into masonry: Cutting into masonry is subject to the following conditions:

a) Delay cutting until the mortar has hardened.

b) Obtain approval to cut into load-bearing walls or walls of hollow units.

c) Obtain approval to cut holes greater than 300 mm square.

d) Perform cuttings in a manner that involves minimal vibration to the wall.

e) Do not chase any walls less than 75 mm thick.

f) In solid walls do not chase deeper than one sixth wall thickness when horizontal or one third when vertical.

g) Do not allow chases to be located back to back in any wall.

- Lintels: olidly build in with 100 mm minimum end bearing (150 mm for steel lintels and boot lintels). Prop heavy lintels and all prestressed lintels until mortar has set. Ensure concrete lintels are built in the correct way up. Provide continuous support under reinforced brick or block lintels. See the Concrete Work section in respect of concrete lintels cast in place.

- Joist hangers: Build in plumb, bearing directly onto walls at the correct level (any adjustment to levels to be made at bed joints below the top course) and tightly abutting the wall face.

- Relieving arches: bearing bars: Bridge (by approved methods) over non-loadbearing ducts, services and other components which pass through walls and which exceed the length of two single wall units.

- Centering: Provide temporary centering for arches and other work requiring support during construction.

- Sills and thresholds: Bed one piece units of stone, concrete or similar material only at the ends to prevent possible fracture due to structural movement; point the open joint on completion of the work.

- Tile sills: Two layers of plain tiles laid broken joints set weathering and projecting 35 mm flush pointed.

- Tile creasing:    Single layer of plain tiles under new copings 35 mm both sides with weather fillets.

- Mechanical key: Unless otherwise specified rake out brick joints approximately 10 mm deep where a mechanical key is required. Do not rake out the joints of loadbearing walls or blockwork without approval.

- Tops of partitions: Unless otherwise specified, finish partitions below soffits as follows:

a) Loadbearing: Pin up to the soffit with drymix mortar well wedged and rammed into position.

b) Non-loadbearing: Fill solid at top with mortar.

c) Non-loadbearing : Provide head restraint

- Grouting: Mix grout in accordance with the specification for mortar, adding only enough water to make a pourable mix suited to its application in the Works.

- Pointing flashings, etc: Point flashings and weatherings (eg flexible materials; asphalt) turned into grooves or joints in a strong mortar.

- Ventilation ducts, etc: Construct small ducts, etc required through external walls with a slight fall to the outer face.

- Toothings: Weather any external toothings left for future extensions to a wall in a weak mortar.

- Corbelling: Unless otherwise specified, do not project any one corbel further than one quarter of the wall thickness;ensure the total corbelled projection does not exceed one half of the wall thickness.

- Flue linings: Build in specified flue linings as follows:

- With spigot joints down caulked with manufacturer's recommended compound and properly jointed; finish smooth internally.

b) Fill joints in surrounding brickwork with mortar.

c) Fill spaces between liner and brickwork solid with mortar.

d) Cut appliance flue pipes to finish flush inside liner.

e) Remove any debris and use a coring bag to ensure a smooth finish.

- Reinforcement: Totally embed reinforcement in the mortar joints. Position reinforcement in the joint as recommended by the manufacturer.

- Fire stopping:    Ensure that all gaps are sealed with mortar to prevent the passage of fire.

- Damp-proof courses: Work to flexible dpcs shall be as follows:

a) In cold weather warm rolls prior to unrolling to avoid cracking.

b) Lay an even bed of mortar for dpc and complete the bed to normal thickness and finish smooth.

c) Ensure mortar is free from any aggregate that could puncture the dpc.

d) In cavity walls provide a separate dpc for each skin and ensure that the edges do not project into the cavity (ie thus forming a potential mortar trap). Support cavity trays wherever possible (eg slates).

e) Fill cavities solid at the base of wall and slope towards the outer skin of the brickwork.

f) Use unjointed lengths as much as possible: lap at least 100 mm at passings: lap full width at angles.

g) Over openings and under sills in cavity walls, and in all other vulnerable locations, use unjointed lengths or seal the lap. Use stop ends to cavity trays over openings and at the ends of stepped and staggered dpcs.

h) Maintain continuity with any adjacent damp proof membranes and form watertight joints.

j) Extend dpcs 150 mm beyond lintels.

k) Meter boxes or the like: extend dpc 150 mm beyond and if cables penetrate the inner skin, take the dpc behind and below the box.

l) At jambs of openings in cavity walls provide dpc in close contact with frames properly fixed to prevent sagging with a full lap to dpc at head.

m) At sills provide dpc turned up at back and ends with front edge projecting 5 mm.

n) Ensure that dpcs are a minimum of 150 mm above ground level, with particular reference to stepped work.

p) Leave edges of dpcs projecting 5 mm from face of external walls. Do not point in mortar.

q) Lay and joint/seal all dpcs in a manner that will direct water to the outside face of walls. Provide adequate weep holes.

r) Ensure compatibility of all materials used in conjunction with adjacent materials.

s) Ensure continuity of insulation.

- Mastic sealants: Apply as follows:

a) Clean and prepare joint surfaces.

b) Apply any specified primer and allow it to dry.

c) Insert specified back-up material (normally plastic foam strips) to ensure sealant is forced against joint faces.

d) Apply specified bond-breaker tape where adhesion is to be avoided.

e) Use correct size nozzle on applicator.

f) Use two-part sealants as soon as mixed and within setting period.

g) Apply sealant steadily and firmly to ensure joint is filled.

h) Neatly finish the sealant with wetted spatula or by other suitable method to form a convex face; feather-edged and concave seals shall be cut out and re-formed.

j) Fillet joints are not acceptable under any circumstances.

WEATHER PRECAUTIONS

- Generally: Take precautions to prevent damage due to unfavourable weather of any description as stated in General Requirements. Take particular precautions as stated hereunder.

- Cold weather: Do not lay wet bricks. Do not lay mortar on frozen surfaces

PROTECTION

- Generally: Provide general protection as specified in General Requirements.

- Exposed tops of masonry: Cover at all times (when work is not proceeding) with waterproof sheeting securely fixed. Provide insulation if necessary.

- Splashing: urn up the inner board of any working platform not in use in a manner to prevent splashing of wall faces built fair or faced.

GS15	CARPENTRY - GENERALLY

- General requirements

- Code of practice: Comply with BS 5268: Parts 2 and 3 The structural use of timber subject to any qualifications given below.

- Materials

- Timber

- All timber: Clean, sound, properly seasoned timber free from any defects or combination of defects, natural or otherwise, making it unsuitable for its function in the Works. Sorted and selected at time of fabrication for suitability for purpose. NOTE: All timber is sawn unless otherwise stated. See 150.

- Named species: Timber species are specified by the standard names given in BS 7359.

- Structural timber: Strength class C16 to BS 5268. The contractor shall select a suitable species and grade to be supplied stress graded to BS 4978. Each piece of timber shall be marked with the grade.

- Grades of timber: See schedule showing the grades, moisture content and uses of structural timber.

- Non-structural timber: Natural defects which will not affect the durability or performance of the timber in use are acceptable. Not acceptable are knots exceeding one third width of the member or any evidence of decay. Regularised if necessary to provide true flat backgrounds for supporting other materials.

- Planed timber for fascias, and so on: Timber for fascias, verge boards and similar external work scheduled for painting shall comply with BS 1186: Part 3, Class 1, selected for suitability from Appendix A of the BS and free from pith, arris knots and plugs.

- Timber finishes and sizes: The following procedures apply:

a) Unless otherwise stated all softwood is deemed sawn and wherever possible sizes are those listed in BS 4471; other sizes not in accordance with this BS shall be sawn from appropriate larger sizes and wastage shall be included in the costs.

b) Regularised (machined on two or four sides) timbers are so described, and a tolerance of 3 mm for widths up to 150 mm or 5 mm for widths over 150 mm shall be allowed as stated in BS 4471.

c) Planed timbers are so described, and these are specified in finished sizes for which a deviation of 2 mm is allowed from stated sizes.

d) Planed kept clean: as (c) but selected for good appearance (for example, no stains or unsightly defects) and kept clean for clear finishing.

- Remarking: Where the grade marking is removed by machining, remark the timber prefixed by the letter    R' in accordance with BS 4978.

- Softwood for flooring: Planed tongued and grooved boarding to BS 1297.

- PRESERVATIVES

- Structural timbers/all timber: Treat the timber in accordance with BS 5268 Part 5 .

- Source of treatment: Inform specifier of the proposed process and firm undertaking the treatment. Provide certificate of treatment for every consignment of timber treated.

- Cutting: Apply treatment after any machining. Brush or spray the preservative in two applications to any cutting or boring subsequent to the treatment. Ensure that there is an adequate supply of preservative to hand if site cutting is envisaged.

- Drying: Undertake water borne preservative treatments in time for it to dry out before allowing the timber to contact any other materials or components, and to comply with the specified moisture content.

- MAN-MADE BOARDS

- Plywood: An exterior grade full waterproof bond plywood: for example:

- a) Canadian Douglas Fir, sheathing grade, unsanded

- b) Finnish Birch marked    Finply Exterior WBP'

- c) British plywood with Makore, Utile or Sapele Veneers, marked BP 101/65

- d) Swedish plywood marked P30 exterior glue.

- e) BS 6566 WBP Bond; Durability M.

- Plywood: Marine plywood to BS 1088 of balanced construction.

- Waferboards and OSB: Sterling Board manufactured by Highland Forest Products, Morayhill, Inverness, IV1 2JQ.

- Chipboard flooring: To BS 5669 Part 2 C2 tongued and grooved on long edges/all edges.

- Chipboard roof decking: To BS 5669, C4 tongued and grooved on long edges/all edges.

- Woodwool: To BS 1105, Type B/reinforced on long edges with rustproofed steel channels/with interlocking edges/prescreeded/.......

- Wood cement particle board: To BS 5669 Part 4......

- THERMAL INSULATION

- Fibreboard: Fibre insulating board/Bitumen impregnated fibre insulating board/Flame retardant fibre insulation board to BS 1142: Part 3.

- Expanded polystyrene boards: Type A Grade SD/HD/EHD to BS 3837.

- MECHANICAL FASTENINGS

- Carpenter's metalwork: Fabricate non-standard metalwork from mild steel in accordance with the Non-Structural Metalwork section. All the metal fastenings described below are steel unless otherwise stated.

- Corrosion protection: Protect all ferrous metalwork, nails, screws, bolts and other mechanical fastenings by galvanizing to BS 729 or other suitable plating for all external work, work in areas of high humidity or corrosive conditions. Sherardising shall comply with BS 4921.

- Nails: Round wire nails to BS 1202: Part 1.

- High grip nails: Annular ringed shank or helical threaded shank to BS 1202: Part 1.

- Screws: To BS 1210.

- Coach screws: To BS 1494: Part 2.

- Coach bolts: To BS 4933 (cup head nibbed bolts).

- Plain hexagon bolts: To BS 4190.

- Washers: Large diameter to BS 4320.

- Expanding bolts: A proprietary fixing comprising corrosion-resistant expanding insert and removable bolt or threaded stud to suit the work being fixed.

- Timber connectors: Galvanized split-ring/shear plate/toothed plate type to BS 1579.

- Straps: Galvanized pre-drilled factory made steel straps not less than 2.5 x 30 mm section/.......

- Joist hangers: To BS 6178, Type (a) or (b); for vertical loading only; to suit timber sizes.

- Frame cramps: Heavy galvanized frame cramps twice holed with fanged end for building in.

- ADHESIVES

- Timber adhesives: Cold setting synthetic resin to BS 1204, Type MR for internal work and Type WBP for external work and areas of high humidity. Ensure the adhesive is unaffected by any timber treatment.

- PREFABRICATED UNITS

- Generally:    Fabricate in properly equipped workshops in accordance with the principles and recommendations of BS 5268: Part 3.

- Information to fabricators: Ensure that the fabricators are provided with all necessary information which shall include the following:

a) Grade of timber if different from BS 5268: Part 3.

b) Moisture content, if required to be lower than 22 per cent at time of fabrication.

c) Truss spacings unless left to fabricator to design.

d) Roof pitch.

e) Span, that is, distance between lowest points of intersection of ceiling tie and rafters.

f) Distance between the outer edge of the wall plate and the intersection point in (e) if more than 50 mm.

g) Clear span between inside edges of wall plates.

h) Overhang, that is, distance between vertical cut end of the rafter and the intersection point in (e).

j) Type of roofing and maximum dead load.

k) Size, number and location of any tanks.

l) Size and location of any openings stating purpose (for example, access, chimney).

m) Details of any permanent lateral bracing.

n) Any camber required.

o) Any necessary drawings.

Preservative treatment if required.

- MEMBRANES

- Self-adhesive bituminous damp proof membranes:.....

- Polythene sheeting: 1200g Visqueen.

- REINFORCED ROOFING FELT

- Sarking felt: Reinforced breathable & untearable by Tyvek.

- SHEATHING FELT

- Bituminous sheathing felt: BS 747 Type 4A (i).

- Brown sheathing felt: BS 747 Type 4A (No 1 inodorous).

- VAPOUR BARRIERS AND DAMP PROOF MEMBRANES

- Vapour barrier: Polythene sheeting: 1200g Visqueen.

- BREATHER MEMBRANES

- Building paper: Breather type to BS 4016.

- STORING AND HANDLING

- Generally: Comply with the basic requirements of General Requirements    Do not store any timber or board materials in recently built wet construction; or anywhere unventilated.

- Timber: Store as follows:

a) Each grade separate and treated timber separate from untreated.

b) Off the ground on levelled bearers in a manner that prevents sagging and permits free air circulation. Separate timbers delivered in strapped packages.

c) If timber is damp or if a preservative is not fully dried out, stack as last but    in stick'.

d) Protect from the weather; keep any temporary covers from close contact with the timber.

- Prefabricated units: Handle and store as follows

a) Use adequate labour to handle singly or handle banded units with mechanical plant, in either instance without causing stress or distortion.

b) Store upright supported at the designed bearing points; provide secure temporary stays to maintain upright; or

c) Store flat and protect as stated above for timber.

- Board materials: Stack flat off the ground on levelled bearers and protect from the weather. Handle without damaging edges, particularly chipboard. Lift boards off stacks, do not drag over the edges of lower boards.

- Timber scheduled for clear finishing: Keep clean (particularly from any staining or the like not easily sanded away).

- Workmanship

- Generally: The following clauses apply to structural timbers. In respect of non-structural and general carpentry work, the more rigorous requirements will be waived subject to the production of work which is fit for its intended purpose.

- First and second fixings: If undertaking any work described in the joinery specification, comply with the relevant requirements given therein.

- Check moisture content: Before fixing any timber, check the average moisture content with a properly calibrated reliable meter. If the percentage is 2 per cent or more above the specified time-of-fixing content, inform the specifier. Do not accept delivery of any prefabricated units unless under 22 per cent moisture content.

- Selecting timber: Only use timber with defects inside those in BS 4978.

- Shaping and jointing timber: Unless otherwise detailed, undertake the work as follows:

a) Saw timber truly square.

b) Bore at right angles to the face.

c) Make joints and provide bearings in a manner that brings and maintains all surfaces in full contact. d) Make joints that tighten under load and resist tension as appropriate to their function.

e) Do not make joints in the length of a structural member unless specified or approved. See clause 370.

f) No defects which reduce the strength of the connection are permitted at joints, bearings or assembly locations.

g) Do not cut, notch or modify timber other than as specified.

h) Nail/screw/bolt joints as necessary. Site glued joints are not permitted unless under special circumstances.

- Joints in the length: See clause 365 (e). These joints are only permitted if:

a) They are detailed on drawings.

b) They can be shown by calculation to develop the full strength of the member.

c) They are made in an approved manner over bearings.

- Nailing: Nail as follows:

a) Use nails of adequate length to provide a secure fixing.

b) Where appropriate, drive on the slant so that the connection does not loosen under load.

c) Avoid splitting by sensible spacing; and pre-drilling if necessary.

d) Do not pre-drill in excess of three-quarter nail diameter NOTE: Do not use masonry nails.

e) Drive screws tight; to facilitate this and to avoid splitting: (i) Pre-bore for the length of, and to the diameter of, the plain shank. (ii) Pre-bore for the diameter of the thread core.

f) Countersink for countersunk screws.

- Plugging: Use durable proprietary fibre composition or plastic plugs let into drilled holes for screw fixings to concrete, masonry and blockwork. Do not nail direct to    nailable' masonry without approval.

- Bolting: Bolt as follows:

a) Use bolt lengths that will project at least one complete thread beyond the head of the nut without undue tightening.

b) Drill holes at right angles to the faces being brought together.

c) Drill holes slightly larger than bolt diameters unless timber has dried to the moisture level in service.

d) Use large thick washers under head and nut.

e) Do not split out the timber on the reverse side when drilling. If necessary to prevent this, drill from both sides after a small lead hole.

g) Tighten so that washers just compress the timber. Do not tighten so much that the washer deforms.

- Glued joints: Make joints designed to be glued as follows:

a) In controlled workshop conditions.

b) Moisture content not exceeding 20 per cent

c) Using an approved proprietary adhesive.

d) Joint faces freshly prepared, cleaned    and clamped together in full contact under pressure.

- Masonry bolts: Ensure adequate penetration into masonry; do not fix expanding bolts near edges of masonry; use resin anchors instead. Use large washers for timber as stated above.

- Timber connectors: Fix as follows: a) Pre- assemble the members being connected and nail or clamp before drilling.

b) Accurately locate the bolt hole by intersecting the centre lines of the members.

c) Use a jig or template for repetitive work.

d) Embed toothed-plate connectors before bolting by compressing the joint faces with a temporary threaded high tensile steel rod with a large thick washer and nut on both sides, or by cramping or other means of applying pressure.

e) Cut grooves for split-ring or shear plate connectors with special tools recommended by the connector plate manufacturers and remove all wood particles before assembly.

f) Using bolts and connectors of the correct size and type, bolt together as clause 390.

- Straps and anchors, and so on: Fix as follows:

a) At each timber contact fix with a minimum of two galvanized screws when the number not detailed.

b) When strapping joists and rafters, and so on, use straps to span at least two members.

c) When strapping to masonry, block any gaps between timber members and masonry.

d) Ensure that any strap taken into a masonry wall cavity is turned down tight against the inner skin for an adequate distance.

e) When fixing straps to tops of masonry walls (for example, lateral restraint at gables) ensure that the fixing is not limited to a small masonry unit (such as a cut block, a single brick).

f) Ensure that all faces intended to be in contact are in close contact without unspecified bending and improvisation.

- Joist hangers: Unless otherwise indicated, use screw-to-wall hangers. Select hangers of the correct size to fit the timber and to suit the loadings. Fix as follows:

a) With all supporting or fixing surfaces in close contact with the background and the timber being supported. A 6mm (max) gap is permitted between end of joist and hanger back plate.

b) Nail to the timber being supported through all pre-drilled holes.

c) Ensure joists are properly levelled and undertake any unavoidable notching carefully and accurately.

- Framing anchors: Subject to approval of the proposed type and locations, framing anchors may be used instead of timber joints. If required, specify them after 285 (they are ideally specified in general terms, such as min metal thickness, corrosion treatment).

- Wall plates: If possible fix wall plates in one piece between changes of direction; otherwise make 100 mm lap joints. Bed in mortar truly level and fix with straps where necessary. Plug and screw or strap to lintels.

- Bearings: Bear timbers in full contact on designed supports. Liberally coat with preservative ends built into masonry. Protect from contact with potentially damp surfaces and structures. Where packings are necessary use material that will not rot or compress.

- Trimming: Properly trim around openings and make suitable joints or provide joist hangers. Ensure upper and lower surfaces are correctly aligned and that trimmers fit tightly. Wherever possible transmit loads to other sound structures.

- Joists: Use regularised timber to ensure level supports for floors and linings, and so on.

- Studding: Construct with tight butt joints securely nailed. Securely fix the top of partitions but ensure that no unintended loading is transmitted from structures above. Use regularised timber when truly flat backgrounds are required.

- Nogging: Provide noggings wherever required. Plug and screw noggings if end fixing prevented by joist hangers.

- Blocking next to walls: Pack between masonry walls and joists, rafters, trusses, and so on with timber blockings at appropriate centres. Keep joists approximately 50 mm from parallel masonry walls.

- Blocking and strutting between joists: Prevent bowing and twisting by solid blocking at all bearings and solid or herringbone strutting at intermediate span as appropriate. Ensure herringbone strutting is in full contact and properly nailed and does not project.

- Bracing: Provide all necessary temporary bracing and brace all rafters and trusses to resist wind forces as in the roof designers's specification.

- Floor boarding: Lay tongued and grooved softwood boarding as follows:

a) Before commencing ensure that joists or other supports are truly level.

b) Lay with staggered heading joints tight butted centrally over supports.

c) Cramp up without damaging edges and twice nail each board to every support; punch nail heads below surface.

d) Remove tongues at access traps and fix with screws; provide noggings as necessary around access openings.

e) Leave perfectly flat and smooth on completion; sand or plane where necessary.

- Chipboard flooring: Lay tongued and grooved chipboard flooring as follows:

a) For two days before fixing, store the boards in conditions similar to those which will occur in the completed building.

b) Before commencing ensure that joists or other supports are correctly spaced and truly level with a firm support at all perimeters.

c) Lay with long edges at right angles to the supports with staggered cross joints centred over supports.

d) Leave movement gaps at abutments based on 2 mm per metre of flooring but not less than 10 mm at perimeter abutments.

e) Nail at 150 mm/....... centres to each support and punch below surface.

f) Provide noggings under all edges without tongues.

g) Form access traps with softwood/hardwood lipped edges and fix with screws: provide noggings as necessary around access openings.

h) On completion sand any edges which are not flush.

- Plywood roof decking: Lay plain edge plywood decking as follows:

a) Before commencing ensure that joists or other supports are correctly spaced and aligned level or to falls. Provide any firrings required.

b) Lay with long edges at right angles to the supports with staggered cross joints centred over supports.

c) Leave movement gaps between boards based on 3 mm at edges and 1.5 mm at ends (based on nominal 2400 x 1 400 mm boards).

d) Fix with flat headed galvanized nails (length 2 times board thickness) at 100 mm/....... centres to each support. Hammer the heads flush.

e) Provide noggings under all edges without tongues.

f) On completion sand any edges which are not flush.

- Chipboard roof decking: Lay tongued and grooved chipboard decking as follows:

a) Before commencing ensure that joists or other supports are correctly spaced and aligned level or to falls as detailed. Provide any firrings required.

b) Lay with long edges at right angles to the supports with staggered cross joints

c) Leave small movement gaps between all the edges of boards and a min. 10 mm gap at abutments.

d) Nail at 100 mm/....... centres with galvanized flat headed nails (length 2 times board thickness) and hammer flush with surface.

e) Provide noggings under all edges without tongues.

f) On completion sand any edges which are not flush.

- Woodwool roof decking: Lay as follows:

a) Before commencing ensure that joists or other supports are correctly spaced and aligned level or to falls. Provide any firrings required.

b) Lay with long edges at right angles to the supports with staggered cross joints centred over supports.

c) Fix with large headed galvanized nails at 150 mm/....... centres taking care to not damage the slabs.

d) Cover joints between pre-screeded panels with 100 mm hessian scrim bedded in sand:cement (1:1) slurry finished smooth.

- Erecting roof trusses: Check that the trusses are undamaged and comply with specification. Fix as follows:

a) Lift as directed by fabricator without causing any stress or distortion.

b) Position correctly and truly vertical.

c) Provide temporary bracing if necessary.

d) Fix on wall plates within 50 mm of the rafter/ceiling tie connection. Do not permit rafters to bear on wall plates.

e) Securely fix all permanent braces and binders and ensure that those in the end trusses tightly abut walls.

f) Fix any specified straps as clause 405.

g) Do not modify in any way without approval. Replace (that is, do not repair) any damaged trusses.

- Work with planed timber: Properly joint together and fix with the joints and fixings concealed. Scribe to backgrounds where necessary. Slightly round all arrises. Treat any part of the work which abuts structures with two coats of preservative compatible with the paint finish. Leave the work ready for decoration.

- Fascias and other external woodwork: Comply with the last clause. Fix with rustproofed fixings and fix in a manner that sheds water naturally. Provide water check grooves where necessary. Do not fix directly to structures; provide sawn preservative treated grounds where necessary and leave air gaps at edges.

- Weatherboarding:    150mm Rough sawn treated timber.

- Flexible sheet membranes: Requirements are as follows:

a) Where mechanically fixed, use large headed fasteners.

b) Ensure the background is fit to receive the sheeting; remove any projections that might cause splitting or puncturing

c) Fix or lay sheets without stretching or creasing.

d) Lap at least 100 mm but only at supports.

e) Lap in a direction that naturally excludes water.

f) Efficiently seal (for example, with s/a tape or adhesive) laps in vapour or water barriers.

g) Similarly seal to adjacent dpc's or membranes.

- General framing and sundry non-structural carpentry work: Provide grounds, battens, bearers, packings, bracketing, cradling, skeleton frameworks and support work of every description for fixings and joinery work, and so on as detailed and in accordance with the following general requirements.

a) Fix with nails, screws and halved joints and so on as necessary to achieve secure fixings.

b) Where necessary wedge into steelwork and provide additional fixings.

c) Fix at spacings to suit the linings and components, and so on being fixed.

d) Provide additional supports around access doors and the like.

e) If necessary use regularised timber when truly flat backgrounds are required.

- Damp locations: Prevent timbers from contact with potentially damp surfaces and structures or provide suitable protection (for example, two flowing brush coats, the first allowed to dry, of preservative).

- Ventilation: Ensure that nothing (for example, insulation) obstructs the designed ventilation routes of the roof structure.

- Quilt / mat insulation: Fix specified insulation of this type as follows:

a) Lay in the largest sizes that are practicable.

b) Lay tightly butted between pieces and up to boundary surfaces.

c) Before laying remove any debris, shavings etc. and seal around any openings for pipes, conduits etc.

d) Keep back from ventilation routes.

e) Do not extend under cold water storage cisterns unless so directed.

f) Do not cover any electric cables without approval.

g) Cut and/or fit tightly around obstructions.

h) Ensure semi-rigid mats which are friction-fitted between timbers are not prone to sagging or slipping; if so, seek instructions (e.g. netting support).

j) Fix vertical insulation at head when suspended.

k) Fix to access hatches and other traps etc. in a manner that will permit their opening but maintain the insulation efficiency.

- Fire stopping: Compressed mineral wool forced into the gaps.

- Carpentry work for services: Subject to approval, undertake general builders work for services installations as follows:

a) Make access traps and fix with screws for ease of removal and replacement.

b) Make holes and notches, and so on where required; make as small as practicable, not closer than 100 mm apart and within the following limitations.

c) Make U-shaped notches by sawing down to a drilled hole; locate only in the upper edge of the outside quarters of beams and joists; max. size one-eighth depth of member.

d) Drill holes through centre line within centre two-thirds of span of beams and joists; max. size one-sixth depth of member.

e) Make notches and holes in other members in locations of least stress and never adjacent to point loads.

f) Fix 2 mm minimum flat steel plates over services in notches under floor boards to prevent nail damage; let plates in flush with top of joists.

g) Provide supports for water cisterns; ensure loads are adequately spread between suitable supports. Use at least three roof trusses to support a 225 litre (actual) cistern, at least four for 315 litres. Provide fully boarded support for plastic cisterns.

h) Provide and fix battens for services equipment and plug walls and so on where required.

- INSPECTION

- Generally: Inspect all structural carpentry. Check that the roof structure provides adequate lateral restraint to the tops of walls (that is, all straps, blocks and noggings in position and fixed as specified). Check all bolts and tighten if necessary; tighten again at end of maintenance period.

- Cleaning: Ensure that all shavings and offcuts are collected up and removed from site. In particular leave no surplus materials in concealed spaces where they could attract vermin or contribute to a fire.

- PROTECTION

- Generally: Provide general protection as specified in General Requirements.

- Floors: Protect floors; lay temporary hardboard sheets or other suitable covers if any floors subjected to construction traffic.

GS20	JOINERY - GENERALLY

- General Requirements

- Code of practice: Comply with BS 1186 to the extent stated here.

- Materials

- Timber

- All timber: Properly seasoned ,straight timber free from any defects or combination of defects, natural or otherwise, making it unsuitable for its function in the works. Sorted and selected at the time of fabrication for suitability for purpose.NOTE: All timber is planed on all faces unless otherwise stated See 195 .

- Named species: Timber species are specified by the standard names given in BS 7359.

- General Joinery: Timber for joinery to be decorated with opaque coatings shall comply with BS 1186: Part 1 as follows:

a) Hardwood or imported softwood Class 2 and Class 3 selected for suitability of usage from Appendix B and C but excluding hardwood described as resinous.

b) Average percentage of moisture content as recommended in Section 3 and Table 2.

c) No exposed pith, arris knots, plugs or inserts permitted on any faces, concealed or otherwise, in external joinery.

- Softwood for joinery: Selected for high quality work from better shipments of unsorted Swedish, Finnish or Russian Redwood or selected merchantable Douglas Fir. Moisture content 9 - 16 per cent according to location of joinery. Growth rings minimum 10 per 25 mm. Grain slope maximum 1 in 10. Free from decayed or loose knots, splits and any sign of fungus/insect attack except pinholes. Permissible defects to be excluded from finished surfaces are:

a) Checks and shakes wider than 3 mm or longer than 300 mm or deeper than one quarter the full corresponding dimension of the piece.

b) Pitch pockets and pith.

c) Arris and splay knots; knots over 6 mm average diameter.

- Clear-finished joinery: Timber for joinery specified as    clear finished' to comply with BS 1186: Part 1 Class CSH.

- Clear-finished joinery: African Mahogany/...... to BS 1186: Part 1 Class 2/Class 1 to BS 1186: Part 1: 1991 Class CSH.

- Sapele/Iroko: Selected for clear finishing from Selects and No 1 Common or equivalent grading, as follows:

a) No evidence of decay or insect attack.

b) Clean and straight with clearly defined arrises.

c) Grain slope not exceeding 1:8.

d) Entirely free from splits, shakes, compression wood, sapwood, pith and brittle heart.

e) Exposed faces showing the same character of grain throughout and free from knots, stains, discolourations and checks.

- Teak, Afrormosia: Selected for very high quality joinery from FAS (Firsts and Seconds) or equivalent grading.

- Hardwood to sample: Provide hardwood to match the sample(s) that may be seen at the specifier's offices.

- Specifier will select hardwood: Allow for hardwood/...... at the prime cost of ...... per ft.3/m3sawn, kilned and delivered to workshops.

- Timber for grounds, etc: Timber permanently concealed in the finished work shall be free from any defects likely to adversely affect the stability or accuracy of the work and shall be treated with preservative as required for carpentry work.

- Timber for lippings and beads, etc: An almost straight grained timber with good machining qualities.

- Decorative timber veneers: Cut from...... min 0.6 mm thickness. Selected to match adjacent similar veneers and/or solid timber of the same species. See clause 290 for balancing veneers.

- Timber sizes: Timber is specified in finished sizes for which no deviation is allowed from any stated size.

- Seasoning and moisture content: Seasoned timber to the specified moisture content before fabrication. If in doubt, ascertain the mean humidity and temperature range finally obtained in the completed building. Prepare kiln drying schedules to ensure that the varying requirements governed by different species, sections and eventual usage are taken into account to minimise kilning defects.

- PRESERVATIVES

- All softwood/softwood for external joinery: Treat the timber in accordance with BS 5589.

- Procedure: Treat the timber after machining and before assembly/Treat the timber after fabrication/Fabricate the joinery from treated timber: brush apply the manufacturer's preservative to all cut or machined surfaces before assembly.

- MAN-MADE BOARDS AND SHEETS

- Plywood: Manufactured from hardwood or softwood and in compliance with BS 6566: Grade II on both faces.

- Plywood: This provides for what the BS terms a general use plywood which has both faces suitable for leaving visible; which will mean also suitable for painting or overlaying. If one face is to be unseen, a cheaper board could be specified as one face Grade III. MR bonding refers to a moisture resistant adhesive to BS 1203; specify INT for dry service and WBP for full waterproof bond. See BS 6566 for full information and particularly Part 7 if it is necessary to specify the durability class.

- Blockboard: Birch faced 5 ply with veneers and glue bond as 215      300.

- Laminboard: With veneers and glue bond as 215 & 330.

- Chipboard: To BS 5669, C1-C5

- Hardboard: To BS 1142, Type S (standard grade)/Type TN (tempered grade)/Type TE (tempered grade).

- Low density fibreboard: Medium board to BS 1142, Type LMN/Type LME.

- High density fibreboard: Medium board to BS 1142, Type HMN/Type HME.

- Flame retardant hardboard: Flame retardant low density fibreboard:

- Laminated plastic sheets: To BS 3794, Part 1 min 1.5 mm in an approved manufacturer's standard pattern range matt finish.

- Balancing veneers: Any board material (which shall include plywood, laminated or particle boards, flush doors) described as veneered one side shall have a compensating veneer on the reverse. This requirement applies to wood, plastic or other veneering material.

- ADHESIVES

- Generally: Synthetic resin to BS 1203 or 1204 (as appropriate) of the correct class for the duty and the type of joint.

- Glue bond: Unless otherwise stated:

a) INT for internal work.

b) MR for internal work in areas of humidity.

c) WBP for external work.

- MECHANICAL FASTENINGS

- Corrosion protection: Protect all metalwork, ferrous nails, screws and other mechanical fastenings by sherardising, galvanizing or other suitable plating for all external work, work in areas of high humidity or corrosive conditions. For acidic woods (such as oak) use brass or stainless steel.

- Nails: To BS 1202 Part 1:1974. For decorated work, lost headed nails suitable for stopping.

- Screws: To BS 1210.

- Cups for screws: Brass or stainless steel heavy pressed socket pattern.

- Expanding bolts: A proprietary fixing comprising corrosion-resistant expanding insert and removable bolt or threaded stud to suit the work being fixed.

- Frame cramps: Heavy galvanized steel cramps twice holed and fanged for building in.

- Dowels: Galvanized steel dowels not less than 12 mm diameter x 100 mm.

- FLUSH DOORS

- Terminology: In this specification the following terms apply: Semi-solidRefers to any flush door with a core other than solid, regardless of the construction. Solid core Refers to any flush door with a core of laminated solid timber strips.

- Flush doors generally: All flush doors and panels, whether contractor-made or proprietary, to be as follows:

a) Of dimensions and tolerances in accordance with BS 4787:Part:1 1980 (1985)

b) Lippings , glue bond and fixings as previously described.

c) Perfectly flat, smooth facings, free from core patterns in accordance with BS 5277:1976 (1985) and BS 5278:1976 (1985)

d) Suitable solid blockings in semi-solid doors, their locations marked.

e) When described as    fire doors'. constructed to pass the fire tests described in BS 476: Part 22 1987 for the time period stated.

- Solid core doors: Subject to 350, construct in one of the two alternative methods as (a) or (b) at the contractor's option.

a) Cut from blockboard as previously specified and hardwood lipped all edges.

b) Constructed from 18 mm max. width solid timber core strips butt jointed and glued with each strip having grain reversed, 4 mm plywood facings and hardwood lipped all edges.

c) Apertures for glass: rebated hardwood lipping with hardwood beads fixed with brass screws and cups.

- Fire doors:To BS 8214:1990. Comply with BS 476: Part 22 1987 on all designated fire doors and provide test reports    for the period and dimensions indicated as required.

- Intumescent seals: Comply with BS 476:Part 23:1987. Provide intumescent seals let into grooves in the top and the vertical edges of all fire doors : use approved seals fixed in a manner so that the doors will pass the stated fire ratings.

- Intumescent seals: This is an alternative to 380 when a more precise specification    is required. Consult the Intumescent Fire Seals Association (IFSA), 20 Park Street, Princes Risborough Bucks HP27 9AH, which publishes useful information sheets or specialist manufacturers.

- Combined intumescent/cold smoke seals: Comply with BS 476: Part 23 and Part 31.1. Provide combined intumescent/cold smoke seals , let into grooves.

- Veneered doors: Ensure the veneers on any pair of doors match; provide a paired sample for approval before general purchase or fabrication is put in hand. Approved samples may be used in the works.

- FITTINGS AND FURNITURE

- Generally: Materials and workmanship for metal and glass, etc forming an integral part of joinery fittings shall comply with specification for the work sections concerned.

- STORING AND HANDLING

- Delivery and storage: Comply with the following:

a) Programme delivery so that joinery will not be subject to avoidable changes in humidity.

b) Transport under total weather protection.

c) Off-load directly into conditions of storage suited to the moisture content of the joinery.

d) Store items necessarily built-in as first fixings off the ground on levelled bearers in a manner that gives weather protection, air circulation and prevents distortion.

e) Do not regard priming as a form of protection.

- Board materials: Stack flat on levelled bearers or stack upright on long edges on purpose-made racks which provide full support slightly out of vertical. Handle without damaging edges. Lift boards off flat stacks, do not drag over the edges of lower boards.

- Ancillary materials: Keep dry.

- Workmanship

- Generally: Produce joinery in accordance with good quality joinery practice as follows:

a) Undertake as much fabrication as possible in humidity controlled workshops equipped with modern machinery manned by skilled joiners.

b) Restrict site work to fixings and other operations that cannot be undertaken as last stated.

- Fabrication and jointing: Subject to any qualifications given below, make joints in accordance with BS 1186: Part 2.

- Fabrication and jointing: Make joints exactly as detailed on the drawings. Where any joints are not indicated, make similar joints to those detailed for similar applications and conditions of use.

- Fabrication and jointing: Joints are not indicated on the drawings which are intended to show the dimensions and final appearance of the joinery. Produce joints by traditional or modern machine-shop practice that comply with the following:

a) Use joints designed to tighten under stress and to not form a weak link in the assembly

b) Wherever possible, use joints that are not wholly dependent upon adhesive.

c) Provide for movement in joints where necessary (e.g. panels fitted into grooves) otherwise glue all joints.

d) Design joints to conceal the end grain of natural wood or the edge of laminated or particle boards.

e) Make joints capable of being assembled dry as a push fit with all joint surfaces in full contact and only fine joint lines visible.

f) Make joints in external work to naturally exclude moisture.

- Assembly joints: Provide assembly joints in large joinery units to facilitate transport and handling. Obtain approval for such joints. Strong concealed joints assembled with rust-proofed greased screws will be suitable.

- Veneers: Comply with the following:

a) Ensure that no fault of the background    photographs' through to the face.

b) Ensure that the core/background is properly conditioned (e.g. moisture content).

c) Apply laminated plastic veneers in accordance with the recommendations of the British Plastics Federation.

d) Use the correct adhesive and remove any traces.

e) Apply a balancer where necessary -see 290

f) Match patterns and tightly butt all joins

g) Use mechanical presses whenever possible.

h) Chamfer external angles unless otherwise specified.

- Finished appearance: Provide the specifier with the opportunity to inspect joinery before any form of priming or decoration is applied. Requirements for the finished appearance of joinery are as follows:

a) Joinery to receive decoration: a fine smooth surface free from any imperfections that might be apparent after decorating.

b) Joinery for clear finishing: finely finished to remove cutter or sanding marks, raised grain, stains or other blemishes.

c) Angles and edges slightly rounded; exposed edges and vulnerable angles pencil rounded (subject to details).

- Moving parts: Moving and closing parts should fit accurately and easily. Visible gaps between fixed and moving parts should be a consistent width not exceeding 1 mm/1.5 mm/2 mm/2.5 mm.

- Mouldings: To be worked on solid unless otherwise specified.

- Trim: Properly mitre or scribe as necessary. Pin and stop for decoration. Fix clear finished work with brass or sherardised pins and stop with matching stopper or otherwise conceal.

- Backboards, etc.: Bevel all round.

- PRIMING AND PAINTING

- Priming: Do not prime or deliver any joinery until approval is given whereupon knot and brush apply two coats of primer as specified in the Painting and Decorating section.

- Priming: Knot and brush apply two coats of primer as specified in the Painting and Decorating section to all joinery before delivery to the works.

- Pre-treatment of clear finished joinery: Brush apply the first coat of specified clear finish before delivery to the Works.

- Bottoms of doors: Paint the full specified finish to bottoms of doors before hanging.

- Backs of frames, etc.: To the backs of all joinery abutting structures brush apply two coats of primer as specified in the Painting and Decorating section/two finishing coats of paint as specified in the Painting and Decorating section/two coats of gloss varnish/a coat/two coats of......

- FIXINGS

- Generally: Subject to specified requirements, undertake secure fixings as follows:

- Unseen fixings: Strength and durability are the only requirements.

- Fixing materials to be decorated: No fixings apparent unless designed for removal (e.g. glazing beads).

- Fixing self-finished and clear finished materials: Glueing, back screwing, secret nailing or other approved unobtrusive methods; alternatively, fix by approved complementary matching methods (e.g. domed screws).

- Fixing brittle materials: Pre-drill potentially brittle materials (e.g. hard compressed man-made boards of the fibre-cement type).

- Fixing clear finished hardwood: Unless secret fixing is practicable, fix decorative hardwoods specified by species with non-ferrous or plated screws with heads let in and pelleted with grain matched plugs of 6 mm minimum thickness cut from the same timber.

- Fixings to concrete and masonry, etc.: Use plugs and screws or expanding bolts as appropriate. Use durable proprietary fibre composition or plastic plugs let into drilled holes. Do not nail direct to    nailable' masonry walls without approval.

- Grounds and packings, etc.: Provide grounds, battens, bearers and packings as necessary for fixing joinery.

- Building in frames, etc.: Temporarily fix frames and build in as follows:

a) Partly remove any horns and brush with preservative.

b) Position truly plumb, check the diagonals and fix temporary bracing and cross-bracing to hold secure.

c) Provide and fix dowels, cramps and waterbars as necessary. Fix cramps at 600 mm maximum centres and not more than 150 mm from top and bottom.

d) Ensure that no construction loads (e.g. lintels) are transferred to frames.

e) Ensure that two generous coats of finishing paint (as specified for painting) are applied all round the back of untreated frames.

f) Ensure that dpc's are fixed to external frames in correct position with clout nails.

g) Ensure that mortar or other specified material is packed between frames and structures.

- Pointing: Deal with this in more detail if it is required (e.g. it could be kitchen/bathroom fittings or it could be around    external door or window frames). This clause is intended to be a reminder.

- Conditioning board materials: Before fixing, condition hardboard, fibreboard and other man-made boards at suitable temperature and humidity levels for an adequate time.

- Templates: Provide templates where necessary (e.g. for precision units fixed into formed structural openings).

- IRONMONGERY AND ACCESSORIES

- Architectural ironmongery: The cost of supplying the ironmongery and screws to the site is included in a prime cost sum. Allow for the cost of screws for butts.

- Architectural ironmongery: Provide ironmongery as scheduled.

- Samples: Provide and maintain on site a sample board containing all selected items of ironmongery.

- Fixing and unfixing: Fix all architectural ironmongery with matching screws. Remove and refix to facilitate decoration or other work. Ease and adjust and oil on completion. Label all keys and hand to specifier.

- TOLERANCES AND STANDARDS OF ACCEPTANCE

- Fixing tolerance: Do not exceed a total accumulative tolerance of 6 mm between each joinery item and structures unless architraves or other masking forms part of the design.

- Joinery tolerance: No tolerance will be allowed for joints in joinery. Should any joints open or moving parts fit other than accurately and evenly, or should any defects of timber other than those permitted be apparent, the joinery shall be repaired or replaced as directed.

- PROTECTION

- Generally: Provide general protection as specified in the section on General Requirements.

- Exposed joinery: Case up stairs, handrails, door frames and other joinery exposed to damage. Wedge up any doors in constant use by work people. Fix architectural ironmongery as a final operation or else wrap it as soon as fixed.

GS25	STRUCTURAL METALWORK - GENERALLY

- General requirements

- Code of Practice: Comply with BS 5950: Part 2 Structural use of steel in building subject to any qualification given below.

- Conditions: See the sections on General conditions and preliminaries.

- Requirements: Comply with the section on General requirements which shall be read in conjunction with (and forms an integral part of) the structural steelwork specification.

- Choice of specialist firm: Ensure that the steelwork is undertaken by specialists who are experienced at working in accordance with BS 5950 and the publications of the British Constructional Steelwork Association, in particular this association's Quality Assurance Certification Scheme.

- Reference specification: This project specification is based upon, and hereby invokes into the contract documentation, the National Structural Steelwork Specification for Building Construction published by the British Constructional Steelwork Association, 4 Whitehall Court, Westminster, London SW1A 2ES (Tel: 071-839 8566). The tender shall be based upon this document, subject to any qualifications hereunder, and shall be kept at the fabricator's works and also at the site during the period that the steelwork is being fabricated and erected.

- Quality assurance: Operate a quality assurance system in accordance with Section 10 of the BCSA Specification referred to above.

- Engineer's specification: See the specification for structural steelwork which follows this specification/which is provided after the architects' specification/which will be found on pages..../which is printed on coloured paper at the end of this volume/..... Any statements contained therein in respect of preliminary and general requirements shall be subject to sections on General Conditions and General Requirements of this specification.

- Scope of steelwork

- Hot rolled sections: As follows:

a) I-sections, tees and channels to BS 4: Part 1

b) Angles to BS 4848: Part 4

c) Hollow sections to BS 4848: Part 2.

- Cold formed structural sections: As follows:

a) Formed from uncoated strip to BS 1449:Part 1.

b) Formed from zinc coated steel strip to BS 2989

c) Hollow sections: Longitudinally welded sections to BS 6363.

- Sheet and strip: Steel not exceeding 3mm thick to BS 1449 Part 1.

- Ordinary bolts and nuts: BS 4190; nuts to be at least the strength grade of the threaded fastening with which they are used.

- Washers: Black washers to BS 4320.

- Holding-down bolts: Indented bolts and nuts to BS 1494: Part 2, Grade 4.6.

- Holding-down bolts: Black bolts as specified fitted with large heavy gauge plate washer spot welded on and positioned for grouting into undercut mortices in foundations.

- STORING AND HANDLING

- Generally: Comply with the basic requirements of General Requirements. Handling shall be by methods that do not distort the steel and minimise surface damage.

- Workmanship

- Fabrication and erection

- Generally: Fabricate at properly equipped workshops with welded joints in accordance with BS 5950: Part 2: Section 3 with tolerances as specified in Section 7. Carry out any necessary straightening or forming in a manner that will not weaken the material.

- Identification: Mark all steel members to facilitate fabrication and erection; marking for erection purposes shall be durable and easily distinguished. Marking should not damage the steel.

- Cutting: Cut by sawing, shearing, cropping or by machine flame. Hand flame cutting may only be used where it is impracticable to cut by other methods. Dress or otherwise ensure cut edges are true to profile with burrs removed.

- Holing: Requirements for holing are:

a) Drill or punch round holes.

b) Twice drill and cut to form slots; alternatively punch in one operation.

c) Align matching holes so that fasteners can pass through at right angles to faces in contact.

d) Holes for bolts may be punched providing the holes are at least 2 mm less in diameter than the required size and reamed to correct diameter.

e) Remove burrs from holes before assembly except that parts clamped and drilled together need not be separated for this purpose.

f) Holes for bolts up to 24 mm shall not exceed the bolt diameter by more than 2 mm; or by 3 mm for larger bolts (except in steel base plates).

- Welding: To BS 5135 undertaken by welders who have recently passed tests to BS 4872:Part 1. Tests to be based on similar conditions to those existing for the steelwork herein specified, using similar materials.

- Bolting: Requirements for bolting are:

a) Where necessary draw the parts for connection together before bolting and provide steel packings to ensure that the total remaining gap between the surfaces being joined does not exceed 2 mm. Insert bolt(s) and tighten.

b) Bolt lengths shall be such that one clear thread shows after the nut is tightened and one clear thread is left between the nut and the unthreaded shank.

c) Provide suitable washers where oversize or slotted holes are specified.

d) Ensure movement connections are free to move within slotted holes.

- Friction grip bolting: Fix HSFG bolts in accordance with 190 above and BS 4604: Part 1; except that the gap specified in 190 (a) shall not exceed 1 mm. Provide tapered washers where surfaces under the bolt head or nut are not parallel.

- Erection: Erect the steelwork as follows:

a) In accordance with the foregoing clauses wherever relevant to site work.

b) Securely fasten steelwork during erection in order that it can withstand all loadings.

c) Provide temporary bracing and restraints until the structure is self-supporting; make such connections in a manner that causes no harm to the permanent structure. Take safety precautions as recommended in BS 5531, Erecting structural frames.

d) Do not make permanent connections or grout the foundation bolts until the members being connected have been aligned, levelled and plumbed.

e) Use drills with care; do not enlarge or damage bolt holes.

f) Seal bolts through hollow sections.

g) Bed bearing surfaces of beams etc on clean prepared bearings; provide levelling wedges and bed on strong dry-mixed cement mortar for the full bearing; do not load until mortar has hardened.

h) Pack column bases to correct level with steel wedges and grout as specified; ensure base plates are drilled to facilitate grouting.

- PROTECTIVE TREATMENTS

- Generally: Subject to specific procedures given at 210 and 215; requirements are as follows:

a) Surfaces which are not to be treated shall be kept clean and free from loose rust, scale or oil.

b) Before assembly provide specified protective treatment to surfaces inaccessible after assembly.

c) Seal hollow members to prevent access of moisture.

d) Provide vent holes in galvanised hollow members; seal after galvanising if these are not large enough to permit internal galvanising.

e) Protect and leave uncoated all joining surfaces of HSFG connections; apply protective treatment after such connections have been approved.

f) Do not apply protective treatment in areas to be welded.

g) Do not apply protective treatment to welds and adjacent metal until cleaned and approved.

GS30	PAINTING - GENERALLY

For Interior Walls & Ceilings - new

- Gypsum plastering

- All surfaces must be clean, suitably dry, free from plaster fines and anything that will interfere with the adhesion of the materials to be applied. Remove any loose material or efflorescence by scraping and dry bristle brushing. Fill any small surface defects with an interior filler or fine surface filler as appropriate in accordance with the manufacturer's instructions. Allow to dry. Rub down with a fine grade abrasive paper. Remove all dust.

- Mist coat

- when finishing with Crown Trade Matt Vinyl Emulsion

- Apply a mist coat of Crown Trade Matt Vinyl Emulsion, thinned in accordance with the manufacturer's instructions, over all areas to be decorated. Allow a minimum drying time of 4 hours in normal drying conditions.

- Surface filling - following priming or mist coat

- Fill any surface imperfections with an interior filler or fine surface filler as appropriate in accordance with the manufacturer's instructions. Allow to dry. Rub down with a fine grade abrasive paper. Remove all dust and touch in the filled areas with the priming or mist coat, thinned as for main areas.

- Prepared sound clean surfaces Crown Trade Matt Vinyl Emulsion decoration - two coats

- Decorate with two coats of Crown Trade Matt Vinyl Emulsion in accordance with the manufacturer's instructions. Allow a minimum drying time of 4 hours between coats in normal drying conditions.

- Important Notes

- Every care is taken to ensure that the information provided is accurate. Akzo Nobel Decorative Coatings Limited is unable to guarantee results as it has no control over the conditions under which the products are applied. For further information, or help, contact the Customer Advice Centre.

- Apply all products in accordance with BS 6150:1991 Code of practice for painting buildings and BS 8000:Part 12:1989 Code of practice for decorative wallcoverings and painting.

- All products should be applied in accordance with the manufacturer's instructions/Technical Data Sheets.

- If there is any doubt whatsoever with any aspect of this Specification it must be brought to the attention of the Specifying Authority before pricing.

For Interior Walls & Ceilings - Previously painted

- Loose and failing material

- All surfaces must be clean, dry and free from anything that will interfere with the adhesion of the materials to be applied. Remove loose and failing material by scraping or brushing with a stiff bristle brush to a sound edge. Feather sound edges with a fine grade abrasive paper. Remove all dust.

- Small repairs to gypsum plastering

- Proprietary plastering system

- Remove any small areas of loose gypsum plastering. Remove all dust. Dampen edges and areas to be made good. Make good with a proprietary plastering system mixed and applied in accordance with the manufacturer's instructions. Allow to dry.

- Nicotine stained surfaces

- Crown Trade Alkali Resisting Primer

- All surfaces must be clean, dry and free from anything that will interfere with the adhesion of the materials to be applied. Wash with hot water and liquid detergent solution to remove any contaminants, frequently changing the water. Rinse thoroughly with clean water to remove all residues. Allow to dry. Apply one coat of Crown Trade Alkali Resisting Primer in accordance with the manufacturer's instructions. Allow a minimum drying time of 1 hour in normal drying conditions.

- Remaining sound paint

- Wash with hot water and liquid detergent solution to remove any contaminants, frequently changing the water. Rinse thoroughly with clean water to remove all residues. Allow to dry.

- Note:    Any glossy surfaces should be wet abraded to provide a key. Rinse thoroughly with clean water to remove all residues. Allow to dry.

- Cracks and surface defects

- Fill any cracks and small surface defects with an interior filler or fine surface filler as appropriate in accordance with the manufacturer's instructions. Allow to dry. Rub down with a fine grade abrasive paper. Remove all dust.

- Patch priming

- Crown Trade Acrylic Primer BS 5082

- Patch prime all bare and filled areas with one coat of Crown Trade Acrylic Primer BS 5082, thinned 10% with clean water. Allow a minimum drying time of 4 hours in normal drying conditions.

- Bring forward

- when finishing with Crown Trade Matt Vinyl Emulsion

- Bring forward patch primed areas with one coat of Crown Trade Matt Vinyl Emulsion. Allow a minimum drying time of 4 hours in normal drying conditions.

- Prepared sound clean surfaces

- Crown Trade Matt Vinyl Emulsion decoration - two coats

- Decorate with two coats of Crown Trade Matt Vinyl Emulsion in accordance with the manufacturer's instructions. Allow a minimum drying time of 4 hours between coats in normal drying conditions.

- Important Notes

- Every care is taken to ensure that the information provided is accurate. Akzo Nobel Decorative Coatings Limited is unable to guarantee results as it has no control over the conditions under which the products are applied. For further information, or help, contact the Customer Advice Centre.

- Apply all products in accordance with BS 6150:1991 Code of practice for painting buildings and BS 8000:Part 12:1989 Code of practice for decorative wallcoverings and painting.

- All products should be applied in accordance with the manufacturer's instructions/Technical Data Sheets.

- If there is any doubt whatsoever with any aspect of this Specification it must be brought to the attention of the Specifying Authority before pricing.

For Interior Timber - New

- Bare smooth planed timber in sound condition

- All surfaces must be sound, clean and free from anything that will interfere with the adhesion of the materials to be applied. Any fixings must be non-ferrous. Prior to painting, the moisture content should not exceed 18%. Rub down with a suitable grade abrasive paper. Remove all dust.

- Sharp edges

- Round all sharp edges to a 1-3mm radius. Remove all dust.

- Knots/resinous areas

- Remove excess resin from knots and other resinous areas by the use of a hot air paint stripper. Wipe immediately with methylated spirit to remove residue. Apply two thin coats of fresh knotting to all exposed knots. Allow to dry.

- Note:    If knots are removed fill with a two-pack proprietary wood filler.

- Fixing holes or surface defects

- Fill with an interior filler or fine surface filler as appropriate in accordance with the manufacturer's instructions. Allow to dry. Rub down with a fine grade abrasive paper. Remove all dust.

- Priming

- Crown Trade Water Based Primer/Undercoat

- Prime all areas to be decorated with one coat of Crown Trade Water Based Primer/Undercoat in accordance with the manufacturer's instructions. Ensure end grain is well coated. Allow a minimum drying time of 4 hours in normal drying conditions.

- Fine filling

- Fill grain and shallow surface defects with an interior fine surface filler in accordance with the manufacturer's instructions. Allow to dry. Rub down with a fine grade abrasive paper. Remove all dust.

- Undercoat

- Crown Trade Water Based Primer/Undercoat

- Apply as an undercoat, over all areas to be decorated with one coat of Crown Trade Water Based Primer/Undercoat in accordance with the manufacturer's instructions. Ensure end grain is well coated. Allow a minimum drying time of 4 hours in normal drying conditions.

- Prepared sound clean surfaces

- Crown Trade Interior Acrylic Gloss decoration

- Decorate with two coats of Crown Trade Interior Acrylic Gloss in accordance with the manufacturer's instructions. Ensure end grain is well coated. Allow a minimum drying time of 6 hours between coats in normal drying conditions.

- Important Notes

- Every care is taken to ensure that the information provided is accurate. Akzo Nobel Decorative Coatings Limited is unable to guarantee results as it has no control over the conditions under which the products are applied. For further information, or help, contact the Customer Advice Centre.

- Apply all products in accordance with BS 6150:1991 Code of practice for painting buildings and BS 8000:Part 12:1989 Code of practice for decorative wallcoverings and painting.

- All products should be applied in accordance with the manufacturer's instructions/Technical Data Sheets.

- If there is any doubt whatsoever with any aspect of this Specification it must be brought to the attention of the Specifying Authority before pricing.

For Interior Timber- Previously painted

- Unsound loose and failing paint

- All surfaces must be clean and free from anything that will interfere with the adhesion of the materials to be applied. Remove all loose and failing paint by scraping and/or the use of a hot air paint stripper, or a proprietary paint remover used in accordance with the manufacturer's instructions. Prior to painting, the moisture content should not exceed 18%. Rub down and feather edges of sound paint with abrasive paper. Remove all dust.

- Fixing holes or surface defects

- Fill with an interior filler or fine surface filler as appropriate in accordance with the manufacturer's instructions. Rub down with a fine grade abrasive paper. Do not break the surrounding surface paint. Remove all dust.

- Remaining sound paint

- Wash with hot water and liquid detergent solution to remove any contaminants, frequently changing the water. Wet abrade sound paint to provide a key. Rinse thoroughly with clean water to remove all residues. Allow to dry.

- Fine grain filling

- Fill grain and shallow surface defects with an Interior fine surface filler in accordance with the manufacturer's instructions. Allow to dry. Rub down with a fine grade abrasive paper. Remove all dust.

- Undercoat

- Crown Trade Water Based Primer/Undercoat

- Apply as an undercoat, over all areas to be decorated, one coat of Crown Trade Water Based Primer/Undercoat in accordance with the manufacturer's instructions. Ensure end grain is well coated. Allow a minimum drying time of 4 hours in normal drying conditions.

- Note:    If a significant colour change is required a second coat of Crown Trade Water Based Primer/Undercoat may be necessary.

- Prepared sound clean surfaces

- Crown Trade Interior Acrylic Gloss decoration

- Decorate with two coats of Crown Trade Interior Acrylic Gloss in accordance with the manufacturer's instructions. Ensure end grain is well coated. Allow a minimum drying time of 6 hours between coats in normal drying conditions.

- Important Notes

- Every care is taken to ensure that the information provided is accurate. Akzo Nobel Decorative Coatings Limited is unable to guarantee results as it has no control over the conditions under which the products are applied. For further information, or help, contact the Customer Advice Centre.

- Apply all products in accordance with BS 6150:1991 Code of practice for painting buildings and BS 8000:Part 12:1989 Code of practice for decorative wallcoverings and painting.

- All products should be applied in accordance with the manufacturer's instructions/Technical Data Sheets.

- If there is any doubt whatsoever with any aspect of this Specification it must be brought to the attention of the Specifying Authority before pricing.