YOUR NAME, ADDRESS & TELEPHONE NUMBERS GOES HERE

(also referred to as The Specifier’ within this document)

OUR REF:				YOUR REF:	-		DATE:       2010

| Descriptions of each specification | Used for |
| --- | --- |
| Part 1 Specification Manual - Preliminaries & tender return form | Client, Tender, Main Contractor & final contract documents. |
| Part 2 Specification Manual - Main Specification & Notes | Client, Tender, Main Contractor, Building Regulations application, Structural Engineer & final contract documents. |
| Part 3 Specification Manual - General Specification | Client, Tender, Main Contractor & final contract documents. Used selectively & only on larger schemes |

SPECIFICATION MANUAL - PART 2

MAIN SPECIFICATION & NOTES

FOR:-

A

AT:-

FOR:-

MR. & MRS.

This Specification Manual must be read in conjunction with Drg. No's **** together with the separate the Structural Engineers details & any other relevant external suppliers or consultants documents where applicable.

HOW TO USE THIS SPECIFICATION MANUAL

This specification manual is split into 2 main parts being:-

PART 1.	The ‘Preliminaries (1)’ section - This section describes the general requirements of the contract & 	the obligations of both parties.    This is a very important section that has a monetary cost to the 	tendering contractor that is deemed to be included within the fixed price quotation for the works.

The ‘tender return form’ at the rear - This section summaries the documents & is to be 	completed by a tendering contractor.    It is intended to be removed from the specification manual (or 	copied) & retuned to the client.

PART 2.	The ‘Specification (2)’ section - This section describes each element of the works which must be (this part)	read in conjunction with all of the drawings.    This section is usually very site specific to the project.    This 	is the main document that describes the work elements & is to support the Building Regulations 	application as well.

The ’Notes’ section  at the rear has general notes that may be applicable to several different 	elemental descriptions throughout the specification manual.    Some Notes are just contractor conditions 	as well.

PART 3.	The ‘Specification (3)’ section - This section ‘generally’ describes all inclusive descriptions 	where only a small section may be relevant for this particular project which become apparent 	after reading the section.    This is to be used as a ‘general reference’ for good practice elements of 	construction for use by The Main Contractor for back up & for areas of work that may need completing 	that are not fully scheduled in detail in the part 2 specification section.

- All reference to ‘ The Specifier’ within this Specification Manual relates to the person or company stated in the top title block of the front page.

- All reference to the employer, client, owner, relates to Mr. & Mrs. ********

- All reference to the contractor, main contractor, builder relates to the person or company quoting to complete the works.

CONTENTS

SPECIFICATION (PART 2)

S	GENERAL DRAWING NOTES

- Do not scale the drawings unless critical dimensions are missing & then you should ask first.

- Figured dimensions only are to be taken from the drawings.    All dimensions are to be checked on site prior to any work.    If in doubt ask.

- The drawings are metric drawings & all dimensions are in millimetres (unless otherwise stated).

- Report any discrepancies to The Specifier immediately.

- All drawings are to be read in conjunction with other related information &/or relevant Consultants drawings where applicable.

- This Specification Manual & the design drawings are copyright & must not be copied without the written consent of The Specifier.

- For approximate estimating only, the Main Contractor may scale from the drawings.

S	WEATHER PROTECTION

- SCAFFOLD HAT:- The works involve extensive opening up of the existing property exposing it to potential rain & wind damage.    It is the specifiers recommendation that the works & the existing property are protected with a ‘scaffold hat’ to ensure full weather protection during the contract.

- PC Sum: The Contractor is to allow a PC sum of £7,000.00 for the supply, erection & removal of this protective scaffolding during the works.

or:-

- LOCALISED COVERING UP:- The works involve limited opening up of the existing property exposing it to potential rain & wind damage.    It is the specifiers requirement that the works & the existing property are protected throughout the contract with temporary & adequate protection to ensure full weather protection during the contract.    It will be the main contractors responsibility to either replace, repair or make good all damaged works to existing or new structures due to inclement weather damage.

S	TREE PROTECTION

- Requirements: The site has some important trees that need to be protected during the works.    It is a requirement that the main contractor complies with the necessary protection works for the important trees identified by the client.    These works will involve erecting temporary fences around the perimeter of the identified trees creating a ‘no go zone’ for plant & materials.    Remove protective fencing after completion of project.

- Tree Protection Order Trees: Should the trees be part of a TPO then the level of protection is higher & the builder is required to comply with the tree protection methods of the relevant Council guidance & / or terms of the Planning Conditions / tree reports.

S	DIMENSION NOTE

- All    measured surveys: have been taken using predominantly internal face to face dimensions which will incur minor errors for varying wall thicknesses.    Any overall dimensions stated on the drawings are to be taken as approximate only, as the wall thicknesses have been estimated using approx. dimensions.    Therefore, new wall locations that are shown to line up with existing are to be taken in preference to a dimension if there is a small discrepancy.    However, all dimensions are to be checked for accuracy against indicated new wall locations, openings etc. & any major dimensional discrepancies of 50mm or more are to be reported to The Specifier for assessment prior to implementing the works.

- All dimensions & levels: to be checked on site by contractor prior to construction.

S	BUILDING GUARANTEE PERIOD (FOR NEW DWELLINGS)

- Requirement: The new property is to be covered with a 10 year minimum buildings guarantee by one of the following methods:-

1.    Using an NHBC registered builder / main contractor:-    It is a requirement that the NHBC registered builder registers this scheme for inclusion within the NHBC warranty & pays all fees required for this purpose.

2.    Using a non NHBC registered builder / main contractor:-    The Local Authority Building Control insurance backed scheme currently called LABC New Home Warranty has been arranged in conjunction with LABC, the representative body for local authority building control, and covers Speculatively Built Residential Housing, Social Housing and Self Build Housing.    LABC New Home Warranty is administered by MD Insurance Services Limited, who have nearly 10 years of experience in managing and running specialist Latent Defects facilities. All fees associated with registering this project under the LABC scheme will be paid for directly by the client.

S	BUILDING DEFECTS LIABILITY - EXTENSION WORK

- Requirement: The new extension, associated works & installations are to be covered with a 1 year building defects liability period from the main contractor.    The 12 month defects liability period will start at the time of practical completion. Any defects occurring during this period are to be put right at the main contractors cost within a reasonable time period.    Any ‘potential’ building defects identified by the client will be reported to the Main Contractor & the Specifier & a site meeting arranged to discuss the issues.    Normal shrinkage or expected minor settlement will be excluded.

S	COMPLIANCE WITH PLANNING    & / OR BUILDING REGULATION CONDITIONS

- It is likely that the approved scheme was subject to certain conditions on the Planning Approval or within the Building Regulations Approval.    The Main Contractor is to obtain a copy of the original Planning & Building Regulations Approval direct off the site owner & check them for    any relevant conditions that may need satisfying either before you start on site or during the works.    Should any Conditions need satisfying or complied with then the Main Contractor is to discuss the issues & options with the client / site owner direct.    Any doubt contact the Specifier direct.

S	OBTAINING A BUILDING REGULATIONS COMPLETION CERTIFICATE

- It is requirement that the works are supervised, inspected & approved by the site visiting Building Inspector from the relevant Local Authority & that the Inspector is called in by the main contractor at the relevant build stages.    The main contractor will be required to obtain a Completion Certificate from the Building Control Dept. as part of the contract for the works which will act as the ‘practical completion date’ for the transfer of any final payments &/or part release of retention monies due to the builder.

S	BUILDERS / SPECIFIERS MATERIALS SELECTION

- This specification contains a mixture of ‘fully specified’ materials & ‘generically described materials‘.    It is appreciated that many types of materials will perform the same function & some can be easier to obtain locally than others.    However, where specific materials have been described, these shall be used at all times.    ‘Equal & approved’ alternative specified materials can be selected provided prior notification has been given to the client and the specifier & approval from them has been obtained.

- ‘Generically described materials’ can be selected by the builder / main contractor provided they meet the required latest British Standards or latest European standards, approved by the Building Control Inspector & are fit for purpose.    Any cost or labour savings made by the contractor on switching to ‘equal & approved’ alternative materials is to be passed on to the client.

S	MATCHING EXISTING BUILDING DETAILS

- Generally: Some of the plans or specification clauses may show or specify details of certain buildings elements that are variance with the existing properties details. (eg - double tile external window cills, eaves corbelling etc.).    This is where the specifier may have missed certain details due to standardisation or generics of drawings or specification methods. However, it is a requirement that the extension works follow through & match in with the existing building details as far as practically possible provided they still maintain good building practice & comply with the Building Regulations.

- Examples: Brick bonding, mortar thickness, colour & pointing, brick detailing at verges, eaves, banding, qoining, headers, plinths, corbelling, cills etc.    Roof overhangs, tilting fillets, hip tiles, ridge tiles, guttering etc.

- Requirement: Should the Main Contractor or any of his trades spot any inconsistencies with the new works not fully matching with the existing, then the Specifier is to be informed immediately & made aware of the situation for further clarification & advice.

S	HEALTH & SAFETY - RISK ASSESSMENTS

- Refer to section P130 of the Part 1 Specification Manual.

Or:-

- CDM Regulations: are relevant for this project & it is a requirement that the client engages a CDM Health & Safety Planning Supervisor to maintain the health & safety file for this project.    This ‘responsible’ person has yet to be engaged.    The Specifier has already advised the site owner of the need for this person.

- The Specifier has already completed a separate health & Safety Risk assessment for this scheme which has been issued to the client & can be seen by the any of the builders tendering for the works.

S	THERMAL PERFORMANCE OF NEW STRUCTURES - GENERALLY

Method used for compliance = Elemental method.

Target max. ‘U’ Values to be as follows:-

Floors = 		0.22w/m2/deg.C

External walls =	0.30w/m2/deg.C

Roof 1 cold deck =	0.16w/m2/deg.C (flat or pitched - cold deck with insulation between joists)

Roof 2 warm deck = 	0.16w/m2/deg.C (flat or pitched - warm deck with insulation over joists or rafters)

Glazing =		1.80w/m2/deg.C (for windows & roof lights - max. area allowed = 25% of the total new 			floor area)

New boiler installation =	High efficiency condensing boilers with a min. SEDBUK efficiency rating of 90% for natural 		gas.

New radiators =	Zone control ability with TRV‘s fitted to each radiator.

Semi-exposed areas =	Required ‘U’ values as per exposed elements.

For this particular project, the insulations used to achieve these max. ‘U’ values will be as follows:-

FLOORS:-

- For ground bearing slabs, & suspended concrete floors use 75mm Celotex floor insulation board (Ref GA3075Z) for an area to exposed perimeter ratio of about 0.50 & below for suspended conc. floors & 0.70 & below for solid ground bearing conc. floors.

- For suspended timber floors use 100mm thk. Celotex floor insulation (Ref GA3100Z) supported on galv. steel saddle clips.    Allow 50mm air gap over insulation to underside of boarding.

- Install 12mm Celotex (GA3012Z) perimeter upstands at exposed wall to floor junctions to reduce cold bridging.

EXTERNAL WALLS:-

- For facing brick walls:- 290mm nom. thickness cavity walls comprising of 105mm facing brick outer skin, 85mm cavity fully filled with 75mm Crown ‘Dritherm 32’ cavity wall insulation bats, 100mm Durox Superbloc inner skin, 19mm 2 coat lightweight plaster internal finish.

or:-

- For externally rendered walls:- 285mm nom. thickness cavity wall with an external skin of 100mm Topblock Toplite standard conc. blocks, 85mm cavity fully filled with 75mm Crown ‘Dritherm 32’ cavity wall insulation bats, 100mm Durox Superbloc inner skin, 19mm 2 coat lightweight plaster internal finish or 12.5mm plasterboard on dabs incorporating 15mm air gap internal finish with 5mm final plaster skim coat.

- For dormer walls & cheeks:- 100mm Crown Dritherm cavity wall insulation bats cut between the studs with a 40mm Celotex (Ref. GA3040Z) internal lining covered with 12.5mm plasterboard & set.

PITCHED ROOFS:-

- Cold Deck - For insulation at joist level:- 200mm Crown Wool insulation quilt laid between the ceiling joists plus 100mm Crown Wool insulation laid over the ceiling joists.

- Cold deck - For insulation at rafter level (sloping soffit):- Two layer system comprising of 40mm Celotex (Ref. GA3040Z) laid under the rafters with 100mm Celotex (Ref. GA3100Z) fixed between the rafters.    Joints to be sealed with Idenden sarking tape.

- Warm deck - For insulation at rafter level:- Two layer system comprising of 60mm Celotex (Ref. GA3060Z) laid over the rafters secured with 25 x 50mm treated s.w counter battens.    100mm Celotex (Ref. GA3100Z) fixed between the rafters & abutting top insulation held in place with fixing pins.    Top side of joints to be sealed with Idenden sarking tape.

FLAT ROOF:-

- Warm deck:- Celotex 116mm o/a thickness composite insulated decking comprising of 110mm Celotex pre-boned to 6mm plywood decking laid over the joists prior to laying of additional 12.5mm OSB decking.

- Cold deck:- Celotex totalling 140mm.    100mm celotex placed between the joists with 40mm Celotex underlining.    Tape all joints with idenden sarking tape.    This roof system requires cross venting & a 50mm min. air gap above the insulation.

SLOPING SOFFIT (scaling ceilings):-

- For new rooms using breathable felts :- 100mm Celotex cut to fit between the rafters with a 40mm Celotex    underlining with the joints sealed with Idenden Sarking tape prior to 12.5mm plasterboard & set.

- For loft conversions utilising existing non breathable felts :-    Add    50 x 50mm battens to the underside of the existing rafters so that 100mm Celotex insulation board can be installed between the rafters & still achieve a 50mm air gap over.    Install a further layer of 40mm Celotex insulation board as an underlining    with the joints sealed with Idenden Sarking tape prior to 12.5mm plasterboard & set.    This roof system needs cross venting.

MEASURES TO REDUCE COLD BRIDGING:-

- Floors:- Perimeter insulation between screed & exposed walls to be 12mm Celotex.

- Lintels:- To arrive pre-insulated from factory.

- Reveals:- Thermabate cavity closers or other approved insulated cavity closer.

- Elemental abutments:- Ensure that all insulation types join each other for a continuous thermal barrier.

GLAZING:-

- For timber or plastic frames: Sealed unit double glazing with 12mm air gap, argon filled, with the inner leaf being Low ’E’ glass.    Frames will be either pressure impregnated timber or PVCu multi-chamber or 20mm thermal break aluminium (powder coated). (refer to relevant section in this spec.)

- For metal frames:    Sealed unit double glazing with 16mm air gap, argon filled, with the inner leaf being Low ’E’ glass.    Frames will be coated aluminium with thermal break seals. (refer to relevant section in this spec.)

ESTIMATED NEW GLAZING AREA FOR THIS PROJECT (elemental compliance):-

- New glazing area = 			****M2

- Existing glazing area covered by extension = 	****M2 (trade off)

- Total new glazing area (net) =		****M2

- New extension internal floor area =	****M2

- The new glazing area is therefore equivalent to ** % of the new extension floor area & complies as this is less than 25%.

NEW HEAT SOURCE (IF REQUIRED):-

- New heat source: to be a high efficiency gas fired balanced flue condensing boiler, sized & selected by the Main Contractors Heating Engineer. Wall mounted boiler preferred but floor standing unit could be utilised if area allows.

- New boiler: must comply with the SEDBUK efficiency rating requirements & be a min. of 90% efficient.    Hot water storage tanks to be Grade 1,2 or 3 cylinders only.

ZONE CONTROL OF HEATING:-

- A programmer, room & cylinder thermostats, motorised valves are to be utilised.    These are all interlocked with the boiler pump, together with thermostatic radiator valves fitted to all new radiators, except in the room where the thermostat is fitted.

S	PULLING DOWN & ALTERATION OF EXISTING STRUCTURES

- The existing ????? & any outbuildings shown to be removed are to be demolished & carted away from site to an approved Council tip or recycling yard including the grubbing out of all foundations & redundant drains.

- The Main Contractor shall include both labour & materials for any shoring, strutting, scaffolding or temporary works in connection therewith, for making good all work disturbed in all trades.

- Pulling down, taking out & cutting away shall be carefully performed & every precaution shall be taken to ensure the safety of the works.

S	DEMOLITIONS - GENERALLY

- Code of practice: Comply with BS 6187 Demolition subject to any qualifications given below. References given in the clauses refer to BS 6187.

- Choice of demolition contractor: Ensure that major demolition is undertaken by skilled operatives who are experienced in working in accordance with BS 6187 and the publications of the National Federation of Demolition Contractors. These documents form the basis of this specification. BS 6187 shall be read in conjunction with this document and its principal clauses are referred to below.

- Scope of demolition: The extent of the work is indicated on plans.    Unless otherwise stated demolish down to the lower level of the ground floor slab & grub out any existing structures such as footings, foundations etc. to a depth of at least 400mm below ground level.

- Undertake survey: Undertake a survey in order to ascertain sufficient information about the structures and their surroundings.    Before commencing work complete the survey to obtain the information referred to in BS 6187 clause 4.1.

- Investigate risks: Investigate the features of the structure (4.1.2), ascertain if shock or vibration could damage surrounding property or equipment therein or buried services (4.1.1) and check the existence of toxic or flammable substances or asbestos (4.1.4).

- Disconnect services: Disconnect, divert, seal off or remove existing services as required (4.1.3).

NEW MATERIALS

- Generally: Any new materials required shall be as specified hereafter in the appropriate work sections.

OLD MATERIALS

- Removal and credit: Remove from the site materials not required by the Employer/client & cart away to an approved council tip or recycling yard. For any such materials of value where the builder can obtain recycling monies (eg lead), allow credit to client.    The Employer reserves the right to retain any materials.

- Salvaging: Carefully salvage materials required by the Employer/client. Sort, clean off any unwanted attachments and neatly stack or store in safe locations on site.

- Bricks: Clean and stack on site any bricks suitable for re-use for general brickwork at the builders discretion.

- Hardcore: Any material complying with the specification for hardcore may be used for this purpose in external environments only. (under patios, paths, drives etc.)

- Asbestos: Should the survey or during demolitions expose asbestos materials of any sort, the demolitions are to stop immediately & the Specifier is to be informed.    The Main Contractor will not be responsible for removing asbestos.    Any specialists required for removing asbestos will be at the clients cost.

METHODS OF DEMOLITION

- Select suitable methods: Employ methods that cause no shocks or vibration likely to damage surrounding property or equipment housed therein or buried services. The use of explosives is forbidden as is the collapsing of large masonry wall areas likely to cause damaging shock waves.

NUISANCE

- Prevent nuisance: Prevent nuisance with particular reference to dust, smoke and noise (5.3.5).    Damp down with water & hose pipe areas of high dust formations.

- Rot infected timber: Ensure that infected timber does not contaminate any materials intended for re-use. Infected timber may be burnt on site subject to the following provisions:

a) not when the wind is strong or unfavourable.

b) not on Saturdays to Mondays inclusive.

c) only by agreement with local authority and adjoining occupiers.

d) restricted to small controlled fires of wood or wood-based materials not giving dense or toxic smoke.

e) well away from any trees or vegetation to be retained.

f) extinguished by 1500 hours.

ADJACENT STRUCTURES NOT DEMOLISHED

- Disconnecting: Disconnect by means causing least damage. Leave no unnecessary projections. Obtain instructions from specifier if the Contractor considers it necessary to leave any temporary supports in position.

- Making good: Make good only to the extent required to ensure safety of persons, weather protection, stability and security. Do not leave any dangerous or unnecessary projections. Remove any infected embedded timber if possible, otherwise cut off flush and soak with creosote. Remove any debris constituting a health or fire risk.

PROTECTION

- Generally: Provide general protection as clause 5.4 of BS 6187 and the section on General Requirements herein.

- Screens: Provide temporary screens or corridors wherever needed for safety, noise or dust control, weather protection, security or other purposes to facilitate the Works.    Leave site secure.

- Exposed structures: Weatherproof exposed walls of existing buildings as required.

- Trees: Protect as stated in and clause 4.2 of BS 6187.

S	ALTERATIONS TO EXISTING STRUCTURES - GENERALLY

- INTRODUCTION: Most new building works will involve an element of alterations to the existing structures.    Identifying the general locations of such areas are shown on the plans or described within the specification manual but the extent of the works may not be fully detailed.    This section enhances some of the general terms used or works shown to indicate that an alterations are deemed to be ‘complete’ & made good to decorating standard.    Non of the alteration work is measured (unless the main contractor quoting for the works is able to provide this level of detail at time of tender) so the builder or main contractor quoting for the works will have to ‘take a view’ on what is required regarding labour, materials & profit margin to complete the works fully.

- NEW MATERIALS

- Generally: New materials shall be as specified hereafter in the appropriate work sections.

- Old materials

- Removal and credit: Remove from the site materials arising from the alteration works which are not required by the Employer. For any such materials of value, allow credit to the client.    The Employer/client reserves the right to retain any old materials on payment to the Contractor of the credit amount allowed.

- Unwanted materials: Remove debris and materials not required for retention or re-use on site. Cart away to an approved council tip or recycling yard. Do not permit such materials to accumulate or become a health or fire risk.

- Salvaging: Materials to be salvaged for the benefit of the Employer fall into two classifications as follows:

- a) Materials described as    ‘set aside for re-use‘.

- b) Materials described as    ‘hand to Employer‘.

- Bricks: Clean and stack on site any bricks suitable for re-use for general brickwork at the discretion of the main contractor.

- Hardcore: Any material complying with the specification for hardcore may be used for this purpose in external environments only. (under patios, paths, drives etc.)

- PRELIMINARY WORK

- Pre-tender - Site visit: In addition to the general implications of the site visit referred to General Conditions, visit all locations given and ensure that the full extent of the work is understood and priced inclusively.    Demands for ‘extra costs’ from the main contractor will not be permitted for any alteration work to the existing structures.

- Post tender (before commencing work)

- Removing debris: Remove all debris, rubbish, loose fittings and the like from the buildings and site and leave tidy. Disinfect if necessary.

- Services: Make safe or divert all services. Liase with the relevant service authorities as necessary. Test the drains if necessary and report on the findings.

- Preliminary investigations: Undertake investigatory cutting away and opening up in order to become acquainted with the condition of the building to be altered. All walls indicated to be removed on plans that are deemed to be non-load bearing MUST be inspected & doubled checked first prior to removal. Any doubts, contact the Specifier immediately. If any investigations of this nature are undertaken by the specifier, provide attendance including labour and suitable hand tools.

- New work: Undertake new work associated with the alterations to the existing building as specified in the appropriate work sections or shown on plans. Remove, adapt or re-fix old work and materials as necessary for a durable & robust installation.

- Redecorations: See Painting and Decorating.

- SAFETY

- Safety of existing / new structures: Provide props, struts, scaffolding and shores, etc as necessary to ensure the stability of the existing structures. Make preliminary investigations and calculations where necessary and accept responsibility for the overall safety of the works and existing structures unless expressly made the responsibility of others in the Contract Documents.

- NUISANCE

- Prevent nuisance: Prevent nuisance with particular reference to dust and noise. Establish and maintain good relations with adjoining occupiers.

- SPOT WORKS

- Description and pricing

- Definition: Spot work is defined as alteration or repair work which is described in self-contained items elsewhere within this specification manual or other works identified separately on site that contain work in any or all trades. The following is deemed to be included with such items:

- a) All work that can be seen or anticipated on the spot' (i.e. the stated location).

- b) Ancillary work and labours normally associated with the type of work involved.

- c) The provision of fixing materials of every description.

- d) Properly bonding, jointing and making good to surrounding work.

- Locations: The location of spot work is stated in most instances. Such locations relate to the building as existing.

- Repetitive work: Repetitive spot work is grouped (i.e. one description covers a number of items of like work) and such work shall be priced to allow for the average circumstances at the various locations.

- Sizes: Any dimensions given with a spot item are approximate. Precise dimensions or other measurements required shall be taken by the Contractor at the Works.

- Unlocated spot works: Work of a type that can be envisaged by reference to description only but is not necessarily located precisely.

- TERMS USED TO DESCRIBE ALTERATIONS OR WORKS SHOWN TO BE COMPLETED ON THE PLANS:-

- List of terms used: The following terms are used:

- Remove: This term is used for work and materials to be taken down or broken up or otherwise unfixed by whatever means the Contractor considers appropriate and removed from site.

- Carefully remove: This term is used for components which are to be retained as stated. Handle such components with care and protect from damage. In pricing such work it will be assumed that the Contractor considers that he is able to unfix and handle without damage.    Make good all damaged surfaces to decorating standard.

- Cut (or adapt) opening: Form the new jambs, level-off at sill and fix only the new lintel.    Make good all damaged surfaces to decorating standard.

- Block opening: Cut away finishings to expose sound structures, cut and bond or otherwise make secure connection and wedge up at top.    Supply & install new skirting as required to match the existing. Make good all damaged surfaces to decorating standard.

- Reduce opening: As last description including forming the new jambs.    Make good all damaged surfaces to decorating standard.

- Extend finishings: Extend the plaster, tiling, skirting or whatever the existing wall, floor (and ceiling if relevant) finishings are over the areas affected or required to be made good and apply one coat of primer to plaster or woodwork or other surfaces scheduled for decoration; decoration is described elsewhere.    When viewing this work note the type and quality of the old work and allow for cutting back and neatly joining and making good all work disturbed.

- Fix only new lintel or beam: Cut away for and build in the new lintel or beam (which, unless otherwise stated, will be a precast concrete or proprietary steel lintel described elsewhere) including cutting and pinning ends and wedging up. Cut away to suit the depth of the new lintel and for 150 mm (min.) bearing at each end.    Supply & install concrete end bearings to Structural Engineers details (if required).

- Remove wall or partition: This also means the removal of the doors, frames, rails and skirtings, etc, which are associated with the wall concerned making good floors, walls and ceilings as required to decorating standard.

- Remove sanitary fitting: Remove the type of sanitary fitting stated or shown and associated pipework including either cutting back and sealing off disused pipes in an approved manner or providing temporary cappings, suitably identified, where pipes are scheduled for re-connection.

- Making good: Making good ALL structures after cutting away and similar work will be deemed to be included with all spot works. Making good finishings is stated where required.

- Protection

- Generally: Provide general protection as specified in the General Requirements section.

- Existing floors: Lay hardboard or other suitable material on existing floors and stairs, etc that are not scheduled for upgrading.

- Temporary openings: Maintain weather protection and security.

S	EXCAVATIONS

- Definition: The term excavation shall include for getting out.    The works shall include for keeping the excavation free from water by pumping, baling or otherwise & grubbing up any roots or normal obstructions.

- Responsibility: The main contractor will be held responsible for any slips or falls of earth etc. & must clear away any such falls & make good all damage to the works arsing therefrom without charge to the client.

- Trenches & Pier holes: Excavate trenches for foundation sizes as indicated on plans & described elsewhere.    Support the trench / hole sides as required, level & ram the base of trench. On completion of concrete / footings work back fill & ram. Remove surplus material from site.

- Surface soil: Allow for stripping any turf, rolling back & stacking. Stripping surface soil & vegetable matter.

- Reducing levels: Complete any further surrounding surface excavations as required to suit floor levels & dpc‘s.

S	NEW FOUNDATIONS (STRIP)

- Concrete strip foundations (grade C20 concrete using sulphate free cement) min. 1M deep or to suit exposed sub-soil conditions, any local trees & to the approval of the site visiting Building Inspector.

- Provide 3 no. 12mm dia. mild steel reinforcing rods in foundation with 100mm bottom cover.

- For cavity external walls:- 	600mm wide x 300mm thick.

- For single skin walls:- 	450mm wide x 300mm thick.

- For internal load bearing walls:- 	450mm wide x 300mm thick.

- Bridging of new or existing drains:-  Drains that pass under building are to be protected with a 450 x 100mm deep conc. slab over pea shingle.    Where drains pass through brickwork footings or foundations, a pre-cast conc. lintel is to be provided over to form a bridge.    Pack around drain with granular fill as drain passes through footing wall or conc. foundation to allow for building settlement.

- All new or existing drains that are to be built over must be pressure tested by air or water prior to covering with the floor construction or earth & inspected by the Building Inspector.    If tests fail permitted limits then the drains are to be repaired or renewed & tested again until a pass is achieved.

- New or existing services:- Build in upvc ductwork through foundations or footings as required to suit locations of new or existing services that need to enter the building at the required locations.    Note that not all of these service duct items will be indicated on the plans.

- Extra depth foundations:- Should the exposed sub-soil conditions dictate deeper foundations beyond the normal 1M deep as determined by the site visiting Building Inspector, then this will be at extra cost to the client.    HOWEVER, it is a requirement that the extra depth cost is ‘at builder cost’ plus 10% profit & attendance rates.    All extra costs need to be substantiated with invoices & labour rates.

S	NEW FOUNDATIONS (TRENCH FILL)

- Trench fill concrete foundations (Grade C20 concrete using sulphate free cement) min. 1M deep or deeper to suit exposed sub-soil conditions, local trees & to the approval of the site visiting Building Inspector.    Finish top of foundation 200mm below finished external ground level to allow 200mm of soil to pass over.

- For cavity external walls:- 	600mm wide x 800mm thick.

- For single skin walls:- 	450mm wide x 800mm thick.

- For internal load bearing walls:- 	450mm wide x 800mm thick.

- Bridging of new or existing drains:-  Drains that pass under building are to be protected with a 450 x 100mm deep conc. slab over pea shingle.    Where drains pass through brickwork footings or foundations, a pre-cast conc. lintel is to be provided over to form a bridge.    Pack around drain with granular fill as drain passes through footing wall or conc. foundation to allow for building settlement.

- All new or existing drains that are to be built over must be pressure tested by air or water prior to covering with the floor construction or earth & inspected by the Building Inspector.    If tests fail permitted limits then the drains are to be repaired or renewed & tested again until a pass is achieved.

- New or existing services:- Build in upvc ductwork through foundations as required to suit locations of new or existing services that need to enter the building at the required locations.    Note that not all of these service duct items will be indicated on the plans.

- Extra depth foundations:- Should the exposed sub-soil conditions dictate deeper foundations beyond the normal 1M deep as determined by the site visiting Building Inspector, then this will be at extra cost to the client.    HOWEVER, it is a requirement that the extra depth cost is ‘at builders cost’ plus 10% profit & attendance rates.    All extra costs need to be substantiated with invoices & labour rates.

S	STEPS IN FOUNDATIONS

- Steps will be permitted to suit levels.    Min. step to be 300mm.    max. step to be 600mm & the overlap to be twice the step size.

S	NEW FOUNDATIONS NEAR TREES

- The foundation depth: is subject to site verification by Building Inspector & must be 600mm below any roots exposed in trench.    Should clay sub-soils be exposed & heave protection required, contact The Specifier immediately for advice on a separate consultant Engineer to design the foundations.

- Extra depth or heave precautions to foundations: Should the exposed sub-soil conditions & local vegetation dictate deeper foundations beyond the normal 1M depth & heave precautions within the foundation design as determined by the site visiting Building Inspector, then this will be at extra cost to the client.    HOWEVER, it is a requirement that the extra depth & heave precautions cost is ‘at builders cost price’ plus 10% profit & attendance rates.    All extra costs need to be substantiated with invoices & labour rates.

S	EXISTING FOUNDATIONS

- The existing foundations that are to take increased loads are to be checked on site prior to construction by means of trial holes for checking by the LA. Building Inspector for the suitability of the existing foundations to take increased loads.    Should any existing foundations be deemed deficient then The Specifier is to be contacted immediately for advice.

- Should a separate & isolated trial hole be required by the site visiting Building Inspector, then is deemed to be included within the main contractors price including backfilling & making good surrounding surfaces.

S	LOCALISED UNDERPINNING

- Generally: Localised underpinning may be required by the site visiting building inspector due to a mis-match of foundation depth (new to old).    This is deemed to be included within the main contractors work.    Should additional underpinning to existing walls / foundations be required due to exposed deficiencies of the existing structure when taking increased loads for example, then this will be at extra cost to the client.    HOWEVER, it is a requirement that the extra underpinning works cost is ‘at builders cost price’ plus 10% profit & attendance rates.    All extra costs need to be substantiated with invoices & labour rates.

S	EXCAVATION - EXPOSING EXISTING SERVICES

- Generally: Should the new excavations expose existing services supplying the main dwelling, then these are to be bridged & accommodated as required (if allowable by the relevant statutory authority) & these works are deemed to be included within the main contractors cost.    Should the existing services need re-routing or renewing whether by the Statutory Authority or by the Main Contractor, then this will be at extra cost to the client.    HOWEVER, it is a requirement that the extra costs associated with such works is ‘at builders cost price’ plus 10% profit & attendance rates.    All extra costs need to be substantiated with invoices & labour rates.

S	BASEMENT FLOOR / FOUNDATION & BASEMENT WALLS

MAIN CONTRACTOR NOTE:-

- By a specialist company yet to be selected.    This will be a nominated sub-contractor selected by the client but under the control of the main contractor.    Main contractor to provide attendance only to this nominated specialist basement contractor.

- The basement structure completed by this specialist will terminate just below ground level ready for the main contractor to install the main build works for the property.

- Main contractor to ensure that ‘pockets’ are built in as required for the new steel beams together with ducting for all services at the relevant entry / exit points.

CURRENT SUGGESTED CONSTRUCTION METHOD:-

- For Building Regulation purposes the Clients Structural Engineer has suggested a reinforced hollow concrete block design on a poured reinforced concrete slab as one of three options.    This may change.    If so details will be supplied to the Building Control Dept. prior to construction.

TANKING METHOD:-

Tanking & waterproofing products from:-

Grace Construction Products Limited
Ajax Avenue
Slough, Berkshire SL1 4BH
United Kingdom

Tel: +44 (0)1753 692929
Fax: +44 (0)1753 691623
email: uksales@grace.com

FLOOR:-

- Use Preprufe - A pre-applied waterproof membrane that develops an adhesive bond to poured concrete to prevent water migration.    Lay onto smoothed out concrete blinding in accordance with the manufactures instructions.    Extend prepruf beyond edge of basement slab by 300mm for sealing to wall tanking membrane.

VERTICAL WALLS:-

- Bituthene 8000 applied to smooth external face of RC hollow concrete blocks after preparing with B2 primer.    Lap & seal Bituthene to Preprufe floor membrane.    Installation to be in accordance with the Manufacturers details & instructions.

TANKING PROTECTION:-

- Prior to backfilling install Bituthene protection board covering the entire surface of the Bituthene 8000 sheet.

WALL TANKING TERMINATION DETAIL:-

- Just before end of finished ground level stop the Bituthene 800 tanking & bring membrane into inside of inner skin of cavity wall using Bituthene LM product (liquid membrane) & terminate against cavity tray.    Refer to standard detail ref. BWK013-06/07 by Grace Construction products for detail section of how this arrangement works.

OTHER SPECIFICATION DETAILS.

- A separate set of tanking specifications & details has been arranged at the back of this specification manual supplied by Grace Construction products that has enhanced details of the products & how it is to be applied.    Reference must be made to this section by the contractor installing the basement.

- It is critical that the workmanship of installing these tanking products are to a high & precise standard to the exact standards stated by Grace Construction Products supplied to achieve the continuous waterproof tanking barrier required.

ASSUMPTIONS.

- The soil report suggests that ground water should not be a problem during the excavation & installation of the basement & therefore provision for dewatering pumps has not been requested or catered for at this time.

S	NEW EXTERNAL WALLS (BRICK / BLOCK)

290mm nom. thickness cavity wall comprising of:-

- Facing bricks external skin: 105mm facing bwk. to match existing (to be prior approved by client & Planning Dept.). Use same size bricks as existing.    Ensure bonding pattern matches the existing facing brick walls.

- Cavity: 85mm cavity filled with 75mm Crown 'Dritherm 32' cavity wall insulation board.

- Wall ties: stainless steel wall ties (5 No. per M2 & at staggered ctrs.- double up wall ties around all opening reveals).

- Inner skin: 100mm Durox Superbloc lightweight concrete block inner skin.

- Footings below ground: dense conc. foundation blocks or common bwk. below g.l. with fine conc. cavity fill terminating 150mm below g.l.

- DPC: Hiload polythene dpc. 150mm above g.l. for both skins; Lap & seal inner skin dpc to floor dpm.

- Mortar: Sand cement mortar with propriatory plasticiser added to the mix. Cured mortar bed to have final colour & pointing to match that of the existing wall.    Refer to separate    section for mortar mixes.

- Wall plates: 100 x 50mm treated s.w. wall plates tied to inner skin bed on mortar with 38 x 5mm    galv. steel straps spanning 600mm of wall & at 2M ctrs.

- Lintels: Galv. steel lintels by Catnic or I.G. Eng. Ltd. (lintel references as stated on plans) with cavity tray dpc's over & external weep holes - NOTE:- to prevent cold bridging & achieve a 'U' value of 1.2 W/M2/deg. C, all steel lintels must arrive on site as fully insulated at the factory.

- Cavity trays: By Cavity Trays Ltd. Selecting each cavity tray appropriate for its use & location.    Equal & approved alternatives may be used.    The use of roll felt will not be permitted.

- Vertical DPC’s: All openings in external cavity wall to have vertical dpc's; cavity wall will be closed around all openings & at plate level with special PVCu insulated cavity closures by Thermabate or similar & approved to prevent cold bridging & achieve a 'U' value of 1.2 W/M2/deg.C.

- Internal finish: Finish internally with 19mm 2 coat plaster.

- DETAILS:-

- Ensure cavity wall insulation board terminates 150mm below finished floor level.

- Ensure that no mortar snots or other debris fall between cavity board to bridge skins of wall.

- Ensure cavity wall insulation abuts roof insulation to reduce cold bridges.

- NOTE 1:- To reduce cold bridging ensure that the wall insulation abuts & joins to the roofing insulation to form a continuous thermal barrier.

- NOTE 2:- Build in stepped cavity trays at roof abutment locations & as work proceeds at the intersection with lead flashings to maintain a continuous water proof barrier of the structure.

- NOTE 3:- Main Contractor to establish cost of best matching brick available prior to tender & include accordingly rather than offering their own PC sum.

S	NEW EXTERNAL WALLS (BLOCK/BLOCK)

285mm cavity wall comprising of:-

- Outer skin: 100mm Topblock Toplite standard.

- Inner skin: 100mm Durox Superbloc.

- Cavity: 85mm cavity filled with 75mm Crown 'Dritherm 32' cavity wall insulation board.

- Wall ties: Stainless steel wall ties (5 No. per M2 & staggered ctrs. - double up wall ties around all opening reveals).

- Footings below ground level: Dense conc. foundation blocks or common bwk. below g.l. with fine conc. cavity fill terminating 150mm below g.l.    3 courses of engineering bricks below dpc. on outer skin.

- DPC: Hiload polythene dpc. 150mm above g.l. for both skins; lap & seal floor dpc to floor dpm.

- Mortar: Sand cement mortar with propriatory plasticiser added to the mix. Refer to separate    section for mortar mixes.

- Wall plates: 100 x 50mm treated s.w. wall plates tied to inner skin bed on mortar with 38 x 5mm    galv. steel straps spanning 600mm of wall & at 2M ctrs.

- Lintels: Galv. steel lintels by Catnic or I.G. Eng. Ltd. (lintel references as stated on plans) with cavity tray dpc's over &    external weep holes.

- Cavity trays: By Cavity Trays Ltd. Selecting each cavity tray appropriate for its use & location.    Equal & approved alternatives may be used.    The use of roll felt will not be permitted.

- Vertical DPC’s: All openings in external cavity wall to have vertical dpc's; cavity wall will be closed around all openings & at plate level with special PVCu insulated cavity closures by Thermabate or similar & approved to prevent cold bridging & achieve a 'U' value of 1.2 W/M2/deg.C.

- External finish: 25mm 2 or 3 coat CM. render to match existing (pebble-dash or smooth finish) with a waterproofing agent added to all of the mixes. Terminate external render just above dpc. & all openings with a stainless steel bell cove former.

- Internal finish: 19mm to coat plaster or 12.5mm Plasterboard on dabs having a 15mm air gap.    Plaster skim & set.

DETAILS:

- Ensure cavity wall insulation board terminates 150mm below finished floor level.

- Ensure that no mortar snots or other debris fall between cavity board to bridge skins of wall.

- Ensure cavity wall insulation abuts roof insulation to reduce cold bridges.

- NOTE 1:- To reduce cold bridging ensure that the wall insulation abuts & joins to the roofing insulation to form a continuous thermal barrier.

- NOTE 2:- To prevent cold bridging & achieve a 'U' value of 1.2 W/M2/deg. C, all steel lintels must arrive on site as fully insulated at the factory.

- NOTE 3:- Build in stepped cavity trays at roof abutment locations & as work proceeds at the intersection with lead flashings to maintain a continuous water proof barrier of the structure.

S	NEW EXTERNAL WALLS (BRICK/FLINT/BLOCK)

400mm o/a cavity wall comprising of:-

- Outer skin: 100mm Durox Superbloc blockwork backing wall with 105mm facing bwk. (215mm total thickness external skin)    quoining at corners & around openings to form panels for knapped flints to match the existing house tied to the backing block wall with stainless steel wall ties (5 No. per M2).    Form flintwork panels with facing bwk. quoining etc. as generally shown on elevations.

- Cavity: 85mm with 75mm Crown ‘Dritherm 32’ cavity wall insulation board using suitable stainless steel wall ties (5o. per M2 & doubled up around all opening reveals).

- Inner skin:    100mm Durox Superbloc blocks.

- Wall ties: Stainless steel wall ties (5 No. per M2 & staggered ctrs. - double up wall ties around all opening reveals).

- Footings below ground level: Dense conc. foundation blocks or common bwk. below g.l. with fine conc. cavity fill terminating 150mm below g.l.    3 courses of engineering bricks below dpc. on outer skin.

- DPC: Hiload polythene dpc. 150mm above g.l. for both skins; lap & seal floor dpc to floor dpm.

- Mortar: Sand cement mortar with propriatory plasticiser added to the mix. Cured mortar bed to have final colour & pointing to match that of the existing wall.    Refer to separate    section for mortar mixes.

- Wall plates: 100 x 50mm treated s.w. wall plates tied to inner skin bed on mortar with 38 x 5mm    galv. steel straps spanning 600mm of wall & at 2M ctrs.

- Lintels: Galv. steel lintels by Catnic or I.G. Eng. Ltd. (lintel references as stated on plans) with cavity tray dpc's over &    external weep holes.

- Cavity trays: By Cavity Trays Ltd. Selecting each cavity tray appropriate for its use & location.    Equal & approved alternatives may be used.    The use of roll felt will not be permitted.

- Vertical DPC’s: All openings in external cavity wall to have vertical dpc's; cavity wall will be closed around all openings & at plate level with special PVCu insulated cavity closures by Thermabate or similar & approved to prevent cold bridging & achieve a 'U' value of 1.2 W/M2/deg.C.

- Internal Finish: 19mm 2 coat plaster.

DETAILS:-

- Ensure cavity wall insulation board terminates 150mm below finished floor level.

- Ensure that no mortar snots or other debris fall between cavity board to bridge skins of wall.

- Ensure cavity wall insulation abuts roof insulation to reduce cold bridges.

- NOTE 1:- To reduce cold bridging ensure that the wall insulation abuts & joins to the roofing insulation to form a continuous thermal barrier.

- NOTE 2:- Build in stepped cavity trays at roof abutment locations & as work proceeds at the intersection with lead flashings to maintain a continuous water proof barrier of the structure.

- NOTE 3:- Main Contractor to establish cost of best matching brick available prior to tender & include accordingly rather than their own PC sum.

S	NEW WALL TO EXISTING WALL JUNCTION

- Where a new external wall is to be built flush with the existing wall, the new bwk. is to be toothed into the existing to maintain bwk. bonding & reduce extensive vertical joints.

- Where the outer skin is to abut an existing wall, (perpendicular junction) the new wall is to be tied to the existing wall using Furfix stainless steel channel ties raw bolted to the existing wall.

- The new inner skin blockwork is to be tied to the existing wall using stainless steel Furfix channel ties raw bolted to the existing wall.

- To prevent damp bridging through the external skin of the existing wall into the extension, the cavity must be maintained by providing a cut-in channel using a commercial disk cutter.    The cut depth must be fully through the existing external bwk.

S	MOVEMENT JOINTS IN EXTERNAL WALLS

Clay Brickwork

- Both B.R.E. Digest 164 and BS5268: Part 3 confirm that movement joints should be provided at 12m intervals in continuous brickwork. Movement joints should be provided in the following positions:-

- Intersecting walls, piers, floors etc.

- Window and door openings;

- Changes in height or thickness of walls;

- Chases in the wall;

- At the location of other movement joints in the building or floor slab.

- Refer to the drawings for details of the locations of the wall movement joints.    The width of the joint in millimetres should be about 30% more than the distance between joints in metres i.e.; movement joints at 12m centres will need to be about 16mm wide. (Unrestrained, unreinforced walls, but can be applied to walls in all situations to ere on the side of safety).

- Wall ties should be provided at 300mm vertical spacings, on either side of the joint. (Masonry is unlikely to develop tensile cracks, except in short returns of less than 1m in length).

Calcium Silicate Bricks

Movement joints should be provided in external walls at intervals of about 7.5m to 9.0m. The width of the joint should be between 10mm and 13mm.

In internal walls, movement joints are only needed if the wall is going to be plastered before the brickwork has dried out.

Lightweight Aggregate Concrete Blocks

Movement joints at 6.0m to 9.0m intervals, with a 10mm wide joint.

Autoclaved Aerated Concrete Blocks

(Refer to Durox and Thermalite literature)
Movement joints at 6.0m intervals, with a joint of 10mm wide.

Seals

Materials for joints to BS6213

- 2-part Polysulphide BS5212

- 2-part Polyurethane BS5212

- Bitumen Rubber type A1/A2 of BS2499

- Flexible Epoxide

The depth of the seal should be at least between 2 x width and 1 x width of the joint.

Filling materials: a pressure of about 0.1N/mm2 should be sufficient to compress the material to 50% of its original thickness.

Filling materials are usually Polymer Foams;

Materials not to be used are: Hemp, fibre board, cork

S	NEW LINTELS

- Lintels: to be galv. steel by either Catnic Lintels Ltd. or I.G. Engineering with cavity tray dpc's over where applicable (lintel references stated on plans). Build in weep holes in outer skin at base of cavity tray for drainage. All lintels purchased by the main contractor must be suitable for the size of external wall cavity indicated.

- Accessories & bearings: All lintels to have 150mm min. end bearing & the cavity trays are to have stopped ends - NOTE:- to prevent cold bridging & achieve a 'U' value of 1.2 W/M2/deg. C, all steel lintels must arrive on site as fully insulated at the factory.

- NOTE 1:- All lintels carrying suspended concrete floors are to be sized & selected by the Lintel manufacturer due to the increased dead loads.    The main contractor is to supply the drawings & specifications to the lintel manufacturer prior to ordering for final selection of all new lintels.

S	EXISTING LINTELS & BEAMS

- Where existing lintels or beams may be taking increased loads due to the new works:-

- The exact size & suitability of the existing lintels & beams cannot yet be determined.    Their suitability to take the increased loadings (where applicable) will be determined on site by the main contractor PRIOR TO CONSTRUCTION of the element in hand by site investigation to expose structural sizes etc.    for viewing by the site visiting Building Inspector.    Should any existing lintels or beams be defective or unsuitable to take the revised, loads then contact the Specifier immediately.

- Replacement costs: Should the existing lintels or beams need replacing then this will be at extra cost to the client. HOWEVER, it is a requirement that the extra depth cost is ‘at builders cost’ plus 10% profit & attendance rates.    All extra costs need to be substantiated with invoices & labour rates.

S	NEW SOLID GROUND FLOOR - GROUND BEARING (UN-INSULATED)

Description of floor make up is top to bottom:

- Screed: 75mm thick nominally reinforced with D49 anti-crack steel mesh.

- Concrete oversite: 125mm conc. (1:2:4) nominally reinforced with A142 anti-crack steel mesh.

- DPM: 1200g polythene dpm.

- Blinding: 50mm lean mix.

- Hardcore fill: 150mm - 300mm clean & sulphate free & consolidated in 150mm layrs.

- All new hard-core must be clean & free of all wood, vegetation, etc.    Second-hand hard-core shall not be permitted.

- Thickened floor beams:    300mm x 300mm for non-load bearing blockwork partitions as required.

- NOTE 1:- Should new floor cover existing air bricks for sub-floor ventilation, then the existing air bricks are to be ducted to new air bricks on new external wall to maintain cross ventilation.

S	NEW SOLID GROUND FLOOR GROUND BEARING (INSULATED)

Description of floor make up is top to bottom:

- Screed: 75mm sharp sand / cement screed nominally reinforced with D49 anti-crack steel mesh.

- Vapour control layer: 1200g polythene vapour barrier.

- Insulation: 75mm Celotex floor insulation board for an area to exposed perimeter ratio of about 0.70.

- Perimeter insulation: Provide perimeter insulation to exposed walls by installing 12mm Celotex insulation strips for the depth of the screed at the abutment of the screed to wall junctions.

- Concrete oversite: 125mm thick conc. (1:2:4) nominally reinforced with A142 anti-crack steel mesh.

- DPM: 1200g polythene dpm lapped & sealed to wall dpc's.

- Blinding: 50mm lean mix finished fully level & smooth.

- Hardcore fill: 150mm - 300mm clean & sulphate free & consolidated in 150mm layers.

- All new hard-core must be clean & free of all wood, vegetation etc.    Second-hand hard-core shall not be permitted.

- Thickened floor beams: 300mm x 300mm for non-load bearing blockwork partitions as required.    Ensure that any partitions are built up off the concrete oversite/thickened floor beams & NOT the screed.

- NOTE 1:- Should new floor cover existing air bricks for sub-floor ventilation, then the existing air bricks are to be ducted to new air bricks on new external wall to maintain cross ventilation.

- NOTE 2:- New finished floor levels to line through with the existing.    Main Contractor to establish existing finished floor levels PRIOR to laying the new floor.    Main contractor to establish final floor covering from client in case screed level needs to be reduced slightly to compensate.

S	NEW SOLID GROUND FLOOR - SUSPENDED CAST INSITU (INSULATED)

- Floor is to be engineered designed by the clients Structural Engineer.    Refer to the engineers details for all details of the suspended concrete floor constructional make up.

Description of floor make up is top to bottom:

- Screed: 75mm thick sharp sand / cement nominally reinforced with D49 steel mesh.

- Vapour control layer: 1200g polythene vapour barrier.

- Insulation: 75mm Celotex floor insulation board for an area to exposed perimeter ratio of about 0.50.

- Perimeter insulation: Provide perimeter insulation to exposed walls by installing 12mm Celotex (GA3012Z) insulation strips for the depth of the screed at the abutment of the screed to wall junctions.

- Concrete oversite: reinforced concrete to Structural Engineers details & suspended onto the inner skin of the new or existing walls.

- DPM: 1200g polythene dpm lapped & sealed to wall dpc's.

- Blinding: 50mm dry mix.

- Fill: 150mm - 300mm clean & sulphate free consolidated hard-core.

- All new hard-core must be clean & free of all wood, vegetation etc.    Second-hand hard-core shall not be permitted.

- Floor beams: Incorporate thickened floor beams for non-load bearing blockwork partitions as required & to structural engineers design.

- NOTE 1:- Should new floor cover existing air bricks for sub-floor ventilation, then the existing air bricks are to be ducted to new air bricks on new external wall to maintain cross ventilation.

- NOTE 2:- Should clay soils be exposed & heave protection required then The Specifier is to be informed immediately for referring to a Structural Engineer.

- NOTE 3:- New finished floor levels to line through with the existing.    Main Contractor to establish existing finished floor levels PRIOR to laying the new floor.    Main contractor to establish final floor covering from client in case screed level needs to be reduced slightly to compensate.

S	NEW GROUND FLOOR (SUSPENDED TIMBER)

- Floor finish: 22mm T & G flooring grade water resistant chipboard decking (Grade 2 or 3).

- Floor joists: 50 x 200mm C16 grade floor joists at 400mm ctrs. with strut noggin bracing at 2M ctrs.

- Fixings: Galv. steel joist hangers securely hooked into cavity (do not build joists into wall unless using the propriatory upvc end caps for sealing into joist & wall).

- Insulation: 100mm Celotex floor insulation board board between joists hung off lightweight galv. steel clips across joists for an area to exposed perimeter ratio of about 0.50.

- Perimeter insulation: Provide perimeter insulation to exposed walls by installing 12mm Celotex (GA3012Z) insulation strips for the depth of the joist & floor that abuts the exposed wall junctions.

- Vapour Control Layer: Cover joists & insulation with 1200g polythene vapour barrier prior to laying of the chipboard decking.

- Sub floor oversite: 100mm conc. oversite sub-floor (level or above external g.l.) on 1200g polythene dpm.

- Blinding: 25mm dry mix.

- Hardcore fill: 150mm clean consolidated hard-core.

- Cross ventilation: 215 x 65mm terra-cotta 'through' vents at 1.2M ctrs. for sub-floor ventilation.    Ensure that a min. 75mm clear air gap is maintained between underside of joists & conc. sub floor.

- Intermediate support walls: 105mm 'honeycomb' bwk. intermediate support walls (if required to break span of floor joists) on conc. 300 x 300mm ground beam foundations & as shown on drg. with 50 x 100mm wall plates strapped to intermediate support walls on a dpc.

- Joist strengthening: Floor joists to be doubled up & bolted together under stud walls where they run parallel.

S	NEW SUSPENDED GROUND BEAM & BLOCK FLOORS (INSULATED)

- Manufacturer / supplier: Use Bison, RMC Housefloor or Trent Jet Floor Systems in strict accordance with the manufacturers instructions (or equal & approved). The Main contractor is to supply manufacturers details & supporting calculations of the new floor beams from the final supplier direct to the site visiting Building Inspector prior to installation.

New block & beam floor to be generally as follows:-

- Lay single beams at 515mm ctrs. on Hiload dpc with Granulite or Thermalite 7N/mm2 blocks between of size 440 x 215 x 100mm.

- Grout 1:4 cement cement/sand slurry well brushed into the floor prior to the laying of

- Insulation: 75mm Celotex floor insulation board for an area to exposed perimeter ratio of about 0.5.

- Perimeter insulation: to exposed walls by installing 12mm Celotex (GA3012Z) insulation strips for the depth of the screed at the abutment of the screed to wall junctions.

- Vapour Control layer: 1200g polythene vapour check over insulation lapped & sealed to all wall dpc's.

- Floor finish: 75mm screed nominally reinforced with D49 anti-crack steel mesh.

- Sub Floor cross venting: Ensure that a minimum floor void of 150mm is maintained to aid underfloor venting.    Install proprietary stepped floor ventilators with cavity trays over at 2M ctrs. & at all corners unless otherwise stated by the manufacturers.

- The sub base floor: Covered with a concrete oversite or plastic 1200g polythene sheeting after applying a weed killer.

ALTERNATIVE FLOOR FINISH:

- 19mm Pellofloor Plus V313 moisture resistant tongue & grooved flooring grade chipboard (type 11/111 of BS. l5669 1979) laid over the insulation.    All floor T & G chipboard decking is to be applied with pva adhesive. Ensure that the vapour check is not damaged when laying of boarding & that it abuts all wall junctions.

- NOTE 1: Should underfloor heating be incorporated within the floor screed, then additional expansion gaps may be required - refer to underfloor heating system used for constructional advice prior to laying.

S	NEW INTEGRAL GARAGE FLOOR - GROUND BEARING SLAB

- Oversite: 150mm conc. (Grade 20) nominally reinforced with D49 mesh.

- Entrance edge beam: Incorporate a 600 x 450mm thickened edge beam across the garage door opening.

- Floor finish: to be trowelled smooth & laid at a small fall towards front door opening.

- DPM: 1200g polythene dpm. lapped & sealed to all walls.

- Ensure that a flexible joint is achieved between conc. floor & wall abutment.

- Blinding: 50mm lean    mix blinding finished fully level & smooth.

- Hardcore fill: 150mm clean & sulphate free consolidated hard-core.

- All new hard-core must be clean & free of all wood, vegetation etc.    Second-hand hard-core shall not be permitted.

- NOTE 1:- Should new floor cover existing air bricks for sub-floor ventilation, then the existing air bricks are to be ducted to new air bricks on new external wall to maintain cross ventilation.

- NOTE 2:- Floor level of integral garage to be a min of 100mm below floor level of main dwelling.

S	NEW STEEL BEAMS

- Design: All beams & padstones are to be designed by clients Structural Engineer. Refer to separate engineers calculations for all sizes & end bearings etc.

- Supply & install: Provide new steel beams as shown on plans;    steelwork to have 100mm min. end bearing on conc. padstones.

- End bearings: Concrete padstones to be made from 21KN 1:2:4 conc. at 28 days.    Precast concrete padstones disc cut to size on site may be used.

- Fire protection: All steel beams are to be incorporated within the floors or ceilings where possible & the fire protection will be afforded by the 12.5mm plasterboard & set ceiling coverings.    If not, provide half hr. fire protection to the ‘down stand’ beams by encasing in 2 layers of 12.5mm plasterboard nailed to timber nogs between flanges & with 1.6mm dia. wire at 200mm ctrs. & 7mm plaster set.

- Lateral support of beam: Where possible steelwork is to receive lateral support.

S	NEW INTERNAL BLOCK PARTITIONS (LOAD-BEARING)

- Blocks: 100mm Durox Superbloc lightweight conc. blocks as indicated on

- Foundations: 450mm wide x 800mm thk. X 1M    deep conc. foundations (or deeper & subject to approval by the site visiting Building Inspector).

- Footings: Common bwk. or dense conc. foundation blocks (sulphate resisting) below g.l.

- DPC: Hiload polythene dpc & sealed to dpm. or vapour check where applicable.

- Finish: 19mm 2 coat plaster finish to both sides.

- Lateral restraint: Provide top lateral restraint to partitions by mechanically tying to floor or ceiling joists at 2M ctrs. with galv. steel straps.

- Lintels: For openings up to 2.4M wide use IG box 100 lintels with 150mm min. end bearing.

- Door linings: s.w. door lining kits to suit finished block wall thicknesses.

S	NEW INTERNAL BLOCK PARTITIONS (NON LOAD-BEARING)

- Blocks: 100mm Durox Superbloc lightweight conc. blocks as indicated built up off new solid conc. floor incorporating thickened floor beams for added support.

- Finish: 19mm 2 coat plaster to both sides.

- Lateral restraint: Provide top lateral restraint to partitions by mechanically tying to floor or ceiling joists at 2M ctrs. with galv. steel straps.

- Lintels: For openings up to 1m wide use IG ‘internal’ type lintels or IG box 100 lintels for larger spans up to 2.4M all with 150mm min. end bearing.

- Door linings: s.w. door lining kits to suit finished block wall thicknesses.

S	NEW INTERNAL STUD PARTITIONS (NON LOAD BEARING)

- Framing: CLS timbers - 95 x 45mm studs at 400 ctrs.

- Noggins: CLS timbers - 95 x 45mm horizontal noggins at 600mm ctrs.

- Head & sole plates: CLS timbers - 95 x 50mm head & sole plates mechanically tied to floor & ceiling for lateral support.

- Sound insulation: Full fill sound deadening quilt between studs (eqivalent density of 10KG per M2.

- Covering: 12.5mm plywood to both sides.

- Finish: 12.5mm plasterboard with 7mm 2 coat plaster skim to both sides.

- Joist strengthening: Timber floor joists to be doubled up & bolted together under stud walls where they run parallel.

- Door linings: s.w. door lining kits to suit finished stud wall thicknesses.

S	DWARF PERIMETER STUD PARTITIONS FOR ROOMS IN ROOFS

DESCRIPTION:-

- Framing: CLS timbers - 95 x 45mm studs at 400 ctrs.

- Noggins: CLS timbers - 95 x 45mm horizontal noggins at 600mm ctrs.

- Head & sole plates: CLS timbers - 95 x 50mm head & sole plates mechanically tied to floor & ceiling for lateral support.

- Insulation: 100mm Crown Dritherm insulation board between studs & lined internally with 40mm Celotex GA3040Z insulation board. Tape joints with Idenden tape.

- Covering: 12.5mm plywood to internal side.

- Finish: 12.5mm foil backed plasterboard with 7mm 2 coat plaster skim to both sides.

- Joist strengthening: Timber floor joists to be doubled up & bolted together under stud walls where they run parallel.

- Door linings: s.w. door lining kits to suit finished stud wall thicknesses.

- NOTE 1 - PERIMETER ACCESS DOORS INTO ROOF VOIDS - Install 4 No. 600mm wide x maximum height access doors into perimeter roof voids for maintenance & access made from 12mm MDF complete with hinges, catch & handles to be agreed.    Insulate doors with 50mm Celotex & edge with timber edging strips & a 3mm plywood backing to hide & protect insulation. Install MDF linings & architrave.    Locations of access doors to be agreed on site.    Alternatively, supply & install preformed insulated door kits to requisite size.    Draft proof seals to all edges.

S	NEW FIREPLACE & STACK

- Aspiration: Contractor to form feature fireplace in lounge from decorative stone or bricks to clients design & choosing, but shall generally conform to the following:-

- Foundation: Mass fill conc. foundation as indicated on plans min. 1M deep or deeper to the approval of the site visiting Building Inspector.

- Min. thickneses: Fireplace back & jambs to be a min. of 215mm thk. solid bwk. or stone.

- Dimensions: Width of fireplace jambs not to exceed 600mm.

- Lintels: Provide a non-composite conc. lintel over fireplace opening at a height above the hearth not to exceed 600mm.

- Hearth: Form a bwk./stone hearth in front of fireplace which must not be less than 500mm projecting beyond fireplace jambs.    Hearth to extend beyond width of jambs by 150mm min. & to have a 25mm upstand fully around perimeter of hearth. Hearth to be a min. thickness of 125mm (incl. conc. floor).

- Flue: Provide 225mm dia. fire clay flue liners (female sockets face up) with 37.5 deg. bends into bwk. stack at first floor level.    Continue liner throughout stack to very top & infill around liner with Vermiculite.

- Stack: Bwk. stack to be fully keyed & tied into wall.    Extend stack through roof to form a min. chimney size of 552 x 552mm chimney flashed to roof with Code 5 lead flashings & soakers with a horizontal dpc. Finish stack 600mm above ridge level (if within 600mm of ridge line) with a decorative engineering bwk. projection finished with a clay pot bedded in sharp sand/cement flaunching.

- Limitations 1: Stack height must not exceed 4.5 times the stacks min. thickness dimension.    For taller stacks the thickness will need to be increased    accordingly or receive some other intermediate lateral restraint bracing from steel tie rods for example.

- Limitations 2: Stacks within 2.3M of a openable window must terminate 1M min. above height of window.    The stack height can be terminated below the ridge height provided the top of the stack is not less than 2.3M away from the roof slope or other such structure when measured horizontally.

- Joist trimming: Trim all floor & roof timbers around stack & breast to ensure that no combustible material is within 40mm of    breast & stack.

S	NEW FEATURE FIREPLACE

- Description: Contractor to form feature fireplace in living room using a fireplace surround kit to clients choosing complete with gas pipe adjacent ready for installation of a room sealed (balanced flue) gas fired stove to clients choosing.

- PC Sum: Main Contractor to allow a PC sum of £3,000.00 for the supply only of the fireplace kit & the feature gas fire.    Installation is deemed to be included    elsewhere within the Main Contractors quotation.

S	INSTALLATION NOTICES

- Description: A durable notice containing information on the performance capabilities of the hearths, fireplaces, flues & chimneys shall be affixed in a suitable place in the building for the purpose of enabling combustion appliances to be safely installed.

S	EXTENDING EXISTING STACK

- Generally: The new extension roof will require the existing chimney stack to be increased in height. Remove top few courses of bricks to existing stack to locate sound undamaged masonry & then extend flue stack by installing new fire clay flue liners (dia. to match existing) surrounded by red engineering bwk.    Extend stack to terminate 600mm above ridge level (if within 600mm of ridge line) or within these limits:-

- Height: Stack height must not exceed 4.5 times the stacks min. thickness dimension.    For taller stacks the thickness will need to be increased    accordingly or receive some other intermediate lateral restraint bracing from steel tie rods for example.

- Limits: Stacks within 2.3M of a openable window must terminate 1M min. above height of window.    The stack height can be terminated below the ridge height provided the top of the stack is not less than 2.3M away from the roof slope or other such structure when measured horizontally.

- Description: Continue liner throughout extended stack to very top & infill around liner with Vermiculite.    Finish stack in a decorative engineering bwk. projection finished with a clay pot bedded in sharp sand/cement flaunching.    Extended chimney flashed to roof with Code 5 lead flashings & soakers with a horizontal dpc.

S	COMBUSTION VENTILATION - GENERALLY

- Requirements: If the extension covers existing combustion ventilation provision or the new extension works have appliances that new provision for combustion ventilation the contractor is to supply & install fresh air vents for the combustion air related to the appliance installed as follows:-

FOR BALANCED FLUE APPLIANCES

- Provided the combustion unit is a sealed unit with a balanced flue there is no requirement to provide a fresh air vent to the room.    If the appliance is within a cupboard or enclosed within a tight space, top & bottom ventilation must be provided to prevent the appliance from overheating.

FOR OPEN FLUE APPLIANCES

- FOR GAS BOILER:- 		450mm2 per 1kW of rated input over 7kW.

- FOR GAS COOKER:- 	5000mm2 for rooms less than 10M3 in volume.

- FOR SOLID FUEL:- 		550mm2 per kW of rated output over 5kW. If the appliance has

draught stabilisers fitted then an additional 300mm2 per kW of free

area is to be provided.

- FOR SOLID FUEL BURNING

- OPEN APPLIANCE:- 		50% of the appliance throat area (see B.S.8303:1986).

- FOR OIL:- 			550mm2 per kW of rated output over 5 kW

- The vents: are to be located within the room that has the burning appliance & ideally as close to the appliance as possible & located so as to prevent drafts.    All areas stated are free areas.    All vents are to be permanent air entries without any adjusters.

S	EXTERNAL FLUE LOCATIONS - GENERALLY

- BALANCED FLUES:-	600mm min. distance to a wall opening for a type 1 oil burning appliance.

300mm min. distance to a wall opening for a type 2 gas appliance.

All flues to be fitted with a durable guard.

All fan assisted flues must be a min. of 2.1M above FFL & 2.5M from the boundary if flue 		projects towards that boundary.

- For all flue sizes refer to table 2 of J1/2/3 Section 2 of the approved Documents of the Building Regulations.

S	MORTAR MIXES - GENERALLY

Application

Cement

Builders (soft) sand

Lime (if chosen)

Sharp sand

House walls - bricks

1

5

2

Ditto Blocks

1

5

1

Retaining walls brick

1

3

2

1

Retaining walls blocks

1

3

1

1

Coping stones & cills

1

2

½

2

Chimneys

1

5

2

Haunching for chimneys

1

1

4

Joints for paving

1

3

Bed for paving slabs

1

6

Pointing brickwork

1

3

2

Floor screed

1

4

External render 1st coat

2

3½

1

3½

Ditto 2nd coat

2

4½

2

4½

NOTES:-

- All cement to be OPC sulphate resisting.

- The mortar mix may need to be adjusted to suit the strength of final bricks & blocks used.    Refer to manufactures of bricks / blocks for recommended mortar mixes.

- In general, the mortar strength should not be greater than the strength of the units used. With some types of materials e.g. calcium silicate, aerated concrete and dense concrete, it is generally advisable to use a weaker mortar in order to reduce the effect of shrinkage stresses.

- It is recommended that deliveries of factory produced lime:sand for mortar should be tipped, unless delivered in skips, onto a clean area with a sealed base, slab or similar prepared area and sheeted when not in use. Sheeting is most important when colour is a requirement, as rain and weathering may otherwise cause separation of some of the fine material.

- To obtain the required mortar, considerable care should be taken on site to add the correct amount and type of cement. Gauge boxes or other accurate measuring vessels should be used when proportioning is by volume. When using coloured mortars, it is strongly advised that the same brand, type and source of cement is used throughout the contract. If possible, gauge by weight.

- For the quantity of cement for the various designations of mortar, see the Table above.

- Only clean water should be used for obtaining the correct consistency.

- Stacked bricks and blocks should be protected. Never use either when saturated with water as this can cause disfiguration of the building and in winter, attack on the masonry by frost. New work should be protected from rain at the end of the working day or when rained off. Scaffold planks placed along the top of walls give at least some protection from saturation by rain. Ideally, the tops of unfinished walls should be properly covered; this is especially important with cavity walls and where perforated bricks are used.

- Although it is recommended to use cement:lime:sand mortars within 2-3 hours, most cement:lime mortars will remain fairly soft for a working day, making cleaning up at the end of each shift reasonably easy. All mixers, spot boards and trowels, etc., should be cleaned at the end of each shift or when changing the colour of the mortar.

- Recommended Mortar Mixes for Various Conditions

- Reference should be made to British Standard BS 5628, having regard to the requirements for strength durability and appearance.

- Maintenance

- Generally, walls constructed with mortars containing lime require the minimum of maintenance.

- There is a real danger of contact dermatitis or serious burns if skin comes into contact with wet mortar. Wear suitable protective clothing and eye protection. Where skin contact occurs either directly or through saturated clothing wash immediately with soap and water. For eye contact immediately wash out eyes thoroughly with clean water. If swallowed wash out mouth and drink plenty of water. The relevant codes of practice, standards and statutory regulations must always be observed.

S	NEW FIRST FLOOR (SUSPENDED TIMBER)

- Decking: 22mm tongue & grooved water resistant flooring grade chipboarding to be nailed at EVERY joist after cramping into position.

- Joists: 50 x 200mm floor joists at 400 ctrs. with strut noggin bracing    at 2M ctrs.

- Trimmers: For trimmers use doubled or tripled up joists bolted together.

- Support: Galv. steel joist hangers hooked into cavity. (do not build joists into wall).

- Ceilings: 12.5mm foil backed plasterboard with 5mm skim coat for ceilings.

- Loads: Floor joists to be doubled up & bolted together under stud walls where they run parallel.

- Lateral wall support: Walls parallel to joists are to receive lateral support using 38 x 5mm galv. steel straps at 1200 ctrs. straps to span 3 joists/rafters with strut noggins between joists at strap locations.

- NOTE 1:- Insulation - For exposed area of first floor (over garage & at eaves cupboards of rooms in roof) provide insulation between the joists by installing 200mm mineral wool insulation quilt between the joists & another 100mm insulation quilt over the joists.

- NOTE 2:-    Fire protection - For added fire protection over integral garage the garage ceiling is to be 2 layers of 12.5mm foil backed plasterboard with lapped & sealed joints finished with 7mm plaster skim after fire stopping all gaps at wall to joist junctions using compressed mineral wool.

- NOTE 3:-    Sound insulation - For extra sound insulation add 100mm mineral woolslab between the new joists (10KG/M3 density).

S	NEW FIRST FLOOR - PRE-CAST CONC. BEAM & BLOCK

- Manufacturer / supplier: Use Bison, RMC Housefloor or Trent Jet Floor System (or equal & approved) in strict accordance with the manufacturers instructions. The Main contractor is to supply manufacturers details & supporting calculations of the new floor beams from the final supplier direct to the site visiting Building Inspector prior to installation.

New block & beam floor to be generally as follows:-

- Lay single beams on 100mm end bearings at 515mm ctrs. with Granulite 7N/mm2 blocks between of size 440 x 215 x 100mm.

- Between blocks install 75 x 40mm nom. tanalized timber battens by Trent & grout the joint above.    Fix 50 x 50mm ceiling battens running parallel with main beams at 550mm ctrs. ready for 12.5mm plasterboard.

- Grout 1:4 cement cement/sand slurry well brushed into the floor.

- Insulation 50mm Celotex floor insulation board with 12mm celotex perimeter insulation to each room for thermal movement of underfloor heating provision.

- Screed: Floor finish to be 75mm screed nominally reinforced with D49 anti-crack steel mesh.

- ALTERNATIVE FLOOR FINISH: 19mm Pellofloor Plus V313 moisture resistant tongue & grooved flooring grade chipboard (type 11/111 of BS. l5669 1979) laid over the insulation.    All floor T & G chipboard decking is to be applied with pva adhesive. Ensure that the vapour check is not damaged when laying of boarding & that it abuts all wall junctions.

- Incorporate service ducts as required either on top of blocks by using a 75mm thk. block instead of 100mm thk. block.    25mm screeding to reduce duct widths.    Alternatively service ducts can be incorporated below the blocks running between the ceiling battens.

- Refer to manufacturers details & instructions;    it is a requirement that the floor manufacturer used shall furnish the contractor with a set of layout instruction drawings & details prior to installation for the main contactor to supply to the Building Control Dept. to discharge the Conditional Approval requirements.

- Lateral wall support: Walls parallel to floor beams are to receive lateral support using 38 x 5mm galv. steel straps at 1200 ctrs. straps to span 2 floor beams.

- NOTE: Should the first floor have underfloor heating installed then it is important to incorporate any additional precautions within the floor / screed to ensure that no cracking occurs due to excessive thermal movement. Reference must be made to the underfloor heating system being used for any extra works that may be required.

S	NEW FLAT ROOF - COLD DECK

CARCASING FROM:-

- 19mm exterior grade waterproofed OSB decking on

- 1:60 falls 50mm x 50mm s.w. firrings at 400mm ctrs on

- 50mm x 200mm joists at 400 ctrs. spiked to 50 x 100mm wall plates.

- 50mm x 200mm strut noggin bracing at 2M max. ctrs.

- 100mm x 50mm s.w. water check fillets to roof verges.

- All roof wall plates to be securely strapped to wall using 38mm x 5mm galv. steel straps at 2000 ctrs. straps to span 600mm of wall.

- Walls parallel to joists are to receive lateral support using 38mm x 5mm galv. steel straps at 1200 ctrs. straps to span 3 joists/rafters with strut noggins between joists at strap locations.

INSULATION:-

- 100mm Celotex insulation installed between the joists to achieve a 50mm air gap over.    Install a further 40mm Celotex insulation board layer under the joists.    Tape joints with Idenden tape.

INTERNAL FINISH:-

- 12.5mm foil backed plasterboard ceiling finished with 5mm "Thistle" board finish plaster.

EXTERNAL DETAILS:-

- White upvc soffits & fascia boards fixed to treated s.w. battened out framing.    Soffit board to incorporate 25mm continuous air vents with fly screens for cross ventilation of roof void.    Ensure that a clear 50mm air gap is maintained above the insulation for adequate cross venting of the roof void.

- Lap felt 450mm min. up under tiles & sarking felt & flash with Code 5 lead.

- Raise felt 150mm up wall & flash with Code 5 lead flashings dressed 150mm up wall into bwk. mortar joint at junction of flat roof & house wall.

FOR HIGH PERFORMANCE GRADE USING POLYESTER BASED ROOFING SYSTEMS.

- Torch-on roofing system using Ruberoid polyester felts.

- First layer to be polyester base or glass base torch-on underlay.

- Second layer to be fully bonded polyester base torch on cap sheet.

- 13mm mineral stone chippings on a polymer modified bitumen based adhesive compound.

GRP FLAT ROOF COVERING

- Supplier: ‘Roof Line’ GRP system. (or equal & approved).

- Installer: Must be trained & approved by system manufacturers.

- Construction of GRP Roofs should only carried out in temperatures above 5 degrees and in dry weather.

- Supply and install 19mm treated timber battens fixed to the roof perimeter, fixed 10mm below the roof deck to allow roof expansion and vapour egress.

- Supply, cut to shape and install one layer of 2mm nom FLEX-BASE Specially Prepared Preformed GRP Sheet, forming all upstands, downturns and incorporating all necessary flashing detail, lapping all sheets by at least 30mm to form the base layer.

- Supply and install water-check and or drip detail preformed GRP roof edge trims, secured to the previously fixed 19mm treated timber battens. Face fix trims every 300mm with appropriate colour coded fixings.

- Laminate together all preformed base layer and detail using 100mm wide 450gsm Chopped Strand Glassfibre Mat consolidated in Pre-accelerated Polyester Laminating Resin creating an intrinsically waterproof single layer structure.

- At one square metre intervals create Secret Mobile Negative Wind Lift fixings and laminate in, using part preformed materials ensuring freedom of joint movement, and laminate in. Only appropriate non-corrosive fixings suitable for the roof deck/joists to be used.

- Wet lay not less than one further layer of 450gsm Chopped Strand Glassfibre Mat consolidated in Pre-accelerated Polyester Laminating Resin to the entire roof area with ratios of Resin to Mat all as per manufacturer's recommendations and allow to cure.

- To the newly created construction apply by brush a liberal covering of Pre-accelerated UV stable Polyester Flowcoat (Gelcoat), Colour to be specified, into which (if required) broadcast a layer of 6mm clean limestone chippings (or similar) prior to cure to ensure a good bond.

- NB: A range of other finishes may also be considered which may be applied during or after the Flocoat is cured. Imitation lead roofs can be constructed incorporating roll mops and flashings.
The above specification can be altered to suit the desired strength/appearance, but the basic principles of an independent roof covering able to tolerate thermal and structural movement must be adhered to.

S	NEW FLAT ROOF - WARM DECK

CARCASSING FROM:-

- Joists: 50mm x 150mm flat roof joists at 400mm ctrs. spiked to 50 x 100mm wall plates.

- Water check fillets: 100mm x 50mm SW water check fillets to roof verges.

- Strut noggin bracing: at 2M max. ctrs.

- Firings: 1:60 fall 50mm x 50mm s.w. firings at 400mm ctrs.

- Roof wall plates: 50 x 100mm treated s.w. timber to be securely strapped to wall using 38 x 5mm galv. steel straps at 2000 ctrs. straps to span 600mm of wall.

- Lateral wall support: Walls parallel to joists are to receive lateral support using 38 x 5mm galv. steel straps at 1200 ctrs. straps to span 3 joists/rafters with strut noggins between joists at strap locations.

INSULATION:-

- Specialist warm deck insulation board system laid over the joists / firings comprising of 116mm overall thickness composite insulated decking with 110mm celotex pre-bonded to 6mm ply decking. Additional layer of 19mm OSB waterproofed Sterlin board decking ready to accept the felts & a vapour control layer.    Install standard board sizes (2.44 x 1.22M) with nogs under all board joints in strict accordance with the manufacturers instructions.

- NOTE: The Contractor may use another specialist warm deck roofing system (equal & approved) provided it is approved by the Building Control Dept. & laid in strict accordance with the manufacturers instructions.

EXTERNAL DETAILS:-

- White upvc soffits & fascia boards fixed to treated s.w. battened out framing.    Soffit board to incorporate 25mm continuous air vents with fly screens for cross ventilation of roof void.    Ensure that a clear 50mm air gap is maintained above the insulation for adequate cross venting of the roof void.

- Lap felt 450mm min. up under tiles & sarking felt & flash with Code 5 lead.

- Raise felt 150mm up wall & flash with Code 5 lead flashings dressed 150mm up wall into bwk. mortar joint at junction of flat roof & house wall.

FOR HIGH PERFORMANCE GRADE USING POLYESTER BASED ROOFING SYSTEMS.

- Torch-on roofing system using Ruberoid polyester felts.

- First layer to be polyester base or glass base torch-on underlay.

- Second layer to be fully bonded polyester base torch on cap sheet.

- 13mm mineral stone chippings on a polymer modified bitumen based adhesive compound.

- Code 5 lead flashings dressed 150mm up wall into bwk. at junction of flat roof & house wall.

- White upvc soffits & fascia boards fixed to treated s.w. battened out framing.

GRP FLAT ROOF COVERING

- Supplier: ‘Roof Line’ GRP system. (or equal & approved).

- Installer: Must be trained & approved by system manufacturers.

- Construction of GRP Roofs should only carried out in temperatures above 5 degrees and in dry weather.

- Supply and install 19mm treated timber battens fixed to the roof perimeter, fixed 10mm below the roof deck to allow roof expansion and vapour egress.

- Supply, cut to shape and install one layer of 2mm nom FLEX-BASE Specially Prepared Preformed GRP Sheet, forming all upstands, downturns and incorporating all necessary flashing detail, lapping all sheets by at least 30mm to form the base layer.

- Supply and install water-check and or drip detail preformed GRP roof edge trims, secured to the previously fixed 19mm treated timber battens. Face fix trims every 300mm with appropriate colour coded fixings.

- Laminate together all preformed base layer and detail using 100mm wide 450gsm Chopped Strand Glassfibre Mat consolidated in Pre-accelerated Polyester Laminating Resin creating an intrinsically waterproof single layer structure.

- At one square metre intervals create Secret Mobile Negative Wind Lift fixings and laminate in, using part preformed materials ensuring freedom of joint movement, and laminate in. Only appropriate non-corrosive fixings suitable for the roof deck/joists to be used.

- Wet lay not less than one further layer of 450gsm Chopped Strand Glassfibre Mat consolidated in Pre-accelerated Polyester Laminating Resin to the entire roof area with ratios of Resin to Mat all as per manufacturer's recommendations and allow to cure.

- To the newly created construction apply by brush a liberal covering of Pre-accelerated UV stable Polyester Flowcoat (Gelcoat), Colour to be specified, into which (if required) broadcast a layer of 6mm clean limestone chippings (or similar) prior to cure to ensure a good bond.

- NB: A range of other finishes may also be considered which may be applied during or after the Flocoat is cured. Imitation lead roofs can be constructed incorporating roll mops and flashings.
The above specification can be altered to suit the desired strength/appearance, but the basic principles of an independent roof covering able to tolerate thermal and structural movement must be adhered to.

INTERNAL FINISH:-

- 12.5mm foil backed plasterboard ceiling finished with 5mm "Thistle" board finish plaster.

- NOTE: External walls & cavity insulation is to be taken up to underside of roof decking to form a continuous thermal barrier & reduce cold bridging.

S	NEW PITCHED ROOF - TRADITIONALLY CARCASSED - COLD DECK

- GENERAL REQUIREMENTS:-

- Code of practice: Comply with BS 5534 Slating and tiling subject to any qualifications given below.

- Materials: Main Contractor to supply samples of roof covering materials for clients / specifies approval prior to ordering.    Main Contractor to establish cost of best matching tile / slate available prior to tender & include accordingly rather than their own PC sum.

- Workmanship - Before slating / tiling: Preliminary checks: Check that pipes, etc penetrating the roofing, cloaks, roof gutters, firebreaks and all other pre-roofing work by other trades are completed, particularly any work involving disturbance to the structural roof timbers. Check that all tilting fillets and firrings, etc are fixed or available for fixing by the slaters / tilers.

- Reuse of existing tiles / slates: Main Contractor may reuse existing tiles or slates that are in sound condition & after checking that they will be suitable for another 30 years minimum life. Reuse in large areas - do not patch apply existing materials.

- Reuse of existing tiles / slates: Main Contractor may not reuse existing tiles or slates.

GENERALLY:-

- Pitch:     **    deg. approx. pitch but to match existing.

- Slates / Tiles:     **    tiles or slates to match existing (to be prior agreed with client & Planning Dept.).

STRUCTURAL ROOF CARCASSING:-

- Rafters: 50 x 125mm at 400 ctrs.

- Ceiling Joists: 50 x 175mm ceiling joists at 400 ctrs. With strut noggin bracing at joist mid span.

- Trimmers: for loft hatch use double joists spiked together.

- Ridge Board: 38 x 225mm.

- Hip rafters: 38 x 175mm hip rafters.

- Valley layer spiking boards: 38 x 200mm valley layer spiking boards.

- Hip layer spiking boards: 38 x 200mm hip layer spiking boards.

- Purlins: 75 x 200mm purlins.

- Struts: 100 x 100mm struts at 1.2M ctrs.

- Wall plates: 50 x 100mm wall plates securely strapped to wall at 2000mm ctrs. using 38 x 5mm galv. steel straps spanning 600mm of wall.

- Eaves overhang: Soffit projections & fascia heights to match the existing roof details.    Use treated s.w. batten framing.

- Barge boarding projections: Projection from wall face & height of barge boarding to match existing house details. Use treated s.w. batten framing combined with gable ladders.

FOR SLATES:-

To roof slopes: Lay slates as follows with spalled edges uppermost:

a) Twice centre nail every slate except very small slates which shall be once head nailed.

b) Lay close jointed with straight horizontal and vertical joints, vertical joints centred over the slate below.

c) Lay close jointed in straight horizontal courses diminishing from eaves to ridge with width of slates selected to centre vertical joints over the slate below.

d) Use thicker slates in lower courses and ensure each slate lays truly flat on the slate below.

e) Use slates of the same thickness in any one course and fix with thinner end uppermost.

f) Maintain bond at ends of courses with slates of extra width.

FOR TILES:-

To roof slopes: Lay tiles as follows:

a) Twice nail every tile in every fifth course.

b) Lay slightly open jointed with straight horizontal and vertical joints, vertical joints centred over the tile below.

c) Maintain bond at ends of courses with width-and-a-half tiles.

DETAILS:-

- Ridges: Ridge coping tiles to match existing roof details.

a) Ensure that the underlay capping strip is in position.

b) Gauge the slating / tiling to ensure that the full lap is maintained in the top course.

c) Bed the specified units in mortar.

d) Fill and gallet exposed ends.

e) Cut mitred joints at hip intersections.

- Hips: Hip copings tiles to match the existing house details.

a) Ensure the additional strip of underlay is in position.

b) Cut slates /tiles to the line of the hip.

c) Bed specified hip fittings in mortar commencing at the lowest point.

d) Fill and gallet exposed ends.

e) Fix hip irons with two galvanized screws.

- For Mitred hips: Form as follows:

a) Ensure the additional strip of underlay is in position.

b) Cut slates / tiles to the line of the hip.

c) Lay width-and-a-half slates in alternate courses/Lay extra wide slates in alternate courses.

d) Fix lead soakers into each course lapped and nailed at head.

- For bonnet hips: Fix as follows:

a) Cut tiles to the line of the hip.

b) Nail the end tile in every course.

c) Commence at foot of hip with exposed end filled and galletted.

d) Lay bonnet tiles to course with the general tiling; nail each unit and bed in mortar.

- Battens: 38mm x 25mm treated s.w. battens at the required gauge.

a) Free from decay or insect attack.

b) Free from any splits and shakes.

c) Wane permissible on one arris only.

d) Knots (or knot holes) less than one third width of any sound surface of occurrence.

e) Treated with preservative: (i) organic solvent; double vacuum or 3 min immersion (ii) Boron diffusion.

f) 22%    Moisture content at time of fixing.

- Securely fix with galvanized nails to every rafter or through counter battens into every rafter as follows:

a) At centres to give the correct slate / tile laps.

b) In long lengths (min length to be supported by at least three rafters with both ends supported).

c) With ends cut square and centred over rafters and nailed without splitting.

d) Packed where necessary to maintain level.

e) Provide additional battens and tilting fillets etc at eaves, ridges and other perimeter features.

f) Provide additional battens if necessary to prevent windlift of horizontal laps in the underlay.

g) Nail penetration into rafters, 50 mm min.

- Nails: Use nails of adequate length and gauge to provide secure fixing as follows:

a) For slates: Aluminium alloy to BS 1202: Part 3/Copper to BS 1202: Part 2/silicon bronze to BS 5534

b) For underlay: Extra large head of aluminium alloy/copper/galvanized steel to BS 1202: Part 1.

c) For battens: Galvanized steel to BS 1202: Part 1 sized to give a secure fixing without splitting the batten.

d) Do not nail through any plumber's metalwork.

- Clips for slates: Aluminium alloy to BS 5534/stainless steel to BS 5534.

- Hip irons: Galvanized steel to BS 5534.

- Mortar: Cement mortar (1:3), coloured to match slate or tile fittings.

a) Use the minimum amount necessary to provide adequate bedding and secure weatherproof fixings.

b) Solidly bed edges and joints of slating / tiling fittings until mortar squeezed out.

c) Immediately strike off surplus mortar and press back with trowel where exposed to form a dense smooth neat finish.

d) Keep mortar off the edge of slates and tiles.

e) Provide a neat finish and reduce the mass of mortar in exposed ends and edges, etc by pushing in tile or slate slips.

- Undercloaks: 6 mm asbestos-free fibre cement board of approved make with the performance standards of asbestos cement to BS 690.    Fix with rougher face uppermost as mortar key.

- Eaves: Lay a double course with undercourse of short slates / tiles head nailed to the bottom batten. Fix with bottom edges in line and projecting over the fascia.

- Verges: Form verges with an upward tilt as follows:

a) Ensure that the underlay extends to the verge line.

b) Lay an undercloak to project 50mm consisting of fibre cement board / a course of slates / a course of plain tiles (but to match the existing house detail) bedded in mortar / fixed with galvanised steel nails.

c) Lay width-and-a-half slates / tiles in alternate courses/ bedded in mortar and finished flush with undercloak.

d) Do not use any cut slates at verges.

- Underfelt: Breathable & untearable roofing felt by Tyvek.    Note - use of this felt eliminates the need to install perimeter & ridge vent venting. Fix as follows:

a) Lay parallel to eaves commencing at eaves and with a slight drape between rafters.

b) Lap 150 mm min at sides and ends with end laps centred over rafters.

c) Nail sufficiently to make secure until battens fixed.

d) Supply & install proprietory eaves carriers dressed under roofing felt, lay over any tilting fillets and extend into gutters. Prevent any sagging which could trap water.

e) Lay a strip over the ridge to overlap general underlay by 150 mm min.

f) Lay 600 mm wide strip at verge below general underlay and extend fully to the eaves line.

g) Lay 600 mm strip at hips over the general underlay and extend fully to the eaves line.

h) Extend up wall at top abutment to suit cover flashing.

i) Upon completion of battening ensure that the roof is proof against adverse weather conditions pending the laying of the slates or tiles.

WEATHERINGS & ABUTMENTS:-

- Soakers: Code 5 lead rolled sheet.

- Flashings: Code 5 lead rolled sheet.

- Valleys - tiles: Special valley tiles over an extra layer of felt.

- Valleys - lead: Form lead valley gutters using 12.5mm marine plywood base; treated softwood tilting fillets to form trough;    cover in Code 5 lead dressed 250mm under tiles & sarking felt;    sharp sand/cement to cut tile edges with waterproofing agent to the mix to seal valley tile / slate verges. Lay an undercloak of fibre cement board fixed with galvanized nails.    Cut tiles / slates to the line of the valley verge to leave    50mm between valley verges.    Lay width-and-a-half tiles in every alternate course bedded in mortar and nailed.

- Open valleys: Form as follows:

a) Ensure the additional strip of underlay is in position under the general underlay before the grp gutter is fixed.

b) Cut slates to line of gutter to leave 100 mm min width between valley verges.

c) Lay width-and-a-half slates /tiles in alternate courses without undercloak or mortar.

- Laced valleys: Form as follows:

a) Ensure the additional strip of underlay is fixed in position.

b) Fix the valley board with feather-edged boards each side over the battening.

c) Fix triangular tilting fillets at base of valley.

d) Continue slate / tile courses upwards in an even curve to the centre of the valley.

e) Form passing courses in valley with double width slates.

- Swept valleys: Form as follows:

a) Ensure the additional strip of underlay is fixed in position.

b) Fix the valley board with feather-edged boards each side over the battening.

c) Cut slates /tiles to correct taper from slates / tiles of sufficient size to maintain specified laps.

d) Lay slates in an even sweep through the valley.

- Abutments at top edges: Ensure that underlay projects above the slating / tiling and finish slating with short course to maintain gauge.

- Abutments at sloping edges: Finish with width-and-a-half slates / tiles in alternate courses. Finish with extra wide slates / tiles in alternate courses and dress in lead soakers to receive flashings.

- Fascias, soffits, barge boarding: treated batten framing to support white upvc’s soffits, fascias & barge boarding.

- Where a roof abuts a new wall: ensure code 5 lead flashings connect to & is dressed under cavity tray dpc’s of wall.

INTERNAL FINISHES:-

- Ceilings: 12.5mm foil backed plasterboard with 5mm plaster skim.

- Loft hatch: Preformed door tray & frame (530mm x 745mm).    Supplier:- www.accesspanels.co.uk (or equal & approved) – formed from 1.0mm and 1.2mm Zintec electro-galvanised mild steel with 25 mm wide picture frame finished in polyester powder coat lightly textured white RAL 9010 20% Gloss.    Hinge & Lock mechanism – Factory fitted removable pivot plates with inserted steel pins located into holes in the frame. Budget or Euro-profile lock case bolted onto the return fold of the door tray with tongue locating into a slot in the frame. Lock access hole covered with white plastic collar and plug or patented ‘Touchflap’.    Draughtstrip – Black expanded polyurethane draughtstrip 3 x 10mm to all four sides. Insulation – Rigid fire resistant insulation block CFC & HCFC Free.    U-Value - 0.35.    Fire Certification - Panels tested by Chiltern International Fire (TRADA) Ltd. Test number Chilt/IF02059 20th May 2002.
Preparation - Prepare a clear trimmed & protected opening in the structure 5mm larger than the panel ordered. Designed to fit between 600mm centre trusses fix cross trimmers spanning between the trusses to form a minimum clear opening of 775 x 553 mm. Line the inner four faces with 10 or 12.5mm thick plasterboard to suit the frame. (Where space is at a premium any suitable insulating material can be used to protect the truss from heat transfer from the metal frame.)
Fitting - Offer up the panel and screw fix using 50 mm long screws through the pre-punched holes to all four sides of the frame. Complete the operation with door in frame if possible. To remove the door from the frame unscrew the right hand pivot pin plate and slide the plate towards the middle of the door. The door can now be slid out of the frame. To replace locate the other pivot pin plate in the frame hole, ensure the centre pin is within its slot then replace the previously removed pivot pin plate ensuring the pin fully locates back into the frame hole. 
Operation - The plastic plug in the door face can be removed with a fingernail. Then use the square drive supplied ‘T’ key to open and close whilst supporting the door.

INSULATIONS:-

- Insulation 1: insulate horizontal ceilings / roof with 200mm Crown Wool between the ceiling joists & 100mm Crown Wool insulation over the joists.

- Insulation 2: For all sloping soffits (scaling ceilings) with breathable felts (new rafters):-

- 1 layer of 100mm Celotex GA3090Z thermal Sheathing fixed between the rafters & another layer of 40mm Celotex GA3040Z underlining fixed to the underside of the rafters.    Tape the joints with Idenden sarking tape.    Cover with 12.5mm foil backed Plasterboard & 5mm skim.

OTHER:-

- Connections: All bolts used in construction of roof to be 12mm with 50mm spread washers & toothed timber connectors - all junctions & connections in roof MUST be bolted (where possible).

- Strapping: All roof wall plates to be securely strapped to wall using 38 x 5mm galv. steel straps at 2000 ctrs. straps to span 600mm of wall.

- Lateral restraint: Gable end walls parallel to rafters & all floor or ceiling joists are to receive lateral support using 38 x 5mm galv. steel straps at 1200 ctrs. straps to span 3 joists/rafters with strut noggins between joists at strap locations.

- Fire breaks: Pack between slating and separating walls with mineral wool. Ensure there is no continuation of underlay through the fire break.

- To resist any horizontal thrusts: imposed from unrestrained rafters that cannot be fixed to parallel joists, the rafter feet are to be tied to perpendicular joists using 38 x 5mm galv. steel twist ties with strut noggins between.    Straps to span 3 joists.

- Ensure roof insulation: abuts with & joins to the wall insulation to form a continuous thermal barrier.

- Loft walkway: Due to insulation felt covering ceiling joists, install 600mm wide x 19mm thk. plywood walk ways through highest section of roofs for safe access for maintenance.

------------------------------------------------------------------------------------------------------------------------------------

S	SINGLE PLY FLAT ROOFING SYSTEM - NON SPECIFIC

- Generally: Final flat roof covering is to be a single ply system by a local specialist yet to be selected. This is made up of single sheets of material in which synthetic polymers are the main component. EDPM or TPO membranes are either loosely laid or fixed directly to the ‘deck’ of the roof. Their life expectancy is 25+ years.

- As these roof coverings are deemed to be specialist & require propriatory trained installers, they are probably outside the scope of the main contractors local flat roofing contractor.

- Requirement: The Main Contractor is to source 2 local single ply flat roofing specialists of his own choice & supply the client / specifier with the two roofing systems for approval & selection.    Roofing system to include all necessary fittings & detailing for a complete finished work item.    A minimum 25 year guarantee of performance & installation certificate will be required from the specialists supplier / installer.

- Specialist suppliers to consider: Sarnafil Ltd. Permanite Engineered roofing Systems, RubberFix, Polyroof Products Ltd.

- BBA Certificate: The final supplier / installer must have a current BBA Certificate for the flat roof single ply roofing system to be used.

S	SINGLE PLY FLAT ROOFING SYSTEM - MATERIAL SPECIFIC

- Waterproof membrane: RubberBond FleeceBack calendared/extruded and vulcanised EPDM membrane with polyester fleece backing

- Manufacturer: Flex-R Ltd 5 Central Park, Bellfield Road, High Wycombe, Buckinghamshire HP13 5HG. Tel 01494 448792. Fax 01494 465393. Email enq@flex-r.co.uk.

- Thickness: 2.54mm including fleece. Colour: Grey/Black

- Installers: Main Contractor to locate approved local installers of the Rubberbond EPDM system for a supply & fix installation.    Main Contractor to provide attendance to the flat roof suppliers / installers.

EXECUTION GENERALLY

BEFORE INSTALLING ROOF COVERING ENSURE THAT:

• Moisture content and stability of substrate: must not impair the integrity of the roof.

• Decking to be fully fitted, secure, smooth and free from defects.

• Surface to be cleaned of all dust/debris.

ADVERSE WEATHER

• The membrane must not be laid in wet or damp conditions or at temperatures below 5 °C unless suitable precautions/measures are taken.

INCOMPLETE WORK: At the end of the working day:

• Temporarily seal the membrane to the deck to prevent any water infiltration.

• Protect overlay board with temporary seal.

WATERPROOF COVERING / ACCESSORIES

ADHESIVE BONDING:

• Lay sheets on a continuous even coating of adhesive, taking care to avoid wrinkles.

• Ensure there is a full bond over the whole surface, with no air pockets.

• Verify that the surfaces to receive the RubberBond FleeceBack adhered membrane are clean, dry, smooth and free from defects.

• Position the membrane and allow for a minimum 75mm overlap. Apply deck adhesive to the substrate with a medium nap roller or squeegee ensuring continuous and even distribution of the adhesive, avoiding puddles.

• Roll the membrane into the adhesive coated substrate avoiding wrinkles in the sheet. Immediately smooth with a squeegee to achieve maximum contact.

ADHESIVE JOINTING FOR BONDED MEMBRANE:

• Lap side joints not less than 75mm ensuring that water will drain over and not into them.

• Seal all side joints using HP-250 Primer and SecurTape.

• At end laps overlay butt edges with a 150mm wide strip of PS Flashing.

PERIMETER DETAILS

• Including upstands, edge trims, drips, kerbs, etc. to be formed from RubberBond FleeceBack membrane in accordance with RubberBond specfications.

TERMINATIONS

• Install plastisol metal cover flashings to abutment walls, dress membrane up the vertical, under the cover flashing and fasten at the top.

ROOF PENETRATIONS THROUGH MEMBRANE

• Roof membrane: Cut around penetrations

- Type: Form from Uncured Pressure Sensitive tape or pre formed flashings.

- Seating: Flush to roof membrane. Roof membrane to flange overlap: 75mm.

SURFACING/COMPLETION

- Upon completion Flex-R Ltd Technical Services should be notified to make a final inspection of the works.

- Roof areas should be clean with all roof outlets clear and running free.

- All work by others necessary to provide a weather tight finish should be satisfactorily completed.

S	FLAT ROOF OF ‘CROWNED TOP’ PITCHED ROOF - UN-INSULATED

CARCASING FROM:-

- 47 x 150mm flat roof joists at 400mm ctrs. spiked to 50 x 100mm wall plates.

- Timber or steel ridge beams as indicated on plans to Structural Engineers design.

- 100mm x 50mm SW water check fillets to roof verges.

- Strut noggin bracing at 2M max. ctrs.

- 1:40 fall 50mm x 50mm s.w. firings at 400mm ctrs.

- Trim around bwk. flue stack using double joists & rafters bolted together to ensure that a gap of 40mm is maintained between face of stack & timbers.

INSULATION:-

- This will be provided by the lower level ceiling.

FOR HIGH PERFORMANCE GRADE USING POLYESTER BASED ROOFING SYSTEMS.

- Torch-on roofing system using Ruberoid polyester felts.

- First layer to be polyester base or glass base torch-on underlay.

- Second layer to be fully bonded polyester base torch on cap sheet.

- 13mm mineral stone chippings on a polymer modified bitumen based adhesive compound.

- 12.5mm foil backed plasterboard ceiling finished with 5mm "Thistle" board finish plaster. Scrim tape all joints.

GRP FLAT ROOF COVERING

- Supplier: ‘Roof Line’ GRP system. (or equal & approved).

- Installer: Must be trained & approved by system manufacturers.

- Construction of GRP Roofs should only carried out in temperatures above 5 degrees and in dry weather.

- Supply and install 19mm treated timber battens fixed to the roof perimeter, fixed 10mm below the roof deck to allow roof expansion and vapour egress.

- Supply, cut to shape and install one layer of 2mm nom FLEX-BASE Specially Prepared Preformed GRP Sheet, forming all upstands, downturns and incorporating all necessary flashing detail, lapping all sheets by at least 30mm to form the base layer.

- Supply and install water-check and or drip detail preformed GRP roof edge trims, secured to the previously fixed 19mm treated timber battens. Face fix trims every 300mm with appropriate colour coded fixings.

- Laminate together all preformed base layer and detail using 100mm wide 450gsm Chopped Strand Glassfibre Mat consolidated in Pre-accelerated Polyester Laminating Resin creating an intrinsically waterproof single layer structure.

- At one square metre intervals create Secret Mobile Negative Wind Lift fixings and laminate in, using part preformed materials ensuring freedom of joint movement, and laminate in. Only appropriate non-corrosive fixings suitable for the roof deck/joists to be used.

- Wet lay not less than one further layer of 450gsm Chopped Strand Glassfibre Mat consolidated in Pre-accelerated Polyester Laminating Resin to the entire roof area with ratios of Resin to Mat all as per manufacturer's recommendations and allow to cure.

- To the newly created construction apply by brush a liberal covering of Pre-accelerated UV stable Polyester Flowcoat (Gelcoat), Colour to be specified, into which (if required) broadcast a layer of 6mm clean limestone chippings (or similar) prior to cure to ensure a good bond.

- NB: A range of other finishes may also be considered which may be applied during or after the Flocoat is cured. Imitation lead roofs can be constructed incorporating roll mops and flashings.
The above specification can be altered to suit the desired strength/appearance, but the basic principles of an independent roof covering able to tolerate thermal and structural movement must be adhered to.

NOTE 1 - Dress & seal flat roof covering over top course of tiles / slates for weathering & prevention of wind uplift.

S	FLAT ROOF OF CROWNED TOP PITCHED ROOF - WARM DECK

CARCASING FROM:-

- 47 x 150mm flat roof joists at 400mm ctrs. spiked to 50 x 100mm wall plates.

- Timber or steel ridge beams as indicated on plans to Structural Engineers design.

- 100mm x 50mm SW water check fillets to roof verges.

- Strut noggin bracing at 2M max. ctrs.

- 1:40 fall 50mm x 50mm s.w. firings at 400mm ctrs.

- Trim around bwk. flue stack using double joists & rafters bolted together to ensure that a gap of 40mm is maintained between face of stack & timbers.

INSULATION:-

- Specialist warm deck insulation board system laid over the joists / firings comprising of 116mm overall thickness composite insulated decking with 110mm celotex pre-bonded to 6mm ply decking. Additional layer of 19mm OSB waterproofed Sterlin board decking ready to accept the felts & a vapour control layer.    Install standard board sizes (2.44 x 1.22M) with nogs under all board joints in strict accordance with the manufacturers instructions.

- NOTE 1: The Contractor may use another specialist warm deck roofing system (equal & approved) provided it is approved by the Building Control Dept. & laid in strict accordance with the manufacturers instructions.

- NOTE 2: Insulation of pitched roof is to be taken up to underside of roof decking to form a continuous thermal barrier & reduce cold bridging.

FOR HIGH PERFORMANCE GRADE USING POLYESTER BASED ROOFING SYSTEMS.

- Torch-on roofing system using Ruberoid polyester felts.

- First layer to be polyester base or glass base torch-on underlay.

- Second layer to be fully bonded polyester base torch on cap sheet.

- 13mm mineral stone chippings on a polymer modified bitumen based adhesive compound.

- 12.5mm foil backed plasterboard ceiling finished with 5mm "Thistle" board finish plaster. Scrim tape all joints.

GRP FLAT ROOF COVERING

- Supplier: ‘Roof Line’ GRP system. (or equal & approved).

- Installer: Must be trained & approved by system manufacturers.

- Construction of GRP Roofs should only carried out in temperatures above 5 degrees and in dry weather.

- Supply and install 19mm treated timber battens fixed to the roof perimeter, fixed 10mm below the roof deck to allow roof expansion and vapour egress.

- Supply, cut to shape and install one layer of 2mm nom FLEX-BASE Specially Prepared Preformed GRP Sheet, forming all upstands, downturns and incorporating all necessary flashing detail, lapping all sheets by at least 30mm to form the base layer.

- Supply and install water-check and or drip detail preformed GRP roof edge trims, secured to the previously fixed 19mm treated timber battens. Face fix trims every 300mm with appropriate colour coded fixings.

- Laminate together all preformed base layer and detail using 100mm wide 450gsm Chopped Strand Glassfibre Mat consolidated in Pre-accelerated Polyester Laminating Resin creating an intrinsically waterproof single layer structure.

- At one square metre intervals create Secret Mobile Negative Wind Lift fixings and laminate in, using part preformed materials ensuring freedom of joint movement, and laminate in. Only appropriate non-corrosive fixings suitable for the roof deck/joists to be used.

- Wet lay not less than one further layer of 450gsm Chopped Strand Glassfibre Mat consolidated in Pre-accelerated Polyester Laminating Resin to the entire roof area with ratios of Resin to Mat all as per manufacturer's recommendations and allow to cure.

- To the newly created construction apply by brush a liberal covering of Pre-accelerated UV stable Polyester Flowcoat (Gelcoat), Colour to be specified, into which (if required) broadcast a layer of 6mm clean limestone chippings (or similar) prior to cure to ensure a good bond.

- NB: A range of other finishes may also be considered which may be applied during or after the Flocoat is cured. Imitation lead roofs can be constructed incorporating roll mops and flashings.
The above specification can be altered to suit the desired strength/appearance, but the basic principles of an independent roof covering able to tolerate thermal and structural movement must be adhered to.

S	LEAD LINED RECESS FLAT ROOF SECTIONS WITHIN NEW PITCHED ROOF TO 	ACCOMMODATE WALL OPENINGS (WINDOWS)

- Description: The new pitched roof will interfere with either the new or existing windows some of which are    identified on the plans.    To accommodate this, sections of the new pitched roof are to be cut away to form recessed areas so that the affected windows remain unaltered.

- Trim out pitched roof as required & indicated on plans.

- For small roof recesses from:-

- 50 x 100mm flat roof joists at 400mm ctrs. Laid to shallow falls spiked to

- 100 x 50mm SW water check fillets to roof verges.

- Strut noggin bracing at 2M max. ctrs.

- 19mm OSB waterproofed decking.

- All roof wall plates to be securely strapped to wall.

- Top wall plates to be fixed to existing walls with 16mm dia expanding bolts & 50mm dia. spread washers at 600mm ctrs.

ROOF COVERING:-

- Code 5 lead lining dressed over top course of tiles / slates.

- 50mm dia. mopstick & laid in accordance with the LSA guidance sheets.

- Nogs under all board joints in strict accordance with the manufacturers instructions.

- Take lead lining up small sides filled in with 19mm OSB waterproofed decking between timber studs.

S	NEW ROOF CONSTRUCTION - PREFORMED ROOF TRUSSES

- Manufacturer / supplier: Preformed roof trusses by Minera Roof Trusses Ltd. (or similar & approved) installed at 600mm ctrs.

- **** degree pitches & in strict accordance with the manufacturers instructions.

- The trusses are to be pressure treated preservative impregnated.    M75 stress graded timber to all Chords.    Manufacture of trusses in accordance with BS. 5628 Part 3 1985.    Refer to the manufacturers designs & calculation sheets for proof of performance.    Trusses supplied are not to be cut under any circumstances,

- Stability bracing & lateral restraint is to be provided to individual truss members as defined in BS. 5268 Part 3 Clause 18.

- 50 x 100mm wall plates securely strapped to wall at 2000mm ctrs. using 38 x 5mm galv. steel straps spanning 600mm of wall.

The design of all prefabricated roof trusses is to be in accordance with the current issues of the following:-

- BS. 5268 Parts 2 & 3 - The structural use of timber.

- BS. CP.3 Chapter V, Part 1 - Dead & imposed loads.

- BS. CP.3 Chapter V, Part 2 - Wind loads.

- BS. 4978 - Structural timber grades.

- BS. 5268 Part V - Preservative treatment of timber.

- Agreement Certificate 20 gauge 80/748.

- Building Regulations.

Cover roof trusses as follows:-

FOR SLATES:-

To roof slopes: Lay slates as follows with spalled edges uppermost:

a) Twice centre nail every slate except very small slates which shall be once head nailed.

b) Lay close jointed with straight horizontal and vertical joints, vertical joints centred over the slate below.

c) Lay close jointed in straight horizontal courses diminishing from eaves to ridge with width of slates selected to centre vertical joints over the slate below.

d) Use thicker slates in lower courses and ensure each slate lays truly flat on the slate below.

e) Use slates of the same thickness in any one course and fix with thinner end uppermost.

f) Maintain bond at ends of courses with slates of extra width.

FOR TILES:-

To roof slopes: Lay tiles as follows:

a) Twice nail every tile in every fifth course.

b) Lay slightly open jointed with straight horizontal and vertical joints, vertical joints centred over the tile below.

c) Maintain bond at ends of courses with width-and-a-half tiles.

DETAILS:-

- Ridges: Ridge coping tiles to match existing roof details.

a) Ensure that the underlay capping strip is in position.

b) Gauge the slating / tiling to ensure that the full lap is maintained in the top course.

c) Bed the specified units in mortar.

d) Fill and gallet exposed ends.

e) Cut mitred joints at hip intersections.

- Hips: Hip copings tiles to match the existing house details.

a) Ensure the additional strip of underlay is in position.

b) Cut slates /tiles to the line of the hip.

c) Bed specified hip fittings in mortar commencing at the lowest point.

d) Fill and gallet exposed ends.

e) Fix hip irons with two galvanized screws.

- For Mitred hips: Form as follows:

a) Ensure the additional strip of underlay is in position.

b) Cut slates / tiles to the line of the hip.

c) Lay width-and-a-half slates in alternate courses/Lay extra wide slates in alternate courses.

d) Fix lead soakers into each course lapped and nailed at head.

- For bonnet hips: Fix as follows:

a) Cut tiles to the line of the hip.

b) Nail the end tile in every course.

c) Commence at foot of hip with exposed end filled and galletted.

d) Lay bonnet tiles to course with the general tiling; nail each unit and bed in mortar.

- Battens: 38mm x 25mm treated s.w. battens at the required gauge.

a) Free from decay or insect attack.

b) Free from any splits and shakes.

c) Wane permissible on one arris only.

d) Knots (or knot holes) less than one third width of any sound surface of occurrence.

e) Treated with preservative: (i) organic solvent; double vacuum or 3 min immersion (ii) Boron diffusion.

f) 22%    Moisture content at time of fixing.

- Securely fix with galvanized nails to every rafter or through counter battens into every rafter as follows:

a) At centres to give the correct slate / tile laps.

b) In long lengths (min length to be supported by at least three rafters with both ends supported).

c) With ends cut square and centred over rafters and nailed without splitting.

d) Packed where necessary to maintain level.

e) Provide additional battens and tilting fillets etc at eaves, ridges and other perimeter features.

f) Provide additional battens if necessary to prevent windlift of horizontal laps in the underlay.

g) Nail penetration into rafters, 50 mm min.

- Nails: Use nails of adequate length and gauge to provide secure fixing as follows:

a) For slates: Aluminium alloy to BS 1202: Part 3/Copper to BS 1202: Part 2/silicon bronze to BS 5534

b) For underlay: Extra large head of aluminium alloy/copper/galvanized steel to BS 1202: Part 1.

c) For battens: Galvanized steel to BS 1202: Part 1 sized to give a secure fixing without splitting the batten.

d) Do not nail through any plumber's metalwork.

- Clips for slates: Aluminium alloy to BS 5534/stainless steel to BS 5534.

- Hip irons: Galvanized steel to BS 5534.

- Mortar: Cement mortar (1:3), coloured to match slate or tile fittings.

a) Use the minimum amount necessary to provide adequate bedding and secure weatherproof fixings.

b) Solidly bed edges and joints of slating / tiling fittings until mortar squeezed out.

c) Immediately strike off surplus mortar and press back with trowel where exposed to form a dense smooth neat finish.

d) Keep mortar off the edge of slates and tiles.

e) Provide a neat finish and reduce the mass of mortar in exposed ends and edges, etc by pushing in tile or slate slips.

- Undercloaks: 6 mm asbestos-free fibre cement board of approved make with the performance standards of asbestos cement to BS 690.    Fix with rougher face uppermost as mortar key.

- Eaves: Lay a double course with undercourse of short slates / tiles head nailed to the bottom batten. Fix with bottom edges in line and projecting over the fascia.

- Verges: Form verges with an upward tilt as follows:

a) Ensure that the underlay extends to the verge line.

b) Lay an undercloak to project 50mm consisting of fibre cement board / a course of slates / a course of plain tiles (but to match the existing house detail) bedded in mortar / fixed with galvanised steel nails.

c) Lay width-and-a-half slates / tiles in alternate courses/ bedded in mortar and finished flush with undercloak.

d) Do not use any cut slates at verges.

- Underfelt: Breathable & untearable roofing felt by Tyvek.    Note - use of this felt eliminates the need to install perimeter & ridge vent venting. Fix as follows:

a) Lay parallel to eaves commencing at eaves and with a slight drape between rafters.

b) Lap 150 mm min at sides and ends with end laps centred over rafters.

c) Nail sufficiently to make secure until battens fixed.

d) Supply & install proprietory eaves carriers dressed under roofing felt, lay over any tilting fillets and extend into gutters. Prevent any sagging which could trap water.

e) Lay a strip over the ridge to overlap general underlay by 150 mm min.

f) Lay 600 mm wide strip at verge below general underlay and extend fully to the eaves line.

g) Lay 600 mm strip at hips over the general underlay and extend fully to the eaves line.

h) Extend up wall at top abutment to suit cover flashing.

i) Upon completion of battening ensure that the roof is proof against adverse weather conditions pending the laying of the slates or tiles.

WEATHERINGS & ABUTMENTS:-

- Soakers: Code 5 lead rolled sheet.

- Flashings: Code 5 lead rolled sheet.

- Valleys - tiles: Special valley tiles over an extra layer of felt.

- Valleys - lead: Form lead valley gutters using 12.5mm marine plywood base; treated softwood tilting fillets to form trough;    cover in Code 5 lead dressed 250mm under tiles & sarking felt;    sharp sand/cement to cut tile edges with waterproofing agent to the mix to seal valley tile / slate verges. Lay an undercloak of fibre cement board fixed with galvanized nails.    Cut tiles / slates to the line of the valley verge to leave    50mm between valley verges.    Lay width-and-a-half tiles in every alternate course bedded in mortar and nailed.

- Open valleys: Form as follows:

a) Ensure the additional strip of underlay is in position under the general underlay before the grp gutter is fixed.

b) Cut slates to line of gutter to leave 100 mm min width between valley verges.

c) Lay width-and-a-half slates /tiles in alternate courses without undercloak or mortar.

- Laced valleys: Form as follows:

a) Ensure the additional strip of underlay is fixed in position.

b) Fix the valley board with feather-edged boards each side over the battening.

c) Fix triangular tilting fillets at base of valley.

d) Continue slate / tile courses upwards in an even curve to the centre of the valley.

e) Form passing courses in valley with double width slates.

- Swept valleys: Form as follows:

a) Ensure the additional strip of underlay is fixed in position.

b) Fix the valley board with feather-edged boards each side over the battening.

c) Cut slates /tiles to correct taper from slates / tiles of sufficient size to maintain specified laps.

d) Lay slates in an even sweep through the valley.

- Abutments at top edges: Ensure that underlay projects above the slating / tiling and finish slating with short course to maintain gauge.

- Abutments at sloping edges: Finish with width-and-a-half slates / tiles in alternate courses. Finish with extra wide slates / tiles in alternate courses and dress in lead soakers to receive flashings.

- Fascias, soffits, barge boarding: treated batten framing to support white upvc’s soffits, fascias & barge boarding.

- Where a roof abuts a new wall: ensure code 5 lead flashings connect to & is dressed under cavity tray dpc’s of wall.

INTERNAL FINISHES:-

- Ceilings: 12.5mm foil backed plasterboard with 5mm plaster skim.

- Loft hatch: Preformed door tray & frame (530mm x 745mm).    Supplier:- www.accesspanels.co.uk (or equal & approved) – formed from 1.0mm and 1.2mm Zintec electro-galvanised mild steel with 25 mm wide picture frame finished in polyester powder coat lightly textured white RAL 9010 20% Gloss.    Hinge & Lock mechanism – Factory fitted removable pivot plates with inserted steel pins located into holes in the frame. Budget or Euro-profile lock case bolted onto the return fold of the door tray with tongue locating into a slot in the frame. Lock access hole covered with white plastic collar and plug or patented ‘Touchflap’.    Draughtstrip – Black expanded polyurethane draughtstrip 3 x 10mm to all four sides. Insulation – Rigid fire resistant insulation block CFC & HCFC Free.    U-Value - 0.35.    Fire Certification - Panels tested by Chiltern International Fire (TRADA) Ltd. Test number Chilt/IF02059 20th May 2002.
Preparation - Prepare a clear trimmed & protected opening in the structure 5mm larger than the panel ordered. Designed to fit between 600mm centre trusses fix cross trimmers spanning between the trusses to form a minimum clear opening of 775 x 553 mm. Line the inner four faces with 10 or 12.5mm thick plasterboard to suit the frame. (Where space is at a premium any suitable insulating material can be used to protect the truss from heat transfer from the metal frame.)
Fitting - Offer up the panel and screw fix using 50 mm long screws through the pre-punched holes to all four sides of the frame. Complete the operation with door in frame if possible. To remove the door from the frame unscrew the right hand pivot pin plate and slide the plate towards the middle of the door. The door can now be slid out of the frame. To replace locate the other pivot pin plate in the frame hole, ensure the centre pin is within its slot then replace the previously removed pivot pin plate ensuring the pin fully locates back into the frame hole. 
Operation - The plastic plug in the door face can be removed with a fingernail. Then use the square drive supplied ‘T’ key to open and close whilst supporting the door.

INSULATIONS:-

- Insulation 1: insulate horizontal ceilings / roof with 200mm Crown Wool between the ceiling joists & 100mm Crown Wool insulation over the joists.

- Insulation 2: For all sloping soffits (scaling ceilings) with breathable felts (new rafters):-

- 1 layer of 100mm Celotex GA3090Z thermal Sheathing fixed between the rafters & another layer of 40mm Celotex GA3040Z underlining fixed to the underside of the rafters.    Tape the joints with Idenden sarking tape.    Cover with 12.5mm foil backed Plasterboard & 5mm skim.

OTHER:-

- Connections: All bolts used in construction of roof to be 12mm with 50mm spread washers & toothed timber connectors - all junctions & connections in roof MUST be bolted (where possible).

- Strapping: All roof wall plates to be securely strapped to wall using 38 x 5mm galv. steel straps at 2000 ctrs. straps to span 600mm of wall.

- Lateral restraint: Gable end walls parallel to rafters & all floor or ceiling joists are to receive lateral support using 38 x 5mm galv. steel straps at 1200 ctrs. straps to span 3 joists/rafters with strut noggins between joists at strap locations.

- Fire breaks: Pack between slating and separating walls with mineral wool. Ensure there is no continuation of underlay through the fire break.

- To resist any horizontal thrusts: imposed from unrestrained rafters that cannot be fixed to parallel joists, the rafter feet are to be tied to perpendicular joists using 38 x 5mm galv. steel twist ties with strut noggins between.    Straps to span 3 joists.

- Ensure roof insulation: abuts with & joins to the wall insulation to form a continuous thermal barrier.

- Loft walkway: Due to insulation felt covering ceiling joists, install 600mm wide x 19mm thk. plywood walk ways through highest section of roofs for safe access for maintenance.

- NOTE 1:- Main Contractor to establish cost of best matching tile available prior to tender & include accordingly rather than their own PC sum.

S	CANOPY & BAY ROOFS - COLD DECK

- GENERAL REQUIREMENTS:-

- Code of practice: Comply with BS 5534 Slating and tiling subject to any qualifications given below.

- Materials: Main Contractor to supply samples of roof covering materials for clients / specifies approval prior to ordering.    Main Contractor to establish cost of best matching tile / slate available prior to tender & include accordingly rather than their own PC sum.

GENERALLY:-

- Pitch:     **    deg. approx. pitch but to match existing.

- Slates / Tiles:     **    tiles or slates to match existing (to be prior agreed with client & Planning Dept.).

STRUCTURAL ROOF CARCASSING:-

- Rafters: 50 x 100mm at 400 ctrs.

- Ceiling Joists: 50 x 100mm ceiling joists at 400 ctrs. With strut noggin bracing at joist mid span.

- Hip rafters: 38 x 150mm hip rafters.

- Wall plates: 50 x 100mm top wall plates securely bolted to wall using M16 expanding bolts & 50mm dia, spread washers at 600mm ctrs.

- Eaves overhang: Soffit projections & fascia heights to match the existing roof details.    Use treated s.w. batten framing.

- Barge boarding projections: Projection from wall face & height of barge boarding to match existing house details. Use treated s.w. batten framing combined with gable ladders.

- Window heads: twin 50mm x 150mm support wall plates over bays supported by 95m x 95mm treated timber posts.

- Corner posts: 100 x 100mm corner posts anchored to bwk. Walls on dpc.

FOR SLATES:-

To roof slopes: Lay slates as follows with spalled edges uppermost:

a) Twice centre nail every slate except very small slates which shall be once head nailed.

b) Lay close jointed with straight horizontal and vertical joints, vertical joints centred over the slate below.

c) Lay close jointed in straight horizontal courses diminishing from eaves to ridge with width of slates selected to centre vertical joints over the slate below.

d) Use thicker slates in lower courses and ensure each slate lays truly flat on the slate below.

e) Use slates of the same thickness in any one course and fix with thinner end uppermost.

f) Maintain bond at ends of courses with slates of extra width.

FOR TILES:-

To roof slopes: Lay tiles as follows:

a) Twice nail every tile in every fifth course.

b) Lay slightly open jointed with straight horizontal and vertical joints, vertical joints centred over the tile below.

c) Maintain bond at ends of courses with width-and-a-half tiles.

DETAILS:-

- Hips: Hip copings tiles to match the existing house details.

a) Ensure the additional strip of underlay is in position.

b) Cut slates /tiles to the line of the hip.

c) Bed specified hip fittings in mortar commencing at the lowest point.

d) Fill and gallet exposed ends.

e) Fix hip irons with two galvanized screws.

- For bonnet hips: Fix as follows:

a) Cut tiles to the line of the hip.

b) Nail the end tile in every course.

c) Commence at foot of hip with exposed end filled and galletted.

d) Lay bonnet tiles to course with the general tiling; nail each unit and bed in mortar.

- Battens: 38mm x 25mm treated s.w. battens at the required gauge.

a) Free from decay or insect attack.

b) Free from any splits and shakes.

c) Wane permissible on one arris only.

d) Knots (or knot holes) less than one third width of any sound surface of occurrence.

e) Treated with preservative: (i) organic solvent; double vacuum or 3 min immersion (ii) Boron diffusion.

f) 22%    Moisture content at time of fixing.

- Securely fix with galvanized nails to every rafter or through counter battens into every rafter as follows:

a) At centres to give the correct slate / tile laps.

b) In long lengths (min length to be supported by at least three rafters with both ends supported).

c) With ends cut square and centred over rafters and nailed without splitting.

d) Packed where necessary to maintain level.

e) Provide additional battens and tilting fillets etc at eaves, ridges and other perimeter features.

f) Provide additional battens if necessary to prevent windlift of horizontal laps in the underlay.

g) Nail penetration into rafters, 50 mm min.

- Nails: Use nails of adequate length and gauge to provide secure fixing as follows:

a) For slates: Aluminium alloy to BS 1202: Part 3/Copper to BS 1202: Part 2/silicon bronze to BS 5534

b) For underlay: Extra large head of aluminium alloy/copper/galvanized steel to BS 1202: Part 1.

c) For battens: Galvanized steel to BS 1202: Part 1 sized to give a secure fixing without splitting the batten.

d) Do not nail through any plumber's metalwork.

- Clips for slates: Aluminium alloy to BS 5534/stainless steel to BS 5534.

- Hip irons: Galvanized steel to BS 5534.

- Mortar: Cement mortar (1:3), coloured to match slate or tile fittings.

a) Use the minimum amount necessary to provide adequate bedding and secure weatherproof fixings.

b) Solidly bed edges and joints of slating / tiling fittings until mortar squeezed out.

c) Immediately strike off surplus mortar and press back with trowel where exposed to form a dense smooth neat finish.

d) Keep mortar off the edge of slates and tiles.

e) Provide a neat finish and reduce the mass of mortar in exposed ends and edges, etc by pushing in tile or slate slips.

- Undercloaks: 6 mm asbestos-free fibre cement board of approved make with the performance standards of asbestos cement to BS 690.    Fix with rougher face uppermost as mortar key.

- Eaves: Lay a double course with undercourse of short slates / tiles head nailed to the bottom batten. Fix with bottom edges in line and projecting over the fascia.

- Verges: Form verges with an upward tilt as follows:

a) Ensure that the underlay extends to the verge line.

b) Lay an undercloak to project 50mm consisting of fibre cement board / a course of slates / a course of plain tiles (but to match the existing house detail) bedded in mortar / fixed with galvanised steel nails.

c) Lay width-and-a-half slates / tiles in alternate courses/ bedded in mortar and finished flush with undercloak.

d) Do not use any cut slates at verges.

- Underfelt: Breathable & untearable roofing felt by Tyvek.    Note - use of this felt eliminates the need to install perimeter & ridge vent venting. Fix as follows:

a) Lay parallel to eaves commencing at eaves and with a slight drape between rafters.

b) Lap 150 mm min at sides and ends with end laps centred over rafters.

c) Nail sufficiently to make secure until battens fixed.

d) Supply & install propriatory eaves carriers dressed under roofing felt, lay over any tilting fillets and extend into gutters. Prevent any sagging which could trap water.

e) Lay a strip over the ridge to overlap general underlay by 150 mm min.

f) Lay 600 mm wide strip at verge below general underlay and extend fully to the eaves line.

g) Lay 600 mm strip at hips over the general underlay and extend fully to the eaves line.

h) Extend up wall at top abutment to suit cover flashing.

i) Upon completion of battening ensure that the roof is proof against adverse weather conditions pending the laying of the slates or tiles.

WEATHERINGS & ABUTMENTS:-

- Soakers: Code 5 lead rolled sheet.

- Flashings: Code 5 lead rolled sheet.

- Fascias, soffits, barge boarding: treated batten framing to support white upvc’s soffits, fascias & barge boarding.

- Where a roof abuts a new wall: ensure code 5 lead flashings connect to & is dressed under cavity tray dpc’s of wall.

INTERNAL FINISHES:-

- Ceilings: 12.5mm foil backed plasterboard with 5mm plaster skim.

INSULATIONS:-

- Insulation 1: insulate horizontal ceilings / roof with 200mm Crown Wool between the ceiling joists & 100mm Crown Wool insulation over the joists.

OTHER:-

- Connections: All bolts used in construction of roof to be 12mm with 50mm spread washers & toothed timber connectors - all junctions & connections in roof MUST be bolted (where possible).

- Strapping: All roof wall plates to be securely strapped to wall using 38 x 5mm galv. steel straps at 2000 ctrs. straps to span 600mm of wall.

- Lateral restraint: Gable end walls parallel to rafters & all floor or ceiling joists are to receive lateral support using 38 x 5mm galv. steel straps at 1200 ctrs. straps to span 3 joists/rafters with strut noggins between joists at strap locations.

- To resist any horizontal thrusts: imposed from unrestrained rafters that cannot be fixed to parallel joists, the rafter feet are to be tied to perpendicular joists using 38 x 5mm galv. steel twist ties with strut noggins between.    Straps to span 3 joists.

CARCASSING:-

- Rafters: 50mm x 100mm rafters at 400 ctrs.

- Ceiling Joists: 50mm x 100mm ceiling joists at 400 ctrs.

- Hip rafters: 38 x 150mm hip rafters.

- Flashing & soakers: Code 5 lead sheet.

- Externals: The soffits are to be framed out with treated s.w. battens for adequate fixings of white upvc soffit & fascia boards.

- 1000g polythene vapour barrier.

- 12.5mm foil backed plasterboard with 5mm plaster skim.

- Insulate roof with 200mm Crown Wool between the ceiling joists & 100mm Crown Wool insulation over the joists.

- All bolts used in construction of roof to be 12mm with 50mm spread washers & toothed timber connectors - all junctions & connections in roof MUST be bolted (where possible).

S	EXTENSION TILED PARAPETS

Generally: Form a pitch tiled roof up-stand parapets around perimeter of new flat roof as generally shown on plans & sections using:-

- GENERAL REQUIREMENTS:-

- Code of practice: Comply with BS 5534 Slating and tiling subject to any qualifications given below.

- Materials: Main Contractor to supply samples of roof covering materials for clients / specifies approval prior to ordering.    Main Contractor to establish cost of best matching tile / slate available prior to tender & include accordingly rather than their own PC sum.

- Workmanship - Before slating / tiling: Preliminary checks: Check that pipes, etc penetrating the roofing, cloaks, roof gutters, firebreaks and all other pre-roofing work by other trades are completed, particularly any work involving disturbance to the structural roof timbers. Check that all tilting fillets and firrings, etc are fixed or available for fixing by the slaters / tilers.

GENERALLY:-

- Pitch:     **    deg. approx. pitch but to match existing.

- Slates / Tiles:     **    tiles or slates to match existing (to be prior agreed with client & Planning Dept.).

STRUCTURAL ROOF CARCASSING:-

- Rafters: 50 x 100mm at 400 ctrs.

- Ceiling Joists: as per flat roof joists.

- Ridge Board: 38 x 225mm.

- Hip rafters: 38 x 175mm hip rafters.

- Valley layer spiking boards: 38 x 200mm valley layer spiking boards.

- Wall plates: 50 x 100mm wall plates securely strapped to wall at 2000mm ctrs. using 38 x 5mm galv. steel straps spanning 600mm of wall.

- Eaves overhang: Soffit projections & fascia heights to match the existing roof details.    Use treated s.w. batten framing.

- Barge boarding projections: Projection from wall face & height of barge boarding to match existing house details. Use treated s.w. batten framing combined with gable ladders.

- Rear upstand: 50 x 100mm vertical studs @ 400 ctrs. on 50 x 100mm head & sole plates.    19mm wpb exterior grade ply fixed to studs. Vertical Breathable & untearable roofing felt by Tyvek. - Note - use of this felt eliminates the need to install perimeter & ridge vent venting.    38 x 25mm vertical battens at 400mm ctrs.    38 x 25mm treated s.w. battens at required guage for plain tile hanging. Plain tile hanging using matching tiles.

- Inner roof drainage: form weirs/gutters through pitched parapet roof voids for draining of surface water from flat roof behind.    Form 250 x 150mm high weirs from 18mm exterior grade plywood lined with Code 5 lead lapped & sealed to tiles & flat roof surface.

FOR SLATES:-

To roof slopes: Lay slates as follows with spalled edges uppermost:

a) Twice centre nail every slate except very small slates which shall be once head nailed.

b) Lay close jointed with straight horizontal and vertical joints, vertical joints centred over the slate below.

c) Lay close jointed in straight horizontal courses diminishing from eaves to ridge with width of slates selected to centre vertical joints over the slate below.

d) Use thicker slates in lower courses and ensure each slate lays truly flat on the slate below.

e) Use slates of the same thickness in any one course and fix with thinner end uppermost.

f) Maintain bond at ends of courses with slates of extra width.

FOR TILES:-

To roof slopes: Lay tiles as follows:

a) Twice nail every tile in every fifth course.

b) Lay slightly open jointed with straight horizontal and vertical joints, vertical joints centred over the tile below.

c) Maintain bond at ends of courses with width-and-a-half tiles.

DETAILS:-

- Ridges: Ridge coping tiles to match existing roof details.

a) Ensure that the underlay capping strip is in position.

b) Gauge the slating / tiling to ensure that the full lap is maintained in the top course.

c) Bed the specified units in mortar.

d) Fill and gallet exposed ends.

e) Cut mitred joints at hip intersections.

- Hips: Hip copings tiles to match the existing house details.

a) Ensure the additional strip of underlay is in position.

b) Cut slates /tiles to the line of the hip.

c) Bed specified hip fittings in mortar commencing at the lowest point.

d) Fill and gallet exposed ends.

e) Fix hip irons with two galvanized screws.

- For Mitred hips: Form as follows:

a) Ensure the additional strip of underlay is in position.

b) Cut slates / tiles to the line of the hip.

c) Lay width-and-a-half slates in alternate courses/Lay extra wide slates in alternate courses.

d) Fix lead soakers into each course lapped and nailed at head.

- For bonnet hips: Fix as follows:

a) Cut tiles to the line of the hip.

b) Nail the end tile in every course.

c) Commence at foot of hip with exposed end filled and galletted.

d) Lay bonnet tiles to course with the general tiling; nail each unit and bed in mortar.

- Battens: 38mm x 25mm treated s.w. battens at the required gauge.

a) Free from decay or insect attack.

b) Free from any splits and shakes.

c) Wane permissible on one arris only.

d) Knots (or knot holes) less than one third width of any sound surface of occurrence.

e) Treated with preservative: (i) organic solvent; double vacuum or 3 min immersion (ii) Boron diffusion.

f) 22%    Moisture content at time of fixing.

- Securely fix with galvanized nails to every rafter or through counter battens into every rafter as follows:

a) At centres to give the correct slate / tile laps.

b) In long lengths (min length to be supported by at least three rafters with both ends supported).

c) With ends cut square and centred over rafters and nailed without splitting.

d) Packed where necessary to maintain level.

e) Provide additional battens and tilting fillets etc at eaves, ridges and other perimeter features.

f) Provide additional battens if necessary to prevent windlift of horizontal laps in the underlay.

g) Nail penetration into rafters, 50 mm min.

- Nails: Use nails of adequate length and gauge to provide secure fixing as follows:

a) For slates: Aluminium alloy to BS 1202: Part 3/Copper to BS 1202: Part 2/silicon bronze to BS 5534

b) For underlay: Extra large head of aluminium alloy/copper/galvanized steel to BS 1202: Part 1.

c) For battens: Galvanized steel to BS 1202: Part 1 sized to give a secure fixing without splitting the batten.

d) Do not nail through any plumber's metalwork.

- Clips for slates: Aluminium alloy to BS 5534/stainless steel to BS 5534.

- Hip irons: Galvanized steel to BS 5534.

- Mortar: Cement mortar (1:3), coloured to match slate or tile fittings.

a) Use the minimum amount necessary to provide adequate bedding and secure weatherproof fixings.

b) Solidly bed edges and joints of slating / tiling fittings until mortar squeezed out.

c) Immediately strike off surplus mortar and press back with trowel where exposed to form a dense smooth neat finish.

d) Keep mortar off the edge of slates and tiles.

e) Provide a neat finish and reduce the mass of mortar in exposed ends and edges, etc by pushing in tile or slate slips.

- Undercloaks: 6 mm asbestos-free fibre cement board of approved make with the performance standards of asbestos cement to BS 690.    Fix with rougher face uppermost as mortar key.

- Eaves: Lay a double course with undercourse of short slates / tiles head nailed to the bottom batten. Fix with bottom edges in line and projecting over the fascia.

- Verges: Form verges with an upward tilt as follows:

a) Ensure that the underlay extends to the verge line.

b) Lay an undercloak to project 50mm consisting of fibre cement board / a course of slates / a course of plain tiles (but to match the existing house detail) bedded in mortar / fixed with galvanised steel nails.

c) Lay width-and-a-half slates / tiles in alternate courses/ bedded in mortar and finished flush with undercloak.

d) Do not use any cut slates at verges.

- Underfelt: Breathable & untearable roofing felt by Tyvek.    Note - use of this felt eliminates the need to install perimeter & ridge vent venting. Fix as follows:

a) Lay parallel to eaves commencing at eaves and with a slight drape between rafters.

b) Lap 150 mm min at sides and ends with end laps centred over rafters.

c) Nail sufficiently to make secure until battens fixed.

d) Supply & install propriatory eaves carriers dressed under roofing felt, lay over any tilting fillets and extend into gutters. Prevent any sagging which could trap water.

e) Lay a strip over the ridge to overlap general underlay by 150 mm min.

f) Lay 600 mm wide strip at verge below general underlay and extend fully to the eaves line.

g) Lay 600 mm strip at hips over the general underlay and extend fully to the eaves line.

h) Extend up wall at top abutment to suit cover flashing.

i) Upon completion of battening ensure that the roof is proof against adverse weather conditions pending the laying of the slates or tiles.

WEATHERINGS & ABUTMENTS:-

- Soakers: Code 5 lead rolled sheet.

- Flashings: Code 5 lead rolled sheet.

- Valleys - tiles: Special valley tiles over an extra layer of felt.

- Valleys - lead: Form lead valley gutters using 12.5mm marine plywood base; treated softwood tilting fillets to form trough;    cover in Code 5 lead dressed 250mm under tiles & sarking felt;    sharp sand/cement to cut tile edges with waterproofing agent to the mix to seal valley tile / slate verges. Lay an undercloak of fibre cement board fixed with galvanized nails.    Cut tiles / slates to the line of the valley verge to leave    50mm between valley verges.    Lay width-and-a-half tiles in every alternate course bedded in mortar and nailed.

- Open valleys: Form as follows:

a) Ensure the additional strip of underlay is in position under the general underlay before the grp gutter is fixed.

b) Cut slates to line of gutter to leave 100 mm min width between valley verges.

c) Lay width-and-a-half slates /tiles in alternate courses without undercloak or mortar.

- Laced valleys: Form as follows:

a) Ensure the additional strip of underlay is fixed in position.

b) Fix the valley board with feather-edged boards each side over the battening.

c) Fix triangular tilting fillets at base of valley.

d) Continue slate / tile courses upwards in an even curve to the centre of the valley.

e) Form passing courses in valley with double width slates.

- Swept valleys: Form as follows:

a) Ensure the additional strip of underlay is fixed in position.

b) Fix the valley board with feather-edged boards each side over the battening.

c) Cut slates /tiles to correct taper from slates / tiles of sufficient size to maintain specified laps.

d) Lay slates in an even sweep through the valley.

- Abutments at top edges: Ensure that underlay projects above the slating / tiling and finish slating with short course to maintain gauge.

- Abutments at sloping edges: Finish with width-and-a-half slates / tiles in alternate courses. Finish with extra wide slates / tiles in alternate courses and dress in lead soakers to receive flashings.

- Fascias, soffits, barge boarding: treated batten framing to support white upvc’s soffits, fascias & barge boarding.

- Where a roof abuts a new wall: ensure code 5 lead flashings connect to & is dressed under cavity tray dpc’s of wall.

INTERNAL FINISHES:-

- Ceilings: 12.5mm foil backed plasterboard with 5mm plaster skim.

INSULATIONS:-

- Insulation 1: insulate horizontal ceilings / roof with 200mm Crown Wool between the ceiling joists & 100mm Crown Wool insulation over the joists.

OTHER:-

- Connections: All bolts used in construction of roof to be 12mm with 50mm spread washers & toothed timber connectors - all junctions & connections in roof MUST be bolted (where possible).

- Strapping: All roof wall plates to be securely strapped to wall using 38 x 5mm galv. steel straps at 2000 ctrs. straps to span 600mm of wall.

- Lateral restraint: Gable end walls parallel to rafters & all floor or ceiling joists are to receive lateral support using 38 x 5mm galv. steel straps at 1200 ctrs. straps to span 3 joists/rafters with strut noggins between joists at strap locations.

- To resist any horizontal thrusts: imposed from unrestrained rafters that cannot be fixed to parallel joists, the rafter feet are to be tied to perpendicular joists using 38 x 5mm galv. steel twist ties with strut noggins between.    Straps to span 3 joists.

- Ensure roof insulation: abuts with & joins to the flat roof insulation to form a continuous thermal barrier.

S	NEW DORMER PITCHED ROOF(S) - COLD DECK

- Generally: This section only describes the main structural elements of the new dormer roofs.    Refer to the main pitched roof specification section for all other relevant details.

STRUCTURAL ROOF CARCASSING:-

- Rafters: 50 x 100mm at 400mm ctrs.

- Ceiling Joists: 50 x 100mm at 400m ctrs.

- Ridge Board: 38 x 225mm.

- Hip rafters: 38 x 175mm hip rafters.

- Valley layer spiking boards: 38 x 200mm valley layer spiking boards.

- Wall plates: 50 x 100mm wall plates securely strapped to wall at 2000mm ctrs. using 38 x 5mm galv. steel straps spanning 600mm of wall.

- Eaves overhang: Soffit projections & fascia heights to match the existing roof details.    Use treated s.w. batten framing.

- Barge boarding projections: Projection from wall face & height of barge boarding to match existing house details. Use treated s.w. batten framing combined with gable ladders.

S	DORMER FLAT ROOF(S) - WARM DECK

- CARCASSING FROM:-

- Joists: 50mm x 150mm flat roof joists at 400mm ctrs. spiked to 50 x 100mm wall plates.

- Water check fillets: 100mm x 50mm SW water check fillets to roof verges.

- Strut noggin bracing: at 2M max. ctrs.

- Firings: 1:60 fall 50mm x 50mm s.w. firings at 400mm ctrs.

- Roof wall plates: 50 x 100mm treated s.w. timber to be securely strapped to wall using 38 x 5mm galv. steel straps at 2000 ctrs. straps to span 600mm of wall.

- Lateral wall support: Walls parallel to joists are to receive lateral support using 38 x 5mm galv. steel straps at 1200 ctrs. straps to span 3 joists/rafters with strut noggins between joists at strap locations.

- INSULATION:-

- Specialist warm deck insulation board system laid over the joists / firings comprising of 116mm overall thickness composite insulated decking with 110mm celotex pre-bonded to 6mm ply decking. Additional layer of 19mm OSB waterproofed Sterlin board decking ready to accept the felts & a vapour control layer.    Install standard board sizes (2.44 x 1.22M) with nogs under all board joints in strict accordance with the manufacturers instructions.

- NOTE: The Contractor may use another specialist warm deck roofing system (equal & approved) provided it is approved by the Building Control Dept. & laid in strict accordance with the manufacturers instructions.

- EXTERNAL DETAILS:-

- White upvc soffits & fascia boards fixed to treated s.w. battened out framing.    Soffit board to incorporate 25mm continuous air vents with fly screens for cross ventilation of roof void.    Ensure that a clear 50mm air gap is maintained above the insulation for adequate cross venting of the roof void.

- Lap felt 450mm min. up under tiles & sarking felt & flash with Code 5 lead.

- FOR HIGH PERFORMANCE GRADE USING POLYESTER BASED ROOFING SYSTEMS.

- Torch-on roofing system using Ruberoid polyester felts.

- First layer to be polyester base or glass base torch-on underlay.

- Second layer to be fully bonded polyester base torch on cap sheet.

- 13mm mineral stone chippings on a polymer modified bitumen based adhesive compound.

- Code 5 lead flashings dressed 150mm up wall into bwk. at junction of flat roof & house wall.

- White upvc soffits & fascia boards fixed to treated s.w. battened out framing.

- GRP FLAT ROOF COVERING

- Supplier: ‘Roof Line’ GRP system. (or equal & approved).

- Installer: Must be trained & approved by system manufacturers.

- Construction of GRP Roofs should only carried out in temperatures above 5 degrees and in dry weather.

- Supply and install 19mm treated timber battens fixed to the roof perimeter, fixed 10mm below the roof deck to allow roof expansion and vapour egress.

- Supply, cut to shape and install one layer of 2mm nom FLEX-BASE Specially Prepared Preformed GRP Sheet, forming all upstands, downturns and incorporating all necessary flashing detail, lapping all sheets by at least 30mm to form the base layer.

- Supply and install water-check and or drip detail preformed GRP roof edge trims, secured to the previously fixed 19mm treated timber battens. Face fix trims every 300mm with appropriate colour coded fixings.

- Laminate together all preformed base layer and detail using 100mm wide 450gsm Chopped Strand Glassfibre Mat consolidated in Pre-accelerated Polyester Laminating Resin creating an intrinsically waterproof single layer structure.

- At one square metre intervals create Secret Mobile Negative Wind Lift fixings and laminate in, using part preformed materials ensuring freedom of joint movement, and laminate in. Only appropriate non-corrosive fixings suitable for the roof deck/joists to be used.

- Wet lay not less than one further layer of 450gsm Chopped Strand Glassfibre Mat consolidated in Pre-accelerated Polyester Laminating Resin to the entire roof area with ratios of Resin to Mat all as per manufacturer's recommendations and allow to cure.

- To the newly created construction apply by brush a liberal covering of Pre-accelerated UV stable Polyester Flowcoat (Gelcoat), Colour to be specified, into which (if required) broadcast a layer of 6mm clean limestone chippings (or similar) prior to cure to ensure a good bond.

- NB: A range of other finishes may also be considered which may be applied during or after the Flocoat is cured. Imitation lead roofs can be constructed incorporating roll mops and flashings.
The above specification can be altered to suit the desired strength/appearance, but the basic principles of an independent roof covering able to tolerate thermal and structural movement must be adhered to.

- FOR LEAD LINED ROOFS:-

- Code 5 lead external lining dressed over 50mm dia. treated s.w. moptick rails installed in accordance with the LSA guidance.    Apply patenation oil as soon as the lead has been applied.

- Dress lead 450mm up under main roof covering.

- Dress lead over verges & eaves & seal with lead clips at required LSA guidance centres to prevent wind uplift.

- Lay lead sheeting on Tyvek untearable & breathable roofing felt.

- INTERNAL FINISH:-

- 12.5mm foil backed plasterboard ceiling finished with 5mm "Thistle" board finish plaster.

- NOTE: External walls & cavity insulation is to be taken up to underside of roof decking to form a continuous thermal barrier & reduce cold bridging.

S	NEW DORMER WALLS & CHEEKS - TILE HUNG

CARCASSING:-

- 100 x 50mm treated vertical studs at 400 ctrs.

- 100 x 50mm treated horizontal noggins at 600 ctrs.

- 100 x 50mm wall plates.

- Twin 50 x 150mm timbers for head lintel over windows.

- Double up rafters under flannel walls & bolt together.

- 9mm marine anti-racking plywood applied internally & externally.

INSULATIONS:-

- Insulate with 100mm Crown Dritherm insulation board between studs & lined internally with 40mm Celotex insulation board. Tape joints with Idenden tape.    Cover with 1000g polythene vapour barrier & 12.5mm foil backed plasterboard & 5mm plaster skim.

- Code 5 lead flashings & soakers.

TILE HANGING WITH PLAIN TILES

- Generally: Fix breathable & untearable underlay felt by Tyvek, 38 x 25mm treated s.w. battens and Plain tiling in accordance with the relevant clauses applicable to roof slopes and as hereafter stated.

- Tile fixing: Twice nail every tile.

- Battens: 38 x 25mm treated s.w. battens securely fix to the background in a manner that will enable the tiles to be nailed without any springing, using galvanized nails or screws as appropriate. Use wall plugs and packings where necessary.

- Bottom course: Fix as stated for eaves to roof slopes. Provide additional battens if necessary.

- Vertical abutments: Form with width-and-a-half tiles in alternate courses and dress in Code 5 lead soakers to receive stepped flashings.

- Vertical angles: Form with width-and-a-half tiles cut and mitred and dress in Code 5 lead soakers.

- Vertical angles: Form with handed angle tiles.

- Verge abutments: Splay cut width-and-a-half tiles and fit closely up to the verge at the end of each course.

- Verge abutments: Turn each course up at the end to meet the verge at a right angle by cutting two width-and-a-half tiles to a suitable taper.

- Exposed edges at windows etc: Finish tiling to straight vertical edge with width-and-a-half tiles in alternate courses and fill with mortar finished flush.

FLASHINGS AND SOAKERS, ETC

- Supply: Ensure that Code 5 lead soakers etc are available in good time for the roofing work.

- Flashings and weatherings: Ensure that the coverings are fixed as soon as the relevant tiling is completed.

S	NEW DORMER WALLS & CHEEKS - SLATE HUNG

CARCASSING:-

- 100 x 50mm terated vertical studs at 400 ctrs.

- 100 x 50mm treated horizontal noggins at 600 ctrs.

- 100 x 50mm wall plates.

- Twin 50 x 150mm timbers for head lintel over windows.

- Double up rafters under flannel walls & bolt together.

- 9mm marine anti-racking plywood applied internally & externally.

INSULATIONS:-

- Insulate with 100mm Crown Dritherm insulation board between studs & lined internally with 40mm Celotex insulation board. Tape joints with Idenden tape.    Cover with 1000g polythene vapour barrier & 12.5mm foil backed plasterboard & 5mm plaster skim.

- Code 5 lead flashings & soakers.

VERTICAL SLATING

Generally: Fix breathable & untearable underlay felt by Tyvek, 38 x 25mm treated s.w. battens and slating in accordance with the relevant clauses applicable to roof slopes and as hereafter stated.

Battens: Securely fix to the background in a manner that will enable the slates to be nailed without any springing, using galvanized nails or screws as appropriate. Use wall plugs and packings where necessary.

Bottom course: Fix as stated for eaves to roof slopes. Provide additional battens if necessary.

Vertical abutments: Form with width-and-a-half slates in alternate courses and dress in lead soakers to receive flashings.

Vertical angles: Form with wide slates cut and mitred; dress in lead soakers nailed at top edge.

Form with sawn slate handed angles.

Verge abutments 1: Splay-cut large slates and fit closely up to the verge at the end of each course.

Verge abutments 2: Turn each course up at the end to meet the verge at a right angle by cutting two larger slates to a suitable taper.

FLASHINGS AND SOAKERS, ETC

- Supply: Ensure that Code 5 lead soakers etc are available in good time for the roofing work.

- Flashings and weatherings: Ensure that the coverings are fixed as soon as the relevant tiling is completed.

S	DORMER WALLS & CHEEKS - LEAD LINED

CARCASSING:-

- 100 x 50mm terated vertical studs at 400 ctrs.

- 100 x 50mm treated horizontal noggins at 600 ctrs.

- 100 x 50mm wall plates.

- Twin 50 x 150mm timbers for head lintel over windows.

- Double up rafters under flannel walls & bolt together.

- 9mm marine anti-racking plywood applied internally & externally.

INSULATIONS:-

- Insulate with 100mm Crown Dritherm insulation board between studs & lined internally with 40mm Celotex insulation board. Tape joints with Idenden tape.    Cover with 1000g polythene vapour barrier & 12.5mm foil backed plasterboard & 5mm plaster skim.

- Code 5 lead flashings & soakers.

LEAD COVERING:-

- Code 5 lead external lining installed in accordance with the LSA guidance.    Apply patenation oil as soon as the lead has been applied.

- Tyvek untearable & breathable roofing felt.

- Lap lead under sarking felt of main roof.

- Code 5 lead flashings & soakers.

S	ALTERATIONS TO EXISTING ROOF FOR NEW ROOM

- Generally: Prior to removing any structural timbers (struts, purlins, ties bracing etc.) temporary & adequate propping must be installed to prevent sag or overstressing of the existing rafters & joists.

- Remove existing purlins & props & provide temporary support to existing rafters & hips.

- Install new 50 x 125mm rafters (Grade C16) alongside existing undersized rafters & bolt members together at 800mm ctrs.

- Trim out for new dormer windows as denoted on plans using double rafters bolted together.

- Trim out for new velux roof lights as denoted on plans using double rafters bolted together.

- NEW FLOOR:-

- Lay new 50 x 200mm floor joists (Grade C16) at 400 ctrs. alongside existing ceiling joists.

- Packing may be required to clear plaster laths of existing ceilings.

- Herringbone brace at 2000 ctrs. using 38 x 50mm struts.

- Cover with 22mm T & G moisture resistant flooring grade chipboard decking.

- Use double joists bolted together for stair well trimming.

- NEW DWARF STUD PERIMETER WALLS:-

- 50 x 100mm studs & noggins at 400mm ctrs.

- 50 x 100mm sole plate.

- Insulate with 100mm Crown Dritherm insulation board between studs & lined internally with 40mm Celotex insulation board. Tape joints with Idenden tape.

- Cover with 12.5mm foil backed plasterboard & 5mm plaster skim.

- CEILINGS:-

- Install new 50 x 100 mm high level ceiling joists/collars at 400mm ctrs. bolted to existing rafters. Insulate small roof void formed with 200mm Crown Wool between the ceiling joists & 100mm Crown Wool insulation over the joists. 1000g polythene dpm.

- 12.5mm foil backed plasterboard with 5mm skim coat.

- INSULATIONS:-

- For all sloping soffits, (existing rafters utilising existing non breathable felts) a special insulation board is to be used to achieve a 50mm min. air gap above the insulation for cross ventilation of the roof void.    This will be 1 layer of 50mm Celotex thermal Sheathing fixed between the rafters to allow a 50mm vented air gap over & another layer of 100mm Celotex underlining fixed to the underside of the rafters.    Tape the joints with Idenden sarking tape.    Cover with 1000g polythene dpm & 12.5mm foil backed Plasterboard & 5mm skim.

- CROSS VENTILATION:-

- Install special ‘retro fit’ upvc 75mm dia. soffit vents @ 300 ctrs. in existing main roof for roof void cross ventilation.

- Remove the existing ridge coping tiles & install new ridge vent ridge coping tiles with an in-built 5mm continuous vent to aid cross venting of the roof void.

S	NEW VELUX WINDOW(S)

- Install the following Velux Roof lights in roof slopes as indicated on plans / elevations.

- 1 new GGL1 Velux window to ???.

- Size = 			780 x 980mm.

- 1 new GGL1 Velux window to ???.

- Size = 			780 x 980mm.

- Glazing =		Protex Star (Laminated with a Low ‘E’ inner leaf sealed unit

double glazing achieving a ‘U’ Value of 1.5.

- Flashing kits = 		type U (for tiles) or type L (for slates).

- Trimmers to form opening = 	50 x 100mm.

- Double up & spike together rafters for each side of Velux.

- Install windows in strict accordance with the manufacturers instructions.

- Insulated studwork shafts: Form studwork shaft through roof from trimmed opening for Velux window down to trimmed opening in ceiling joists using CLS 45 x 95mm studs & noggins at 400mm ctrs.    Fill stud voids with 100mm Dritherm cavity wall batts.    Line internally with 40mm Celotex prior to installing 12.5mm foil backed plasterboard & 5mm plaster skim.

S	NEW LEADWORK

- Material: All flashings, soakers, weatherings, roofing & cladding etc. required for the works must be in milled lead produced to BS 1178 (1982).

- The size & thickness of the sheet for the particular application together with the installation details is to be in strict accordance with the Pocket Guides No. 1 & 2 issued by the Lead Sheet Association (LSA) to maintain a high degree of materials & workmanship.

- Guides: If the Main Contractor or his bonafide subcontractors are not familiar with these guides then contact The Specifier for the details.

- NOTE: Flashband or zinc materials will not be permitted.

S	SCHEDULE OF GALVANISED STEEL FIXINGS, STRAPS TIES ETC.

- REQUIREMENT: To ensure that all structural elements are tied together the following galv. steel fixings are to be used to ensure lateral support & bracing to all structures.

- WALL PLATES:- Fixed to wall at 2000mm ctrs using 38 x 5mm galv. steel straps spanning 600mm of wall.

- FLAT ROOF JOIST ENDS:- Fixed to wall plate using galv. steel framing anchor or skew nails.

- TRUSSED RAFTERS:- Fixed to wall plate using galv. steel framing anchors or galv. steel twist ties anchored to wall.

- GABLE END WALLS:- Tied to rafters at 2000mm ctrs using 38 x 5mm galv. steel ties hooked into cavity & with nogs between rafters & packer between wall & rafter. Straps to span 3 rafters.

- FLANK WALLS AT ROOF LEVEL:- Tied to ceiling joists at 2000mm ctrs using 38 x 5mm galv. steel ties hooked into cavity & built into wall with nogs between joists & packer between wall & joist. Straps to span 3 joists.

- FLANK WALLS AT FIRST FLOOR LEVEL:- Tied to floor joists at 2000mm ctrs using 38 x 5mm galv. steel ties hooked into cavity & built into wall with nogs between joists & packer between wall & joist. Straps to span 3 joists.    For joist ends built into wall restraint type galv. steel joist hangers are to be used or 38 x 5mm galv. steel straps fixed to wall & joist at 2000mm ctrs.

S	NEW STAIRS

- Generally: Supply & install a hardwood timber stair set to be manufactured by a specialist supplier.    The design of the balustrades, hand rails etc. shall be chosen by client.    Prior to manufacture, the stair well is to be site measured.    The new stair set shall 'generally' conform to the following:-

- No. steps = 14 total;

- Landings = 800 x 800mm clear.

- Rise = 187mm, 2618mm overall (to be checked).

- Going = 230mm, 2990mm overall.

- Pitch = 42 deg. max.

- 2 R+G = 604

- 90 x 90mm Newel posts.

- 50mm diam. hand rails.

- Balustrades to be 900mm high on stairs (following line of nosing) & 900mm high on landings with a 100mm max gap between members.

- Stair width = 900mm overall, & 800mm clear.

- 50mm min. going to tapers.

- 2M clear headroom to stairs & landings.

- 800mm clear landings.

- 12.5mm plasterboard & skim to soffit stairs.

- PC Sum: Main Contractor to allow a PC sum of £3,000.00 for the supply only of the new stair set (complete) to clients choosing.    Installation is deemed to be included elsewhere within the Main Contractors quotation. Main Contractor to furnish the client with a selection of stair set design options & supplier options for their consideration.

S	NEW STAIR SET MANUFACTURE NOTE

- Precautions: It is sometimes difficult to assess clearance heights above & below new stair sets for practicality & conformity to Building Regulations due to variations in each stair manufacturers construction methods.    Therefore, it is a requirement that the main contractors specialist stair manufacturer is to visit site & take his own constructional dimensions of the proposed stair well, all wall & door locations, floor to floor heights & note any other possible obstructions to confirm that the new stair case will comply with the Building Regulations & will not obstruct any existing doorways, access routes etc.    After his dimensional survey & larger scale drawing, should a discrepancy arise with that indicated on the Specifier plans, Specifier is to be informed immediately for advise prior to manufacture of the new stair set.

S	NEW INTERNAL JOINERY

- Generally: For ease of builders pricing, it is suggested that all joinery items are to match the existing property & to the satisfaction of the client.    Therefore……..

- New internal doors to match existing.

- New door ironmongery to match existing.

- New softwood skirtings & architraving to match existing.

- The client may require an upgrade to these joinery items.    However, for the benefit of forming a tender price basis the contractor is to allow for matching items & any upgrade of fittings will be treated an extra cost item.

- Supply & fit all joinery items using best practice methods.

Or…..

As specified here:-

- NOTE:- The final manufacturer/supplier of the new internal/external doors, architraving & skirting etc. has yet to be established.    Main Contractor is to allow the following PC sums for the purposes of tendering:-

- Internal doors (complete with linings & ironmongery) = 	£200.00 per door. (supply only)

- 65mm Architrave - MDF O gee =			£        0.50 per metre. (supply only)

- 150mm Skirting - MDF O gee = 			£        1.00 per metre. (supply only)

- Installation & fit of the joinery is deemed to be included elsewhere within the main contractors quotation.

S	FIRE PRECAUTIONS

- Garage ceiling: to be fire stopped at ceiling to wall junctions using compressed mineral wool.    Garage ceiling to be 2 layers of 12.5mm plasterboard with lapped & sealed joints with 5mm plaster skim coat.

- Fire doors: Doors marked as 'half hr fire doors' indicated on the plans are to be 'complete assemblies' (meaning finished door assembly ready fitted into frame/lining & fitted with all ironmongery) of half hr. fire resistance to comply with BS8214:1990.    The installation of the fire door shall be vindicated by test to BS476 Part 22 or expert assessment based upon the appropriate & relevant tests.    Fire Doors identified by the suffix 'S' should be fitted with an edge seal proven satisfactory when tested in accordance with BS476:Part 31.1 leakage rate not to exceed 3M3m/hour when tested at 25Pa.    A self closing devise must also be fitted.    Alternatively the Contractor may install the fire door set system from components on site provided it is completed in strict accordance with the fire test certificate that accompanies the fire door including all ironmongery specified for that system.

- Garage floor: to be at least 100mm below floor to main dwelling.

- Stair soffits: Provide 12.5mm plasterboard with a 7mm plaster cover to soffits of new stairs.

- Smoke detection system: Supply & install new comprehensive smoke detection system to BS 5446 Part 1 being mains operated with battery back up & interlinked (one set off all alarms sound).    Install alarm/smoke/heat sensors as denoted on plans.

- Door self closers: Provide self-closing devices to all doors that are off the hallway & landing areas (the protected shaft).

- New second floor door: to be half hr. fire resistant (30/30) & fitted with a self- closing device.    Fire door to have intumescent edge seals.

- Secondary means of escape - windows: All first floor windows in a new dwelling, or bedroom extensions or rooms formed as ‘inner rooms’ must have opening lights/sashes large enough to escape through in the event    of being trapped within a room by fire.    To meet this requirement all such windows must have a minimum unobstructed openable area of at least 0.33M2 & not to be less than 450mm wide & 750mm high.    Also the bottom of the openable area should not exceed 1100mm above the floor or be less than 800mm off the floor.

- Secondary means of escape - roof lights: Roof dormer windows are to be set so that the distance from the dormer window cill to the eaves line does not exceed 1.7M.

- Recessed down lighters: Maintain fire protection over all recessed down lighters by installing proprietary 30 minute fire proof cloth hoods over down lighter that also allows the fittings NOT to overheat & to give a degree of sound insulation as well. Fire protection covers/hoods to conform to BS 476 Pt 23.    Alternatively, use light fittings that already comply as a complete unit.

- Door upgrades: All new & existing internal doors forming access onto stair enclosure (hall & landings) must be fitted with an intumescent strip (side & head of door or lining) to provide FD20 rated first resistance doors.

- Fire fighting appliance access: Front driveway / access to highway must not be less than 3.1M for access provision for fire fighting appliances.

S	NEW KITCHEN FURNITURE

- Description: The line of the kitchen units shown on the plans is only illustrative at this stage.    The client will be arranging for a specialist kitchen designer to layout the kitchen units at a later stage.    The client may also like to seek advice from the main contractor regarding suppliers & recommendations.

- PC sum: The contractor is to allow within his quotation a p.c. sum of £15,000.00 for the supply only of new kitchen units chosen by client.

- The contractor will then obtain the units selected by client and install accordingly.    The collection & installation of the kitchen will be included within the main contractors quotation.

- PC sum: The contractor is to allow for 5M2 of coloured ceramic tiling to clients choosing.    To the same requirements as above, the contractor is to allow a p.c. sum of £400.00 for the supply only of these tiles.    Fixing & grouting will be included within the main contractors quotation.

Or:-

Description: The line of the kitchen units shown on the plans is only illustrative at this stage.    The client will be arranging for a specialist kitchen designer to layout, supply & install the kitchen units at a later stage.

The Main Contractor is to allow attendance only onto this nominated sub-contractor to ensure that the following is completed:-

- Waste pipe / soil pipe spurs are located in the required locations for the kitchen fitter / supplier to install their own waste plumbing.

- Domestic hot water & Mains cold water spurs are located in the required locations for the kitchen fitter / supplier for their own hot & cold water plumbing.

- Electrical spurs are located in the correct locations for the kitchen fitter / supplier to install their own electrical fittings.

- Any other minor & reasonable builders work required by the kitchen fitter / supplier that may be necessary for the efficient installation of the new kitchen & utility.

- The clients kitchen fitter / supplier will also be responsible for all wall tiling.

S	NEW BATHROOM, SHOWER ROOM, EN-SUITES & WC.'S.

- PC sum: The contractor is to allow within his quotation a p.c. sum of £1,500.00 for the supply only of all new sanitary appliances & equipment which will be later chosen by client.

- PC sum: The contractor is to allow for 5M2 of coloured ceramic tiling to clients choosing.    The contractor is to allow a p.c. sum of £500.00 for the supply only of these tiles.    Fixing & grouting will be included within the main contractors quotation.

- The contractor will then obtain the units selected by client and install accordingly.    The collection & installation will be included within the main contractors quotation.

S	NEW RAIN-WATER GOODS

- Supplier: All rain-water goods to be by Osma with all necessary stopped ends, outlets, fixing brackets etc.    laid in accordance with BS5955, BS8301 & diagram 9 of Section 2 of Part H1 of the Building Regulations.    Should this be at variance with matching in with the existing then the supplier / manufacturer of the rain water goods is to be amended to a more suitable supplier. Any cost variations will be adjusted within the final contract price.

DETAIL:-

- 100mm dia. black half round UPVC guttering.

- 63mm dia. black upvc. rain-water down pipes.

- 100mm dia. upvc surface water drains bedded & surrounded in 150mm pea shingle (1:60 falls).

- New rubble filled soak-aways min. 5M away from buildings & capped with a thin layer of concrete or polythene sheet.

- For deep or high capacity soak-aways hollow perforated conc. rings are to be used with a conc. roof cover & manhole lid.

- Provide rodding access points at all changes in directions.

- Where cover to pipes is less than 600mm protect with a concrete paving slab in accordance with item A16 & diagram A4 of Section 2 of Part H1 of the Building Regulations.

- Capacity of soak-aways to be based upon a 12mm rainfall for the drained area & subject to a on site porosity test viewed & approved by the Building Inspector.

S	NEW RAIN-WATER (FOR DORMERS)

As generally called for on main roof.

- 100mm dia. black half round upvc guttering.

- 63mm dia. black upvc. rain-water down pipe discharging onto main roof.

- Existing or new surface water drainage from main roof.

S	DRAINAGE WARNING NOTE

- The existing or new drain runs or inspections chambers indicated on these plans are only indicative & preliminary at this stage.    Not all drain runs & chambers could be established.    Any new drain runs or inspection chambers shown may need to be adjusted during construction to suit exact locations of the existing drains when the ground works can be opened up & existing drains exposed. Any cost variations will be adjusted within the final contract price.

S	NEW DRAINS

- None proposed:    Subject to excavations for the foundations, it is anticipated that there are no new drains or adjustment to the existing drains.    Should this be incorrect then any new or replacement drains will be an agreed extra cost.

ALTERATION WORKS TO THE EXISTING DRAINAGE SYSTEM:-

- The existing inspection chamber within extension is to be raised using 215mm semi-engineering brickwork in cement & sand mortar 1:3.

- Install a new double seal screwed down cover & frame to finish flush with the new screed.

- Cover to be heavy duty galv. steel with a recessed lid to take floor screed.

- The existing drains under new extension are to be protected with a 450 x 150mm conc. slab cast over granular infill of pipe surround.

- Where existing drains pass through new foundations/footings, a bridge is to be formed by using pre-cast conc. lintels to ensure that no building load is placed onto the existing drains.

- Pack around drain with a compressible material (polystyrene) as it passes through wall prior to backfilling to allow for settlement.

- NOTE: All existing drains that are to be built over must be pressure tested by air or water prior to covering with the floor construction or earth & inspected by the Building Inspector.    If tests fail permitted limits then the drains are to be repaired or renewed & tested again until a pass is achieved.    If existing drains need replacing due to failing pressure tests then this will be at extra cost to the client.

S	NEW DRAINS (CLAY)

- Generally: Drains to be laid & constructed in accordance with Building Regulations H1 1985, BS. 6297 & BS. 8301, Codes of Practice for Building Drainage), & to LA. Approval & a Test Certificate of air tightness to be provided upon completion.

- Re-route drains as shown on plans.

- New underground foul drain runs and branch drains to be constructed with 100mm dia.    'Supersleve' vitrified clayware drain-pipes with polypropylene connectors (or equal & approved).

- Drains to be laid to falls of approximately 1:40 & bedded & surrounded in 150mm pea shingle.

- Drains that pass under building are to be protected with a 450 x 150mm concrete slab cover.

- Where new or existing drains pass through brickwork footings or foundations, a pre-cast conc. lintel is to be provided over to form a bridge.

- Pack around drain with 75mm polystyrene batts as drain passes through footing wall or conc. foundation.

- All drain joints are to remain flexible.

- NOTE: All new or existing drains that are to be built over must be pressure tested by air or water prior to covering with the floor construction or earth & inspected by the Building Inspector.    If tests fail permitted limits then the drains are to be repaired or renewed & tested again until a pass is achieved.

S	NEW INSPECTION CHAMBER / MANHOLES (BRICKWORK)

- New inspection chambers or manholes to be constructed in 215mm semi-engineering brickwork in cement & sand mortar 1:3 on 150mm concrete base unless otherwise specified.

- Internal chamber dimensions of 750 x 700mm with 110mm diam.

- vitrified clayware h.r. channel and bends with steep concrete benching.

- Bitumen coated cast iron cover and frame set in cement mortar 1:3.

- For manholes exceeding 1000mm deep, internal chamber dimensions to be minimum of 1200 x 750mm with step irons built into chamber walls.

- 150mm concrete cover slab.

- Bitumen coated cast iron cover & frame set in cement mortar.

- Internal chambers: Install a new double seal screwed down cover & frame to finish flush with the new screed.

- Cover to be heavy duty galv. steel with a recessed lid to take floor screed.

SCHEDULE OF MINIMUM INTERNAL SIZES FOR MANHOLES

- 0.6M or less depth to invert =	190mm dia.

- 1.0M or less depth to invert =	450mm x 450mm

- 1.0M & greater depth to invert =	1200mm x 750mm.

S	NEW DRAINS (UPVC)

- Generally: Drains to be laid & constructed in accordance with Building Regulations H1 1985, BS. 6297 & BS. 8301, Codes of Practice for Building Drainage), & to LA. Approval & a test certificate of air tightness to be provided upon completion.

- New drains & associated equipment to be by OsmaDrain.

- Sealed back inlet gullies (BIG) as shown on drawing set on a concrete base.

- New underground foul drain runs and branch drains to be constructed with 100mm dia. upvc OsmaDrain-pipes with ring seal joints.

- Drains to be laid to falls of approximately 1:40 & bedded & surrounded in 150mm pea shingle prior to backfilling.

- Drains that pass under building are to be protected with a 450 x 100mm deep conc. slab over pea shingle.

- Where drains pass through brickwork footings or foundations, a pre-cast conc. lintel is to be provided over to form a bridge.

- Pack around drain with granular fill as drain passes through footing wall or conc. foundation to allow for building settlement.

- Should any drains cross vehicular accesses, then the drains are to be protected with a 450 x 100mm deep conc. slab over the pea shingle prior to backfilling.

- NOTE: All new or existing drains that are to be built over must be pressure tested by air or water prior to covering with the floor construction or earth & inspected by the Building Inspector.    If tests fail permitted limits then the drains are to be repaired or renewed & tested again until a pass is achieved.

S	NEW INSPECTION CHAMBERS / MANHOLES (UPVC)

- New inspection chambers to be preformed upvc (polypropylene) by OsmaDrain installed in strict accordance with the manufacturers instructions.

- All chambers to be cut & adapted on site to suit required invert depths & all unused inlets are to be blanked off using special plugs.

- Bitumen coated cast iron covers & frames (Class C or B depending upon loadings) set on a 150mm min. concrete cover ring around top of inspection shaft.

- Manholes to be backfilled using granular fill.

- Install a new double seal screwed down cover & frame to finish flush with the new screed.    Cover to be heavy duty galv. steel with a recessed lid to take floor screed.

SCHEDULE OF MINIMUM INTERNAL SIZES FOR MANHOLES

- 0.6M or less depth to invert =	190mm dia.

- 1.0M or less depth to invert =	450mm x 450mm

- 1.0M & greater depth to invert =	1200mm x 750mm.

S305	NEW WASTE PLUMBING (ABOVE GROUND DRAINAGE)

- Manufacturer: 'OsmaSoil' & 'OsmaSeal' upvc waste-pipes & fittings (or equal & approved) to BS.4514:1978 & installed to BS.5572:1978 & Section 1 of Part H1 of the Building Regulations. (or equal & approved).

- SVP: 100mm dia. soil & vent pipes with balloon grating terminal & 'Varipitch' pipe flashing.

- SVP terminations: All svp's to open air are to terminate 900mm above the highest opening into dwelling that is within 3M.

- Access: Bolted access plates to be provided on soil at WC. connection & at low level.

- Waste-pipes & fittings to sinks & sanitaryware as follows:-

- Sinks -				76mm deep seal sink trap - 40mm dia. waste-pipe.

- Washing machines & dishwashers -      	40mm dia. waste stand pipe with a 76mm deep seal trap.

- Baths & showers -			76mm deep seal trap - 40mm dia. waste.

- Basins & bidets -			76mm bottle trap - 40mm dia. waste.

- WC. pan -			'Multikwik' adaptor to soil stack or drain.

- Clearing eyes to be provided on waste-pipe runs at all change of directions.

- Waste pipe runs exceeding 3M from appliance to svp/big is to be increased to 50mm dia. & the traps are to be anti-vac traps.

- Stub stacks: For svp’s denoted as stub-stacks - This will be a svp terminating within the room with a Dergo air inlet valve to be fitted at a height above the flood level point of the adjacent appliance.    Encase stub stack with vented casing for neat finish & detailing.

S	NEW MINI SEWAGE PUMPING STATION

- Background: Initial investigations reveal that natural fall drainage cannot be installed for the new works that will fall back to the main sewer (or other treatment system).

- Main Contrcator to supply & install a new mini foul sewer pumping station as a complete package from ITT Flygt Pumps Ltd., Colwick, Nottingham, NG4 2AN. Tel. 0602 400111. (or equal & approved).

The package is to include:-

- High density polythene pump chamber;    Flygt submersible pump;    Flygt ENH10 level control system;    discharge pipework & valves within chamber;    light pedestrian access cover;    invert pipework;    control panel;    extra:- 100M of 50mm diam. polythene discharge pipework with all necessary connectors etc.

- The contractor is to prior liase with Flygt Pumps Ltd. to ensure that the correct pump & system is selected.

- The contractor is to provide an electrical supply to the unit.

- Back fill around chamber using sand, shingle or other non cohesive material.    If a high water table is suspected, then the chamber is to be fully encased in concrete.

- Final connection to main sewer manhole is to be inspected & supervised by    an Officer from the Drainage Dept. or Building Inspector of the Local Authority.

S	NEW HEATING & PLUMBING - EXTENDING THE EXISTING SYSTEM

GENERALLY:-

- Any new heating or alteration to the existing system to be designed by the main contractors Heating & Plumbing Engineer.    This specification is for guidance only (outline of requirements - performance specification).    The existing heating system is to be prior inspected by the Heating Engineer & any recommendations for improvements or safety issues is to be given to the client & incorporated during the works.

RADIATORS:-

- Install new radiators (by Stelrad) as shown on drg. & plumb into the existing central heating system.

- Provide all necessary radiator valves (Thermostatic control & one being Lockshield to each radiator).

- New radiators are to be sized to provide an internal environment temp. of 21 deg. C when there is an outside ambient of -5 deg. C.

- Install radiators in locations as denoted on plans.    Main contractor to reconfirm all radiator locations with client direct prior to installation.

HEATING BY UNDERFLOOR PIPES - GROUND FLOOR:-

- Design & supplier: Under-floor heating throughout ground floor of extension (except garage) using specialist plastic pipe buried within the screed of the ground floor.    Proprietary system has yet to be established but a proposal from the main contractors Heating Engineer is to be presented to the client or his design agent for discussion, assessment & approval purposes PRIOR TO INSTALLATION.

- Manifold: This will require a specialist manifold thermostatically controlled for a reduced temperature serving the ground floor from the existing boiler which is designed for a higher temperature serving the new & existing radiators.

- Two temperature circuits: As two heating temperature circuits are required (one for low temp. under floor & one for higher temp. panel radiators) recommendations & advice from the main contractors heating engineer will again be required.

- Suggested under-floor heating zones (individually controlled) to be suggested by Main Contractors heating Engineer.    Zone temperature control will be required for each area via room thermostats or other control device to be agreed.

DOMESTIC HOT & COLD WATER SUPPLY:-

- Provide domestic hot & cold water plumbing to new sink, washing machine, dishwasher, basin, WC., shower, bath, bidet etc. or as shown on drawing.

- Plumb into existing system.

- Insulate all pipes in exposed positions.

HOT WATER STORAGE:-

- Hot water storage: Provide & install a new 190 litre copper indirect hot water cylinder which is to be pre-insulated & to incorporate a 15 amp immersion heater.

- Reconnect to existing domestic hot water system.

Or:-

HOT WATER STORAGE:-

- Convert the existing domestic hot & cold water supply to a mains water fed pressure system.    Install pressure reducing valves, safety devices as required & if necessary.

- Remove existing feed & expansion tank & cold water storage tank from existing roof void.    Remove & cap off all redundant pipe work.

- Hot water storage: Supply & install a replacement hot water cylinder which is to be a Megaflow by Heatrae Sadia.    Size to be 190 litres fitted with electric immersion heater.    Cylinder to be pre-insulated & installed with expansion vessel in accordance with the manufacturers instructions.

HEAT SOURCE:-

- Retain the existing boiler.    It is anticipated that the extra heat demand for the extension works will be accommodated by the existing boiler. As this boiler can be easily replaced in the future, the client will run the extended heating system for one whole season to see how it performs.

- Convert: The client intends to convert system to mains fed water pressure & the main contractors Heating & Plumbing Engineer is to check that the existing boiler is suitable for conversion.

- Servicing: After the heating & plumbing works have been completed, the existing boiler is to be serviced by a registered ‘Gas Safe’ engineer.

SAFETY DEVICES:-

- A temperature relief valve will be located directly on the hot water storage vessel & that the stored water temperature will not exceed 100 deg. C.

- The discharge pipe from the tundish should be of suitable metal & its size should be at least one pipe size larger than the outlet pipe on the safety devices.    The pipe should be laid to a continuous fall & be no longer than 9M unless the bore is increased in size.    The pipe should discharge in a visible but safe place such as a gully where there is no risk of    contact with the hot water persons in the vicinity of the building.

- The non-self resetting thermal cut out should be connected to the direct or indirect heat source or primary heat flow control device in accordance with the current Regulations of the IEE.

- The Hot Water Service (HWS) cylinder is to be fitted with a three-way mid position valve on the primary circuit and controlled by a cylinder thermostat wired into the heating / hot water control system. This is to be set up to give priority to hot water recovery at periods of maximum HWS demand.    It is envisaged that the control thermostat will be linked by a radio signal to the central control unit / programmer, local to the boiler.

S	NEW HEATING & PLUMBING - MAINS PRESSURE - COMPLETE NEW SYSTEM

GENERALLY:-

- Requirement: This is to be completed by a ‘Gas Safe’ registered heating & plumbing engineer under the direct control of the main contractor.    The heating & plumbing design is to be completed by the Main Contractors Heating & Plumbing Engineer & this section below is simply a performance brief or outline schedule of requirements of what is required.

- Options: The Heating & plumbing Engineer is to discuss issues & options with the client or his Agent & a scheme design / materials proposal is to be prepared by the engineer & presented to the client or his agent for approval.

- All distribution pipework to be in copper with joints & fittings to BS. Digest 80 & BS. 5572.      All pipework to be sized in accordance with the CIBS guide.

- Options: The heating system is to utilise the existing pipes & radiators where possible & considered acceptable by the Heating Engineer.    Extend into the existing heating system with new pipes & radiators.

- Design: Any new heating installation to be designed by the main contractors heating & plumbing engineer.      This performance specification is for guidance only.

- Install new radiators (by Stelrad) as shown on drg. & plumb into new central heating system.

- Provide all necessary radiator valves (1 Thermostatic control & 1 Lock shield with drain off to each radiator).

- Sizing: New radiators & under floor heating are to be sized to provide an internal environment temp. of 21 deg. C when there is an outside ambient of - 4 deg. C.

- Design: The domestic heating & plumbing design is to be completed by main contractors Heating & Plumbing Engineer & to be ‘Gas Safe’ registered.

NEW WALL MOUNTED RADIATORS:-

- Manufacturer: Install new radiators (by Stelrad) as shown on drg. & plumb into the existing central heating system.    Radiators to be supplied to site ready painted in gloss white.

- Controls: Provide all necessary radiators, with two valves (One angle pattern TRV; One angle pattern lockshield valve, with integral drain off port), per radiator.

- New radiators are to be sized to provide an internal environment temp. of 21 deg. C when there is an outside ambient of - 5 deg. C.

- Install radiators in locations as denoted on plans.    Main contractor to reconfirm all radiator locations with client direct prior to installation.

HEATING BY UNDERFLOOR PIPES - GROUND FLOOR:-

- Design & supplier: Under-floor heating throughout ground floor of extension (except garage) using specialist plastic pipe buried within the screed of the ground floor.    Proprietary system has yet to be established but a proposal from the main contractors Heating Engineer is to be presented to the client or his design agent for discussion, assessment & approval purposes PRIOR TO INSTALLATION.

- Manifold: This will require a specialist manifold thermostatically controlled for a reduced temperature serving the ground floor from the existing boiler which is designed for a higher temperature serving the new & existing radiators.

- Two temperature circuits: As two heating temperature circuits are required (one for low temp. under floor & one for higher temp. panel radiators) recommendations & advice from the main contractors heating engineer will again be required.

- Suggested under-floor heating zones (individually controlled) to be suggested by Main Contractors heating Engineer.    Zone temperature control will be required for each area via room thermostats or other control device to be agreed.

NEW HEAT SOURCE:-

- Remove the existing gas boiler & install a new high efficiency condensing balanced flue gas boiler (SEDBUK approved & rated 90% efficiency) in new utility as shown.

- New heat source: The new heat source will be a wall mounted gas fired high efficiency condensing balanced flue boiler to be sized by Main Contractors Heating Engineer but is to heat the entire house & the domestic hot water with a 25% overcapacity for future requirements.

- The installation is to be carried out by a Gas Safe Registered gas fitter under the direct control of the main contractor.

- Flue: The balanced flue is to be taken through the wall as denoted on plans.

- Balanced flue is to be a min. of 300mm away from any wall opening.

- Flue must not project towards boundary if less than 2.5M away.

- Mains pressure: This will be a mains pressurised boiler complete with expansion vessel.

CONTROLS:-

- The heating system is to incorporate a central thermostat positioned in the hallway & a 7 day programmable timer controlling the heating & domestic hot water requirements.    This is to be positioned in the kitchen.

DOMESTIC HOT & COLD WATER PLUMBING:-

- Locations: Provide domestic hot & cold water plumbing to new sink, washing machine, dishwasher, basin, WC., shower, bath, bidet etc. or as shown on drawing.

- Plumb into existing system.

- Outside taps: Supply & install two outside taps as shown on plans - one at rear & one in garage.

- Insulate all pipes in exposed positions.

HOT WATER STORAGE:-

- Supply & install a replacement hot water cylinder which is to be a Megaflow by Heatrae Sadia.    Size to be 190 litres fitted with electric immersion heater.    Cylinder to be pre-insulated & installed with expansion vessel in accordance with the manufacturers instructions.

- Form new hot water cylinder cupboard in location as indicated on plans made from studwork partitioning. Form slatted pine shelves above tank for airing.

SAFETY DEVICES:-

- A temperature relief valve will be located directly on the hot water storage vessel & that the stored water temperature will not exceed 100 deg. C.

- The discharge pipe from the tundish should be of suitable metal & its size should be at least one pipe size larger than the outlet pipe on the safety devices.    The pipe should be laid to a continuous fall & be no longer than 9M unless the bore is increased in size.    The pipe should discharge in a visible but safe place such as a gully where there is no risk of    contact with the hot water persons in the vicinity of the building.

- The non-self resetting thermal cut out should be connected to the direct or indirect heat source or primary heat flow control device in accordance with the current Regulations of the IEE.

- The Hot Water Service (HWS) cylinder is to be fitted with a three-way mid position valve on the primary circuit and controlled by a cylinder thermostat wired into the heating / hot water control system. This is to be set up to give priority to hot water recovery at periods of maximum HWS demand.    It is envisaged that the control thermostat will be linked by a radio signal to the central control unit / programmer, local to the boiler.

S	OIL TANK RELOCATION & SUPPLY PIPING

- Requirements: Main Contractor to relocate existing oil storage tank to new location within rear garden onto a new concrete base.

Tank to be sited:

- Form a solid base, level and at least 42mm thick, that extends a minimum of 300mm around the footprint of the tank

- 1800mm away from non-fire rated eves of a building

- 1800mm away from openings (such as doors or windows)

- 1800mm away from any appliance flue terminals

- 1800mm away from a non-fire rated building or structure (ie: garden shed)

- 760mm away from non fire rated boundaries (ie: wooden fence)

- If it is not possible to meet these requirements, a fire rated barrier with at least 30 minutes protection should be provided, extending 300mm higher and wider than each applicable face of the tank.

In order to avoid requiring formal Planning Permission:-

- Capacity of tank not to be greater than 3500L capacity

- Higher than 3m above ground level

- Nearer to the public highway than the closest applicable corner of the property.

- Oil design: The domestic plumbing design is to be completed by main contractors fully qualified Heating & Plumbing Engineer & is a registered installer with the Oil Firing Technical Association (OFTEC).    OFTEC members are approved to certify that their work meets building regulations.

S	EXISTING BOILER COMBUSTION AIR

- Requirement: If the extension covers the existing combustion ventilation provision, the main contractor must install a new 215 x 215mm terracotta through air brick at low level in external wall to maintain combustion air to existing open vented boiler.    Make good all structures.

S	HEATING CERTIFICATION & OCCUPIER USE

- The heating system is to be calibrated & fully commissioned & a certificate of compliance submitted to the Council before occupation.    A full set of heating system operating & maintenance instructions must also be provided to the occupier.

S	NEW ELECTRIC’S

- Requirement: In order to comply with Approved Document ‘P’ of the Building Regulations, the Electrical Contractor must belong to a qualifying body recognised on the Governments ‘Competent Persons Scheme’ so that the new electrical works can be self certified in order for compliance.    The competent electrician will have to belong to government approved scheme of which a number have already been given approval.    These are NICEIC, ELECSA Ltd, British Standards Institution & Zurich Certification.    A certificate of compliance will be required from the Electrical Contractor to be provided to the Building Control Dept. & the client immediately after completion of the electrical works (not longer than 30 days after completion).

PROPOSED WORKS:-

- Install new electrical fittings as shown on drawing.

- Allow for all necessary wiring, trunking, conduit etc. to suitable ring mains (determined by main contractors electrical engineer).

- Final locations of fittings to clients choosing.

- All wiring to be concealed & chased into walls & to be in accordance with the latest edition of the IEE Regs.

- One light fitting in 4 must be a low energy fitting as denoted on plans (fixed lighting that ONLY takes lamps having a luminous efficiency of 40 lumens per circuit watt).

- PC Sum: Main Contractor to allow a PC sum of £500.00 for the supply only of new light fittings to clients choosing.    Installation of these lights is deemed to be included within the Main Contractors quotation.

ANCILLARY WIRING & FITTINGS:-

- Install coaxial cable for TV    & Radio within walls as work proceeds from TV/Radio socket plates within rooms as indicated on plans into roof void.    Leave 5M tails within the roof void so that they can be connected to a central loft aerial with a powered splitter (supplied & installed by others).

- Smoke alarm / detection system: Supply & install a hard wired comprehensive smoke alarm system to be installed by the Main Contractors Electrical Engineer during the works. To comply with BS 5446 Part 1.

- Telephone cabling: Install multi-core approved telephone cabling within walls as work proceeds from external service duct to telephone sockets with rooms as denoted on plans.

- External electrical sockets: Install a weather proof external double socket as indicated on plans.

- External lighting: Install external lighting circuits for new external lamps as shown on plans.

- NOTE 1 - Electrical switches & sockets must be located on the wall between a height of 450mm & 12000mm above the floor.

S	METERS & CONSUMER UNITS

GAS:-

- Install a new gas meter box in a convenient location built into wall & cavity as work proceeds. Contact Gas Board to arrange for diversion of incoming gas supply & new gas meter within new gas meter box.

ELECTRIC:-

- Install a new electric meter box in a convenient location built into wall & cavity as work proceeds.    Supply & install an electrical master switch,    fuse unit & consumer unit having mini RCCB circuit breakers within 2M of the electric meter box.    Contact Electricity Board to arrange for diversion of incoming gas supply & new electric meter within new electric meter box.

MAINS WATER:-

- Locate the existing water mains & cap off for a    temporary supply for water during the works.    Make final connection when the works are completed.    Contact Water Board to arrange for diversion of incoming water mains & new water meter within new water meter box.

- All costs associated with connecting to the electricity & water supply (temporary & permanent) is to be included by the Main Contractor.

- PC Sum: As these service board costs are unknown at this stage the main contractor to allow a PC sum of £2,000.00 for all works & fees that will be required by any or all of the above statutory Authorities.

S	ADJUSTMENT OF EXISTING SERVICES

- Services adjustments: It is likely that some of the existing walls to be removed or adjusted will have electrical, heating    or plumbing services attached that will need to be relocated or capped off as the case may be to the clients requirements.    These items have not been indicated on the drawings for clarity but the Contractor is to satisfy himself as to the extent of these adjustment works deemed to be included by inspecting the site prior to tendering.

S	FLOOR LEVELS

- All new finished floor levels: are to line through with the existing finished floor levels without the need for steps or gradients unless otherwise indicated on plans.    Should this not be possible The Specifier & the client is to be informed for alternative assessments.

S	NEW BIFOLD SIDE DOOR SET - WHITE POWDER COATED ALUMINIUM

- All new external doors are to be certified to BS PAS24-1: 1999 (Doors of enhanced security).

- All new locks to BS 3621: 2004

- PC Sum: Main contractor to allow a PC sum of £6,000.00 for the supply & installation of the new bifold door set located as indicated on plans.

- Material to be powder coated aluminium, multi-chamber, 20mm thermal break frames & sealed unit double glazed.    Doors to be by specialist manufacturer under the main contractors contract.    Level threshold base plate (no up-stand for threshold) . 8000mm2 or 4000mm2 adjustable trickle ventilators built into frame as required under a separate section of the Specification.

- Supplier: Main Contractor to furnish client with 2 alternative suppliers for clients consideration.    Client may source their own option for the bifold door set as a nominated supplier.

GLAZING:-

- Sealed unit double glazing with 16mm air gap, argon filled, with the inner leaf being Low ’E’ glass.    Frames will be coated aluminium with thermal break seals.

- Provide a mastic seal joint at window frame to wall junctions.

NOTE 1: New doors are to be ordered after the formation of the opening when the opening can be site measured by the final door supplier.

NOTE 2:    Do not install the doors until all lintels or beams have been fully loaded by the structure above due to deflection & the tight tolerances required by the sliding tracking systems.

S	NEW WINDOWS & EXTERNAL DOORS - HARDWOOD

- All new windows are to be certified to BS 7950 (Windows of enhanced security).

- All new external doors are to be certified to BS PAS24-1: 1999 (Doors of enhanced security).

- All new locks to BS 3621: 2004

- All new windows to be standard timber casement by Boulton & Paul.

- Window style to be 'Sovereign' hardwood made from selected dark red Meranti preservative treated.

- All windows to come complete with two coat satin finish, weather stripped, 32mm rebates, glazing beads for sealed unit double glazing, rust proofed hinges, 140mm cills, 8000mm2 or 4000mm2 adjustable trickle ventilators built into frame as required under a separate section of the Specification, & brass plated locking handles & window stays.

- 25mm thk. solid hardwood window boards to all windows with bull nose projections.

GLAZING:-

- Sealed unit double glazing with 12mm air gap, argon filled, with the inner leaf being Low ’E’ glass.

- Glazing to bathrooms, WC'S etc. to be obscure glazed using a Pilkington 'Cotswold' pattern glass. (sealed unit double glazed).

- Provide a brown mastic seal joint at window frame to wall junctions.

NOTE 1: All windows must have a minimum    opening area of 5% of the room floor areas.    Refer to elevation drawings for opening sashes.

NOTE 2: All new windows are to be ordered after the formation of the openings when the openings can be site measured by the window supplier.

NOTE 3:    Do not install the windows or doors until all lintels or beams have been fully loaded by the structure above due to deflection.

S	NEW WINDOWS & EXTERNAL DOORS - PRE-TREATED SOFTWOOD

All new windows are to be certified to BS 7950 (Windows of enhanced security).

All new external doors are to be certified to BS PAS24-1: 1999 (Doors of enhanced security).

All new locks to BS 3621: 2004

All new windows to be standard timber casement by Boulton & Paul.

Window style to be as indicated on elevations made from pressure impregnated preservative treated softwood.

All windows to come complete with    weather stripping, 32mm rebates, glazing beads for sealed unit double glazing, rust proofed hinges, 140mm cills, 8000mm2 or 4000mm2 adjustable trickle ventilators built into frame as required under a separate section of the Specification, & brass plated locking handles & window stays.

GLAZING:-

Sealed unit double glazing with 12mm air gap, argon filled, with the inner leaf being Low ’E’ glass.

Glazing to bathrooms, WC'S etc. to be obscure glazed using a Pilkington 'Cotswold' pattern glass. (sealed unit double glazed).

25mm thk. MDF window boards to all windows with bull nose projections.

Provide a mastic seal joint at window frame to wall junctions.

NOTE 1: All windows must have a minimum    opening area of 5% of the room floor areas.    Refer to elevation drawings for opening sashes.

NOTE 2: All new windows are to be ordered after the formation of the openings when the openings can be site measured by the window supplier.

NOTE 3:    Do not install the windows or doors until all lintels or beams have been fully loaded by the structure above due to deflection.

S	NEW WINDOWS & EXTERNAL DOORS - CUSTOM MADE PRE-TREATED SOFTWOOD

- All new windows are to be certified to BS 7950 (Windows of enhanced security).

- All new external doors are to be certified to BS PAS24-1: 1999 (Doors of enhanced security).

- All new locks to BS 3621: 2004

- All new windows to be custom made by a specialist joinery firm under main contractors contract.    Windows to be specially made to fit resultant openings in a sash arrangement as indicated on plans.

- Each 3 panel window is to have two opening casements.

- Window frames to be made from top quality pre-treated softwood stained black to match existing.

GLAZING:-

- Sealed unit double glazing with 12mm air gap, argon filled, with the inner leaf being Low ’E’ glass.

- Glazing to bathrooms, WC'S etc. to be obscure glazed using a Pilkington 'Cotswold' pattern glass. (sealed unit double glazed).

- All windows to come complete with, weather stripping, rebates, glazing beads, rust proofed hinges, 140mm cills, 8000mm2 or 4000mm2 adjustable trickle ventilators built into frame as required under a separate section of the Specification,    & locking handles & window stays to match existing.

- 25mm thk. MDF window boards to all windows with bull nose projections.

- Provide a mastic seal joint at window frame to wall/post junctions (matching colour of frames).

NOTE 1: All windows must have a minimum    opening area of 5% of the room floor areas.    Refer to elevation drawings for opening sashes.

NOTE 2: All new windows are to be ordered after the formation of the openings when the openings can be site measured by the window supplier.

NOTE 3:    Do not install the windows or doors until all lintels or beams have been fully loaded by the structure above due to deflection.

S	NEW WINDOWS & EXTERNAL DOORS - WHITE POWDER COATED ALUMINIUM

- All new windows are to be certified to BS 7950 (Windows of enhanced security).

- All new external doors are to be certified to BS PAS24-1: 1999 (Doors of enhanced security).

- All new locks to BS 3621: 2004

- All new windows to be aluminium, multi-chamber, 20mm thermal break frames & sealed unit double glazed.    Windows to be by specialist manufacturer under the main contractors contract.    New windows are to match the existing as far as possible.    140mm upvc cills, 8000mm2 or 4000mm2 adjustable trickle ventilators built into frame as required under a separate section of the Specification.

- 25mm thk. MDF window boards to all windows with bull nose projections.

GLAZING:-

- Sealed unit double glazing with 16mm air gap, argon filled, with the inner leaf being Low ’E’ glass.    Frames will be coated aluminium with thermal break seals.

- Glazing to side windows & wc. to be obscure glazed using a Pilkington 'Cotswold' pattern glass    (sealed unit double glazed).

- Provide a mastic seal joint at window frame to wall junctions.

NOTE 1: All windows must have a minimum    opening area of 5% of the room floor areas.    Refer to elevation drawings for opening sashes.

NOTE 2: All new windows are to be ordered after the formation of the openings when the openings can be site measured by the window supplier.

NOTE 3:    Do not install the windows or doors until all lintels or beams have been fully loaded by the structure above due to deflection.

NOTE 4:    Main Contractor to furnish client with proposed window sample for clients approval prior to ordering.

S	NEW WINDOWS & EXTERNAL DOORS - WHITE UPVC

- All new windows are to be certified to BS 7950 (Windows of enhanced security).

- All new external doors are to be certified to BS PAS24-1: 1999 (Doors of enhanced security).

- All new locks to BS 3621: 2004

- All new windows to be white upvc multi-chamber frames & sealed unit double glazed.    Windows to be by specialist manufacturer under the main contractors contract.    New windows are to match the existing if possible.    140mm white upvc cills, adjustable 8000mm2 or 4000mm2 adjustable trickle ventilators built into frame as required under a separate section of the Specification.

- 25mm thk. MDF window boards to all windows with bull nose projections.

GLAZING:-

- Sealed unit double glazing with 12mm air gap, argon filled, with the inner leaf being Low ’E’ glass.    Frames will be either pressure impregnated timber or PVCu multi-chamber. (refer to relevant section in this spec.)

- Glazing to side windows & wc. to be obscure glazed using a Pilkington 'Cotswold' pattern glass    (sealed unit double glazed).

- Provide a white mastic seal joint at window frame to wall junctions.

NOTE 1: All windows must have a minimum    opening area of 5% of the room floor areas.    Refer to elevation drawings for opening sashes.

NOTE 2: All new windows are to be ordered after the formation of the openings when the openings can be site measured by the window supplier.

NOTE 3:    Do not install the windows or doors until all lintels or beams have been fully loaded by the structure above due to deflection.

S	NEW FRONT DOOR SET

All new external doors are to be certified to BS PAS24-1: 1999 (Doors of enhanced security).

All new locks to BS 3621: 2004

PC Sum: Contractor to allow a PC sum of £1,500.00 for the supply only of a new front door set    made from hardwood timber & double glazing to a design & style of clients choosing.    Installation & fitting is deemed to be included within the main contractors quotation.

NOTE 1: Front door set to have a level threshold with weather seal & with no up stand (15mm max) & have a 1:20 external ramped access to the front door.

NOTE 2: New front door set is to be ordered after the formation of the openings when the openings can be site measured by the door set supplier.    Should the door set selected by the client be a pre-formed item then the opening is to be made to the set size plus 10mm tolerance.

NOTE 3:    Do not install the doors until all lintels or beams have been fully loaded by the structure above due to deflection.

S	SECONDARY MEANS OF ESCAPE FROM WINDOWS

All first floor windows in dwellings must have opening lights/sashes large enough to escape through in the event    of being trapped within a room by fire.    To meet this requirement all such windows must have a minimum unobstructed openable area of at least 0.33M2 & not to be less than 450mm wide & 750mm high.    Also the bottom of the openable area should not exceed 1100mm above the floor or be less than 800mm off the floor.

S	VENTILATION REQUIREMENTS - GENERALLY

HABITABLE ROOMS.

1.	Opening windows to have 1/20th floor area for rapid ventilation.

2.	8000mm2 controllable trickle vents located in the head of the window frames for background

ventilation.

3.	No requirement for mechanical or passive stack ventilation.

KITCHENS.

1.	Opening windows of no min. size for rapid ventilation.

2.	4000mm2 controllable trickle vents located in the head of the window frames for background

ventilation.

3.	Mechanical extraction from fans to external air consisting of 30 litres/sec adjacent to a hob or

60 litres per sec. if fan located elsewhere in kitchen.

UTILITY ROOM

1.	Opening windows of no min. size for rapid ventilation.

2.	4000mm2 controllable trickle vents located in the head of the window frames for background

ventilation.

3.	Mechanical extraction from fans to external air consisting of 30 litres/sec.

SANITARY ACCOMMODATION EXCLUDING BATHROOMS & SHOWER ROOMS.

1.	Opening windows to have 1/20th floor area for rapid ventilation or mechanical extraction as

stated in '3' below.

2.	4000mm2 controllable trickle vents located in the head of the window frames for background

ventilation.

3.	Mechanical extraction from fan to external air consisting of 6 litres/sec if no windows are in the

room.

BATHROOM & SHOWER ROOM.

1.	Opening windows of no min. size for rapid ventilation.

2.	4000mm2 controllable trickle vents located in the head of the window frames for background

ventilation.

3.	Mechanical extraction from fan to external air consisting of 15 litres/sec.

S	NEW EXTRACT FANS - GENERALLY

- Provide & install 'Expellair' mechanical extract fans as indicated on plans.    Duct fans to outside air & flash to external surfaces accordingly.    Ducting of fans to outside air can be either through the roof / ceilings or via walls.    Fans are to provide the following min. extract rates:-

- Kitchen -		30 litres per sec. if a cooker hood extractor or:-

60 litres per sec. if an adjacent fan.

- Bathrooms & Shower rooms -	15 litres per sec.

- Sanitary rooms -		6 litres per sec. if the room has no window.

- Fan operation: For rooms with windows, the fan is to be operated by an adjacent pull cord isolator switch for intermittent use by the owner.

- For rooms without windows, the fan is to operate in conjunction with light switch.

- All fans are to have an adjustable overrun period of 15 mins. minimum.

S	ROBUST DETAILS FOR AIR TIGHTNESS - EXTENSION WORKS

- Generally: It is a requirement that air leakage from new buildings must not exceed 10 m3/hr at 50 Pa. to prevent heat loss.    Therefore all detailing of the junctions & jointing of ALL the building elements & components must be ’robust’ to prevent air leakage.    In practical terms this means lapping & sealing of all materials to prevent air leakage other than by ‘designed in’ means.    Proving of compliance can be demonstrated by testing using specialist equipment after construction but this is not required for domestic extension work.

- However, even though this particular residential development may not require specific testing for compliance, the main contractor must take every care to ensure that all junctions & laps are sealed tight using correct type of seal & positioning combined with proper supervision of workmanship to reduce air leakage to these defined acceptable limits.

S	ROBUST DETAILS FOR AIR TIGHTNESS - NEW DWELLING HOUSE

- It is a requirement that air leakage from new buildings must not exceed 10 m3/hr at 50 Pa. to prevent heat loss.    Therefore all detailing of the junctions & jointing of ALL the building elements & components must be ’robust’ to prevent air leakage.    In practical terms this means lapping & sealing of all materials to each other at all abutments & junctions to prevent air leakage other than by ‘designed in’ means.

- Special tapes & sealing materials are available for this purpose & the main contractor is advised to research what is available to ensure compliance for a good air tight building.

- Proving of compliance will need to be demonstrated by testing (pre-completion testing) using specialist air pressure equipment after construction & this is to be arranged by the main contractor & a compliance test certificate will be required to confirm that the air leakage figure used for the SAP heat loss calculation is be quall to or better than the on site test.

- Workmanship: The main contractor must take every care to ensure that all junctions & laps are sealed tight using correct type of seal & positioning combined with proper supervision of workmanship to reduce air leakage to these defined acceptable limits.    Should the on site air leakage test fail, then the main contractor is to re-seal all areas required at his own expense & pay for another air pressure re-test.

- Costs: The initial air pressure test will be paid for direct by the client.

S	NEW CONSERVATORY STRUCTURE - (MATERIAL TO BE DECIDED)

- Generally: This item will not form part of the contractors quotation at this stage.    This will be supplied & installed by a specialist manufacturer selected by client.

- This option may change during or just before the contract where the main contractor may be required to install the conservatory foundations & concrete floor during the main extension works.    Client will instruct builder accordingly after extra costs have been established agreed.

- The material & installer for the conservatory will be chosen by client but a general performance specification is stated below:-

- Frame to be patented high performance system of either hardwood, upvc or white aluminium.    The structure is to be supported by a design specification including stability calculations & proof of performance.

- All glazing is to be sealed unit double glazing using safety or laminated glass that complies to the current requirements of Part N of the Building Regulations.

- At the abutment to the extension flank wall, a g.r.p or lead lined box gutter is to be installed flashed into wall.

- All flashing to the existing & extension walls is to be Code 5 lead.    Flash band will not be permitted.

- 50% of the glazed walling is to incorporate opening sashes for ventilation.

- The ridge is to incorporate automatic vents to avoid overheating.

- The roof sheeting is to be triple wall polypropylene translucent shear roof sheeting between supporting spars.

- Provide a mastic seal joint at all frame to wall junctions.

- The foundations for the perimeter dwarf wall are to be 290mm cavity facing brick walling on foundations as specified for the extension.    Raft foundations or lesser foundations will not be permitted.

- The existing rear house wall above the conservatory roof is to be coated with 2 coats of a clear silicone waterproofer.

- Provide & install 4 roof spot light points, local internal switch & 2 No. twin 13amp switched electrical sockets to clients locations within conservatory.    Conceal all wiring.

S	NEW GLAZING NOTE

- Requirement: All new glazing that is below 800mm    off the floor for windows & 1500mm off the floor for doors or glazed side panels are to    comply with Part N of the Building Regulations & to be in accordance with BS. 6206A:-

- Glazing to be either special safety glass with laminated film to retain broken glass or toughened to resist impact & if broken shatter into small pieces that are non harmful.

S	NEW GAS CARCASSING

Provide gas points in the following locations:-

- Kitchen cooker position.

- Gas boiler position.

- The lounge fireplace.

- All carcassing is to be in strict accordance with the local Gas Boards requirements for residential dwellings & any voids housing gas pipes must be vented directly to the open air.    Installation to be completed to BS. CP. 331 & 413.

S	NEW PIPE CASINGS

- Requirement: All groups of pipe runs, soil-vent pipes & extract fan ducts within house are to be encased using 12mm MDF boarding on a 45mm x 45mm softwood framing.    Provide all necessary low level access panels for rodding & maintenance.

S	NEW WARDROBES

- Requirement: As indicated on plans, a certain number of built in wardrobes are to be installed within the bedrooms.    The wardrobes are to be custom made from 22mm MDF board applied to 45mm x 45mm timber batten framing to suit the openings & are to incorporate white melamine shelving units that can be adjusted to suit the clients requirements.    The style & type of wardrobe doors are to be prior approved by client.    All wardrobe carcassing will then be painted inside & out to a colour of clients choosing.

Or:-

- Requirement: As indicated on plans, a certain number of built in wardrobes are to be installed within the bedrooms.    The wardrobes are to be custom made to suit the openings & are to incorporate white melamine shelving units that can be adjusted to suit the clients requirements.    The style & type of wardrobe doors are to be prior approved by client.

- PC Sum: The main contractor is to allow a PC sum of £3,000.00 for the supply & installation of built in wardrobes by either the main contractors joinery specialist or a nominated sub-contractor selected by the client.

S	NEW COVING

- Requirement: All rooms, hallways & landings etc. of new dwelling are to receive 127mm plaster moulded coving at wall to ceiling junctions.    Plaster dab fix & grout all joists.    Profile of coving to clients choosing.    Builder to provide client with coving profile samples for selection.

Or:-

- Requirement: All extended rooms, or new rooms including hallways & landings etc. of new extension are to receive 127mm plaster moulded coving at wall to ceiling junctions.    Plaster dab fix & grout all joists.    Profile of coving to clients choosing.    Builder to provide client with coving profile samples for selection.

S	PLASTERING & MAKING GOOD EXISTING EXTERNAL WALLS THAT BECOME INTERNAL 	WITHIN    EXTENSION (EXCLUDING NEW GARAGES)

- All existing external walls that become internal within extension are to be repaired & made good as required (especially under removed eaves of existing pitched roofs) & then primed with a sealing/boding agent to reduce possible staining.    Plaster existing walls with 19.2 coat plaster.    Fit new skirtings, architrave's etc. as required for a fully finished job.

S	NEW DECORATING (FOR NEW EXTENSION WORKS ONLY)

Prior to decorating all surfaces must be prepared, primed & filled.

NEW EXTERNAL WALLS:-

3 coats Deluxe Weather shield brilliant white paint to new facing bwk. infill panels.

2 coats preservative matt black exterior paint to all new woodwork.

INTERNAL:-

Contractor to allow a provisional sum of £500.00 for this item to be used in whole or in part as decided by the client.

INTERNAL PLASTERED WALLS & CEILINGS:-

3 coats emulsion paint (1 mist + 2 full coats) to colour of clients choosing.

INTERNAL & EXTERNAL JOINERY:-

Knot, stop & prime new wood.    Apply 2 undercoats & 1 top gloss coat to colour of clients choosing.

S	FLOOR COVERINGS

- Yet to be decided.    Contractor to allow the following PC sums:-

- For kitchen - Allow a PC sum of £1,000.00 (supply & fit) for floor tiling of the kitchen.

- For Ensuite, bathroom & ground floor cloaks - Allow a PC sum of £1,000.00 (supply & fit) for floor tiling all these rooms.

Or:-

- This item has yet to be decided & as such is fully excluded from the main contractors work at this stage.    Nearer the end of the contract the client may decide to ask the builder for a tender price for laying of floor coverings when known.

- NOTE - Prior to the laying of the floor screed, the main contractor is to establish from the client the intended floor covering (if any) in case the final finished screed level needs to be adjusted to allow for the floor covering thickness.

S	ADDITIONAL SOUND INSULATION WORKS

- The floor & stud partitioning forming the bathroom & ensuite shower room on the first floor are to receive additional sound proofing for compliance with the Building Regulations comprising of:-

- FOR STUD PARTITIONS - Double tack each side of walling surrounding bathroom & shower with 2 layers of 12.5mm plasterboard to each side. This will achieve a 10kg/m2 min. density of wall. Also install 100mm of Rockwool dense insulating batts between voids to achieve a min. density of 10kg/m3.

- FOR THE FIRST FLOOR - Install 100mm of dense Rockwool insulating batts between the joists to fit the same requirements as above.

S	NEW GARAGE WALLS

- 105mm nom. thickness single skin wall comprising of:-

- Facing bricks external skin: 105mm facing bwk. to match existing (to be prior approved by client & Planning Dept.). Use same size bricks as existing.    Build in brick piers as denoted on plans.

- Footings below ground: dense conc. foundation blocks or common bwk. below g.l. with fine conc. cavity fill terminating 150mm below g.l.

- DPC: Hiload polythene dpc. 150mm above g.l. for both skins; Lap & seal inner skin dpc to floor dpm.

- Mortar: Sand cement mortar with propriatory plasticiser added to the mix. Cured mortar bed to have final colour & pointing to match that of the existing wall.    Refer to separate    section for mortar mixes.

- Wall plates: 100 x 50mm treated s.w. wall plates tied to inner skin bed on mortar with 38 x 5mm    galv. steel straps spanning 600mm of wall & at 2M ctrs.

- Lintels: Galv. steel lintels by Catnic or I.G. Eng. Ltd. (lintel references as stated on plans).

- Vertical DPC’s: All openings in external wall to have vertical dpc's.

- Internal finish: Non - Rubbed trowel handle joints.

- NOTE 1:- Main Contractor to establish cost of best matching brick available prior to tender & include accordingly rather than offering their own PC sum.

S	NEW DETACHED GARAGE FLOOR - GROUND BEARING

- Oversite: 150mm conc. (Grade 20) nominally reinforced with D49 mesh.

- Entrance edge beam: Incorporate a 600 x 450mm thickened edge beam across the garage door opening.

- Floor finish: to be trowelled smooth & laid at a small fall towards front door opening.

- DPM: 1200g polythene dpm. lapped & sealed to all walls.

- Ensure that a flexible joint is achieved between conc. floor & wall abutment.

- Blinding: 50mm lean    mix blinding finished fully level & smooth.

- Hardcore fill: 150mm clean & sulphate free consolidated hard-core.

- All new hard-core must be clean & free of all wood, vegetation etc.    Second-hand hard-core shall not be permitted.

S	NEW GARAGE PITCHED ROOF

- GENERAL REQUIREMENTS:-

- Code of practice: Comply with BS 5534 Slating and tiling subject to any qualifications given below.

- Materials: Main Contractor to supply samples of roof covering materials for clients / specifies approval prior to ordering.    Main Contractor to establish cost of best matching tile / slate available prior to tender & include accordingly rather than their own PC sum.

- Workmanship - Before slating / tiling: Preliminary checks: Check that pipes, etc penetrating the roofing, cloaks, roof gutters, firebreaks and all other pre-roofing work by other trades are completed, particularly any work involving disturbance to the structural roof timbers. Check that all tilting fillets and firrings, etc are fixed or available for fixing by the slaters / tilers.

GENERALLY:-

- Pitch:     **    deg. approx. pitch but to match existing.

- Slates / Tiles:     **    tiles or slates to match existing (to be prior agreed with client & Planning Dept.).

STRUCTURAL ROOF CARCASSING:-

- Rafters: 50 x 125mm at 400 ctrs.

- Ceiling Joists: 50 x 175mm ceiling joists at 400 ctrs. With strut noggin bracing at joist mid span.

- Trimmers: for loft hatch use double joists spiked together.

- Ridge Board: 38 x 225mm.

- Hip rafters: 38 x 175mm hip rafters.

- Valley layer spiking boards: 38 x 200mm valley layer spiking boards.

- Hip layer spiking boards: 38 x 200mm hip layer spiking boards.

- Purlins: 75 x 200mm purlins.

- Struts: 100 x 100mm struts at 1.2M ctrs.

- Wall plates: 50 x 100mm wall plates securely strapped to wall at 2000mm ctrs. using 38 x 5mm galv. steel straps spanning 600mm of wall.

- Eaves overhang: Soffit projections & fascia heights to match the existing roof details.    Use treated s.w. batten framing.

- Barge boarding projections: Projection from wall face & height of barge boarding to match existing house details. Use treated s.w. batten framing combined with gable ladders.

FOR SLATES:-

To roof slopes: Lay slates as follows with spalled edges uppermost:

a) Twice centre nail every slate except very small slates which shall be once head nailed.

b) Lay close jointed with straight horizontal and vertical joints, vertical joints centred over the slate below.

c) Lay close jointed in straight horizontal courses diminishing from eaves to ridge with width of slates selected to centre vertical joints over the slate below.

d) Use thicker slates in lower courses and ensure each slate lays truly flat on the slate below.

e) Use slates of the same thickness in any one course and fix with thinner end uppermost.

f) Maintain bond at ends of courses with slates of extra width.

FOR TILES:-

To roof slopes: Lay tiles as follows:

a) Twice nail every tile in every fifth course.

b) Lay slightly open jointed with straight horizontal and vertical joints, vertical joints centred over the tile below.

c) Maintain bond at ends of courses with width-and-a-half tiles.

DETAILS:-

- Ridges: Ridge coping tiles to match existing roof details.

a) Ensure that the underlay capping strip is in position.

b) Gauge the slating / tiling to ensure that the full lap is maintained in the top course.

c) Bed the specified units in mortar.

d) Fill and gallet exposed ends.

e) Cut mitred joints at hip intersections.

- Hips: Hip copings tiles to match the existing house details.

a) Ensure the additional strip of underlay is in position.

b) Cut slates /tiles to the line of the hip.

c) Bed specified hip fittings in mortar commencing at the lowest point.

d) Fill and gallet exposed ends.

e) Fix hip irons with two galvanized screws.

- For Mitred hips: Form as follows:

a) Ensure the additional strip of underlay is in position.

b) Cut slates / tiles to the line of the hip.

c) Lay width-and-a-half slates in alternate courses/Lay extra wide slates in alternate courses.

d) Fix lead soakers into each course lapped and nailed at head.

- For bonnet hips: Fix as follows:

a) Cut tiles to the line of the hip.

b) Nail the end tile in every course.

c) Commence at foot of hip with exposed end filled and galletted.

d) Lay bonnet tiles to course with the general tiling; nail each unit and bed in mortar.

- Battens: 38mm x 25mm treated s.w. battens at the required gauge.

a) Free from decay or insect attack.

b) Free from any splits and shakes.

c) Wane permissible on one arris only.

d) Knots (or knot holes) less than one third width of any sound surface of occurrence.

e) Treated with preservative: (i) organic solvent; double vacuum or 3 min immersion (ii) Boron diffusion.

f) 22%    Moisture content at time of fixing.

- Securely fix with galvanized nails to every rafter or through counter battens into every rafter as follows:

a) At centres to give the correct slate / tile laps.

b) In long lengths (min length to be supported by at least three rafters with both ends supported).

c) With ends cut square and centred over rafters and nailed without splitting.

d) Packed where necessary to maintain level.

e) Provide additional battens and tilting fillets etc at eaves, ridges and other perimeter features.

f) Provide additional battens if necessary to prevent windlift of horizontal laps in the underlay.

g) Nail penetration into rafters, 50 mm min.

- Nails: Use nails of adequate length and gauge to provide secure fixing as follows:

a) For slates: Aluminium alloy to BS 1202: Part 3/Copper to BS 1202: Part 2/silicon bronze to BS 5534

b) For underlay: Extra large head of aluminium alloy/copper/galvanized steel to BS 1202: Part 1.

c) For battens: Galvanized steel to BS 1202: Part 1 sized to give a secure fixing without splitting the batten.

d) Do not nail through any plumber's metalwork.

- Clips for slates: Aluminium alloy to BS 5534/stainless steel to BS 5534.

- Hip irons: Galvanized steel to BS 5534.

- Mortar: Cement mortar (1:3), coloured to match slate or tile fittings.

a) Use the minimum amount necessary to provide adequate bedding and secure weatherproof fixings.

b) Solidly bed edges and joints of slating / tiling fittings until mortar squeezed out.

c) Immediately strike off surplus mortar and press back with trowel where exposed to form a dense smooth neat finish.

d) Keep mortar off the edge of slates and tiles.

e) Provide a neat finish and reduce the mass of mortar in exposed ends and edges, etc by pushing in tile or slate slips.

- Undercloaks: 6 mm asbestos-free fibre cement board of approved make with the performance standards of asbestos cement to BS 690.    Fix with rougher face uppermost as mortar key.

- Eaves: Lay a double course with undercourse of short slates / tiles head nailed to the bottom batten. Fix with bottom edges in line and projecting over the fascia.

- Verges: Form verges with an upward tilt as follows:

a) Ensure that the underlay extends to the verge line.

b) Lay an undercloak to project 50mm consisting of fibre cement board / a course of slates / a course of plain tiles (but to match the existing house detail) bedded in mortar / fixed with galvanised steel nails.

c) Lay width-and-a-half slates / tiles in alternate courses/ bedded in mortar and finished flush with undercloak.

d) Do not use any cut slates at verges.

- Underfelt: Breathable & untearable roofing felt by Tyvek.    Note - use of this felt eliminates the need to install perimeter & ridge vent venting. Fix as follows:

a) Lay parallel to eaves commencing at eaves and with a slight drape between rafters.

b) Lap 150 mm min at sides and ends with end laps centred over rafters.

c) Nail sufficiently to make secure until battens fixed.

d) Supply & install propriatory eaves carriers dressed under roofing felt, lay over any tilting fillets and extend into gutters. Prevent any sagging which could trap water.

e) Lay a strip over the ridge to overlap general underlay by 150 mm min.

f) Lay 600 mm wide strip at verge below general underlay and extend fully to the eaves line.

g) Lay 600 mm strip at hips over the general underlay and extend fully to the eaves line.

h) Extend up wall at top abutment to suit cover flashing.

i) Upon completion of battening ensure that the roof is proof against adverse weather conditions pending the laying of the slates or tiles.

WEATHERINGS & ABUTMENTS:-

- Soakers: Code 5 lead rolled sheet.

- Flashings: Code 5 lead rolled sheet.

- Valleys - tiles: Special valley tiles over an extra layer of felt.

- Valleys - lead: Form lead valley gutters using 12.5mm marine plywood base; treated softwood tilting fillets to form trough;    cover in Code 5 lead dressed 250mm under tiles & sarking felt;    sharp sand/cement to cut tile edges with waterproofing agent to the mix to seal valley tile / slate verges. Lay an undercloak of fibre cement board fixed with galvanized nails.    Cut tiles / slates to the line of the valley verge to leave    50mm between valley verges.    Lay width-and-a-half tiles in every alternate course bedded in mortar and nailed.

- Open valleys: Form as follows:

a) Ensure the additional strip of underlay is in position under the general underlay before the grp gutter is fixed.

b) Cut slates to line of gutter to leave 100 mm min width between valley verges.

c) Lay width-and-a-half slates /tiles in alternate courses without undercloak or mortar.

- Laced valleys: Form as follows:

a) Ensure the additional strip of underlay is fixed in position.

b) Fix the valley board with feather-edged boards each side over the battening.

c) Fix triangular tilting fillets at base of valley.

d) Continue slate / tile courses upwards in an even curve to the centre of the valley.

e) Form passing courses in valley with double width slates.

- Swept valleys: Form as follows:

a) Ensure the additional strip of underlay is fixed in position.

b) Fix the valley board with feather-edged boards each side over the battening.

c) Cut slates /tiles to correct taper from slates / tiles of sufficient size to maintain specified laps.

d) Lay slates in an even sweep through the valley.

- Abutments at top edges: Ensure that underlay projects above the slating / tiling and finish slating with short course to maintain gauge.

- Abutments at sloping edges: Finish with width-and-a-half slates / tiles in alternate courses. Finish with extra wide slates / tiles in alternate courses and dress in lead soakers to receive flashings.

- Fascias, soffits, barge boarding: treated batten framing to support white upvc’s soffits, fascias & barge boarding.

- Where a roof abuts a new wall: ensure code 5 lead flashings connect to & is dressed under cavity tray dpc’s of wall.

INTERNAL FINISHES:-

- Ceilings: Non - leave open.

INSULATIONS:-

- Insulation: Non proposed.

OTHER:-

- Connections: All bolts used in construction of roof to be 12mm with 50mm spread washers & toothed timber connectors - all junctions & connections in roof MUST be bolted (where possible).

- Strapping: All roof wall plates to be securely strapped to wall using 38 x 5mm galv. steel straps at 2000 ctrs. straps to span 600mm of wall.

- Lateral restraint: Gable end walls parallel to rafters & all floor or ceiling joists are to receive lateral support using 38 x 5mm galv. steel straps at 1200 ctrs. straps to span 3 joists/rafters with strut noggins between joists at strap locations.

- To resist any horizontal thrusts: imposed from unrestrained rafters that cannot be fixed to parallel joists, the rafter feet are to be tied to perpendicular joists using 38 x 5mm galv. steel twist ties with strut noggins between.    Straps to span 3 joists.

S	NEW FRONT GARAGE DOORS - UP'N'OVER

- Standard 7' x 7' garage doors in a 75 x 75mm hardwood frame.

- Doors to be to a design & material yet to be selected by the client purchased from either Garadoor or Henderson complete with tracking & locks etc.

- Supply & install an electric automatic garage door opening device with remote control operation.

- PC Sum: Main Contractor to allow a PC sum of £1,000.00 for the supply only of a new garage door to clients choosing.    Installation of the garage door is deemed to be included elsewhere within the Main Contractors quotation.

S	NEW FRONT GARAGE DOORS - SIDE HUNG TIMBER

- Front garage doors to be specially made legged & braced side hung door sets from pre-treated softwood from a specialist joinery firm & to have single glazed highlight panels as indicated.

- 75 x 75mm pre-treated softwood framing.

- Door ironmongery to have central cylinder key lock with top & bottom galv. steel drop stays.

- Clip back door open stays (galv. steel) are also to be installed.

S	FRONT PORCH / PORTICO FEATURE

- PC Sum - Main Contractor to allow a PC sum of £4,000.00 for the supply only of a stone front porch feature from a specialist supplier yet to be finalised (probably Haddon stone ‘Bath stone’ range or similar & approved).    This will come in the form of preformed units as a complete kit.

- Erection & installation by the Main Contractor is deemed to be included elsewhere within the main contractors quotation.

- The porch will be erected off a concrete foundation base (by basement specialist).    Main contractor to include for the formation of a stone landing/base for the porch area yet to be finalised.

- Main contractor to then form roof covering using Code 5 lead dressed over mopstick on a 12mm thk. Plywood deck & joists to shallow falls.    Raise lead 150mm up wall & dress into mortar joint.

S	REQUIRED DISABLED DETAILING FOR PART M

The following criteria must apply to the following elements:-

- EXTERNAL PARKING AREA - to be at least 2.4M wide x 4.8M long with a 600mm wide clear access space fully around the parking space to allow a disabled person to ‘elite’ from a vehicle.

- ACCESS TO DWELLING - The new dwelling will avoid steep slopes & steps.    Any gradients if possible will not exceed 1:20. There will be a ramped access up to the front door not exceeding 1 in 20 & the level threshold.

- LEVEL THRESHOLD - The new front door is to have a level threshold (no upstand greater than 15mm).

- ELECTRICAL SOCKET HEIGHTS - To be placed within a height zone of between 450mm & 1200mm from the finished floor level.

- DOOR TO GROUND FLOOR CLOAKS - to open outwards.

- FRONT DOOR - clear opening width (widest requirement) = 800mm minimum (for a non head on approach with a 900mm corridor or passage way)

- INTERNAL DOORS - clear opening widths (widest requirement) = 800mm minimum (for a non head on approach with a 900mm corridor or passage way)

- EXTERNAL DRIVES - Hard surfacing only - No loose chippings etc.

- DISABLED ACCESS - is to be provided from edge of site (existing access into new dwelling) which will entail no thresholds higher than 5mm, the formation of a shallow ramp providing level access to the new front door & a firm driveway avoiding loose gravel.

S	EXTERNAL LANDSCAPING (DRIVEWAY, PAVING, PATHS, PLANTERS, RETAINING WALLS ETC.)

- Yet to be established.    Not part of the main contractors quotation at this stage.    However, making good of existing external surfaces & structures is deemed to be included within the main contractors works & tender price.

S	NEW DRIVEWAY (TARMAC)

- Excavate top 300mm of soil & fully compact base.

- Lay in 200mm of well consolidated hoggin with a 50mm 'Bitmac' base course.

- 50mm of 19mm graded bituminous macadam with a 19mm fine cold asphalt wearing top course.

- All layers well compacted & rolled to a level smooth surface.

- Provide a 300mm wide x 300mm deep 19mm graded gravel channel around perimeter of driveway at junction with lawns of which the new driveway must fall to.

S	NEW PAVED AREAS

- Excavate top 300mm of soil & fully compact base.

- Lay in 200mm of well consolidated hoggin with a 50mm lean mix cover.

- Bed in conc. paving slabs of colour & style determined by client.

- 15mm gauge mortar joints.

- Slabs to be fully level & even & are to fall away from the building.

S	NEW GARDEN RETAINING WALL

- Build a new 330mm thick brickwork retaining wall from red engineering bwk. built up off a new 600 x 225mm thk. conc. foundation at a suitable depth.

- Cap wall with a row of engineering bwk. soldier course.

- Build in bwk. steps with walled sides as indicated.

- Backfill behind wall with gravel/shingle to aid drainage & prevent water pressure build up.

- Install 38mm dia. upvc pipe bottom drain holes at base of wall to allow for drainage into new 200mm wide French drain filled with shingle.

- Retaining wall height not to exceed 900mm high for this thickness of solid wall.

S	OUTLINE SCHEDULE OF OTHER WORKS

- Generally: This schedule is a general outline description of the associated works indicated on the plans.    It is not meant to be a concise schedule but the contractor is required to carry out all the works required whether specifically mentioned here or not for a fully finished & made good job ready for decorating.    The main contractor is required to ‘take a view‘ on the works required & price in all the works required accordingly.    These work items are NOT measured works:-

- Form new doorways or adjust openings etc. in existing walls as indicated on plans.    First check that any walls denoted as being removed as ‘non-load bearing’ or not showing a new support beam over, are in fact, non-load bearing by exposing surface finishes & investigating top of wall.    Contact The Specifier immediately should the walls prove load bearing.    Carefully cut openings with a disc cutter after sealing off work area & installing temporary & adequate temporary propping & support of surrounding existing structures reliant upon wall & remove wall section carefully by hand to avoid vibration & shock damage to rest of house.    Install new lintels or beams as denoted on plans or within Structural Engineers etails. Make good reveals, close cavities & make good all structures to decorating standard.    Install new door frames or linings as required.    Plaster & make good.

- Breaking up part of the existing solid conc. Floor: Part of the existing solid concrete floor of the existing dwelling will need breaking & for excavation of the trenches for the new internal wall foundation for partitions that are to be load bearing.    This will require extensive opening up & making good of the existing floors once the new foundations & partitions are in place.    This will include relocating or adjusting any buried services, pipes etc. & the installation / repair of the existing floor dpm.    It is anticipated that the existing concrete floor is a ground bearing slab.    Should this not be the case once the builder starts to open up the existing floor then the works are to stop & the Specifier is to be informed immediately.

- Uneven floor levels: When removing existing internal walls or partitions, it may transpire that the existing floor or ceiling levels are uneven or do not line through with each other.    Should this be the case, any remedial works to ensure level floors or ceilings will be at extra cost to the client to be agreed when the options have been assessed & the best solution identified.

- Make good all external surfaces damaged by the installation of the new works.    Any new external landscaping works may be part of a separate works schedule to be agreed as an extra cost item.

S	SUMMARY OF P.C. SUMS

The following p.c. sums are to be allowed for within the main contractors quotation:-

Bricks:-			£650.00 per 1000

Tiles for main roof:-		£600.00 per 1000

Internal door sets:-		£200.00 per door

Architraving:-		£      0.50 per linear metre

Skirting:-			£      1.00 per linear metre

Bathroom, ensuite & cloaks wall tiling;-	£    70.00 per square metre

The above PC sums will be adjusted to suit actual builders cost for materials after re-measuring on site.    An agreed allowance for wastage for all items (excl. doors) will be included.

The following p.c. sums are to be allowed for within the main contractors quotation:-

Feature fireplace & gas fire:-		£

Staircase:-		 		£

Kitchen furniture:-    			£

Kitchen wall tiling:-			£

Bathroom equipment etc.:-		£

Bathroom wall tiling:-			£

Light fittings:-			£

Security alarm system:-			£

Coving:-				£

Kitchen floor tiling:-			£

Bathroom, ensuite & cloaks floor tiling:-	£

Front door sets:-			£

Gas, Electricity & Water Board fees:-		£

Scaffold hat weather protection:-		£

----------------

Total:-				£    .00 (amount of money to be included within the Main 						Contractors quote)

DETAILS:-

- All p.c. sums allowed for within this specification are to be adjusted to actual invoice figures.

- Should any item likely exceed the allowed p.c. sum, the client is to be informed of the difference prior to ordering the goods.

- Some PC sums are for the ‘material’ & ‘supply’ element only where the installation & fixing of the PC sum items is deemed to be included elsewhere within the Main Contractors price.

- Other PC sums are for both the ‘supply & fix’ element.    Please refer to the separate descriptions under each heading for what the PC sum include.

- There will be a 5% allowance on top of these sums (as adjusted) for the main contractors profit or a charge for the contractors attendance.

- Any trade discounts offered to the contractor on these items will be passed on to the client.

S	CONTINGENCY PROVISIONAL SUM.

- Allow: The contractor is to allow within the quotation a provisional sum of £1,000.00 for any unspecified or additional works required by the client.    The client is at liberty to use in whole or in part or dispense with at his/her request the use of this provisional sum allowance.

- Requirement: Any works or materials ordered under this sum are to be ‘prior quoted for’ in writing by the main contractor.    No additional works are to be started without a written quotation.

S	BUILDERS TENDER PRICING OF THE WORKS

- Requirement: All plans & specifications are a general outline indication / description of the works required.    We have tried to identify & specify as much detail as we can but it is not meant to be a 100% concise for economy of scale of the information provided.    The works are very much reliant upon what is ‘normally expected’ for a fully completed & robust set of building works & each trade engaged by the main contractor is expected to carry out all the works required whether specifically mentioned here or not for a fully finished set of works ready for decorating.    The main contractor is required to ‘take a view‘ on the works required & price in all the works as supply & fix items accordingly.    Requests for extra price items or extras from the main contractor during the works that are deemed to be ‘technical exploitation’ of the tender documents will not be permitted.    However, all genuine variations that will incur additional costs or obvious omissions of the scheme will be considered as extra cost items & there is provision / methods of pricing within this specification for that purpose.

S	VARIATIONS DURING THE WORKS

- Variations: are likely for any building project during the works on site to suit client changes, builder changes, obvious errors & omissions etc. All variations (adds & omits to the final contract price) are to be identified & valued during the works for each specific item of work or materials or both & confirmed in writing by the builder for clients approval.

- Disagreements or challenges: on costed variations may arise from time to time & these are to be explained & enhanced by discussion / extra documentation as required.    Final mutal agreement between the builder & client must be obtained prior to installation / omission of the variation on site.    Contact the Specifier for any unresolved costings for a view / opinion.

- Variations value: is to be based on builders cost price plus 10% for profit & attendance for all priced work.

- Dayworks costing: is to be avoided for additions or extras but if this is agreed as the fairest method between client & main contractor then the following labour rates will apply:-

- Tradesmen:- £25.00 per hour (for normal working hours)

- Labourers:-      £15.00 per hour (for normal working hours)

- Materials:-    Cost price to builder plus 10% margin.

S	PARTY WALL AGREEMENTS

- Requirement: The excavation for the new building works will now implicate the effects of the Party Wall Act & as such will require formal Party Wall Notices to be served together with the issue of the awards by specialist Party Wall Surveyor employed directly by the client.    Refer to the Party Wall Surveyors details & awards for any extra precautions & methods of construction that may be relevant for this project PRIOR to any excavations taking place.    NO WORK IS TO BE STARTED UNTIL THE MAIN CONTRACTOR IS IN RECEIPT OF THESE PARTY WALL AWARD DOCUMENTS.

Or:-

- Observation: It is anticipated that the new building works will not require compliance under the Party Wall Act 1996 as the excavations are a distance away from any adjoining structures.    However, this is only a view by the specifier atht could be incorrect.    Therefore, the client & the final selected builder is to seek advice from a Party Wall Survey to check this aspect of compliance prior to starting the works on site.

GENERAL NOTES

1.	All constructional timber to be:-

i).      Class C16 or better (or as otherwise stated by Structural Engineer within his calculation details or 	marked on plans) & graded at an average moisture content not exceeding 20%.

ii).    Be clearly marked as ‘DRY’ or ‘KD’.

iii). Be preservative treated using Hicksons Vac Vac or similar & approved.

2.	New facing brickwork & tiles to be prior agreed with the client & Planning Dept. before supplying to site.

3.	All roof wall plates to be securely strapped to wall using 38 x 3mm galv. steel straps at 2000mm ctrs. 	straps to span 600mm of wall.

4.	Walls parallel to rafters & all floor or ceiling joists are to receive lateral support using 38 x 3mm galv. 	steel straps at 1200 ctrs. straps to span 3 joists/rafters with strut noggins between joists at strap 	locations.

6.	Floor & ceiling joists to be herringbone braced at 2000 ctrs. using 50 x 200mm strut noggins

for joists of 150mm in depth or greater.

7.	Contractor to ensure that all adequate precautions are taken to prop. & support all structures required 	whilst new work is carried out.

8.	All work shall be carried out in a sound, neat, durable & workmanlike manner to the satisfaction of the 	Local Authorities Building Inspector & the client.

9.	All dpc's, dpm's & cavities to be married & made continuous.

10.	All window & door sizes are nominal only.    Openings to suit actual frame sizes.

11.	All items & materials to be installed in strict accordance with the manufacturers instructions &

recommendations.

12.	Scrim joint all new plasterboard ceilings & studwork partitions.

13.	Great care is to be exercised to ensure that no mortar or other debris falls into the cavities during 	construction forming a possible bridge for water penetration to the inner skin.

14.	All reference to the 'client' within this Specification relates to Mr. & Mrs. ******* & they

are at liberty to amend & alter any specified items as they deem necessary, provided compliance with 	the Building Regulations & good codes of practice are still maintained.

15.	It is a requirement that the main contractor is to ensure that all trades employed are specialists

within their own field to maintain a good quality of workmanship.

16.	The contractor shall be responsible for the co-ordination & proper execution & completion of the works 	& protection of the site.    The contractor shall arrange & ensure that all the sub-contractors requirements 	for storage, chases, holes, recesses, fixings & the like are met before the work is put in hand to avoid 	conflict with other works.

17.	The contractor shall provide all necessary temporary lighting for the works.

18.	The items of work specified within this Specification have been described in reasonable detail but the 	contractor shall consider them in conjunction with the drawings & the actual work

involved on site & shall allow for everything necessary for carrying out the work in the best 	manner 	whether specifically mentioned or not.

19.	Method of payment for the works to be agreed between client & contractor.    However, no monies are to 	be paid in advance of the works.

21.	Any contractor or contractors sub-contractor quoting for the works is advised to visit site before 	tendering to ensure that he is fully aware of the existing site conditions etc.